Propeller 2 Spin Manual

# NOTICE: Work In Progress

This document is not complete; it  is a work in progress.

The following indicators appear in different sections to describe their state:

- {tbw} 	- needs to be written

- {rough} 	- rough draft

- {draft} 	- revised; needs review/finalization

- no mark 	- considered official, though may be enhanced over time

#### COPYRIGHTS AND TRADEMARKS

This documentation is copyright © 2024 by Parallax Inc. By downloading or obtaining a printed copy of this documentation you agree that it is to be used with, or with products containing, the Parallax Propeller 2 P2X8C4M64P microcontroller. Any other uses are not permitted and may represent a violation of Parallax copyrights, legally punishable according to Federal copyright or intellectual property laws. Any duplication of this documentation for commercial uses is expressly prohibited by Parallax Inc. Duplication for educational use is permitted, subject to the following Conditions of Duplication: Parallax Inc. grants the user a conditional right to download, duplicate, and distribute this text without Parallax's permission. This right is based on the following conditions: the text, or any portion thereof, may not be duplicated for commercial use; it may be duplicated only for educational purposes when used solely in conjunction with Parallax products, and the user may recover from the student only the cost of duplication.

Parallax, Propeller Spin, and the Parallax logos are trademarks of Parallax Inc. If you decide to use any trademarks of Parallax Inc. on your web page or in printed material, you must state that "(trademark) is a (registered) trademark of Parallax Inc." upon the first appearance of the trademark name in each printed document or web page. Other brand and product names herein are trademarks or registered trademarks of their respective holders.

#### DISCLAIMER OF LIABILITY

Parallax Inc. makes no warranty, representation or guarantee regarding the suitability of its products for any particular purpose, nor does Parallax Inc. assume any liability arising out of the application or use of any product, and specifically disclaims any and all liability, including without limitation consequential or incidental damages even if Parallax Inc. has been advised of the possibility of such damages.

#### INTERNET DISCUSSION LISTS

Parallax maintains active web-based discussion forums for people interested in Parallax Propeller products, at forums.parallax.com.

#### ERRATA

While great effort is made to assure the accuracy of our texts, errors may still exist. If you find an error, please let us know by commenting/suggesting on live documentation, or by sending an email to editor@parallax.com. We continually strive to improve all of our educational materials and documentation, and frequently revise our texts. Occasionally, an errata sheet with a list of known errors and corrections for a given text will be posted to our website, www.parallax.com. Please check the individual product page’s free downloads for an errata file.

#### SUPPORTED HARDWARE AND FIRMWARE

This manual is valid with the following hardware and firmware versions:

| ###### Hardware | ###### Firmware |
| --- | --- |
| P2X8C4M64P | Rev B/C |

#### CREDITS

Authorship: Jeff Martin

With many thanks to everyone in the Propeller Community and staff at Parallax Inc.

# TABLE OF CONTENTS

NOTICE: Work In Progress	1

COPYRIGHTS AND TRADEMARKS	2

DISCLAIMER OF LIABILITY	2

INTERNET DISCUSSION LISTS	2

ERRATA	2

SUPPORTED HARDWARE AND FIRMWARE	2

Hardware	2

Firmware	2

CREDITS	2

TABLE OF CONTENTS	3

PREFACE	6

2: Spin2 Language Reference	6

Structure Of Propeller Applications	6

Minimal Spin2 Object Example (single PUB block only)	6

Minimal PASM2 Object Example (single DAT block only)	6

Spin2 Object Example (all blocks used)	6

Block Designators	7

Propeller 2 Object Blocks	7

CON	7

VAR	8

OBJ	8

PUB/PRI	8

DAT	8

Comments	8

Categorical Listing Of Propeller 2 Spin Language (Spin2)	8

Block Designators	8

Configuration	8

Cog Control	9

I/O Control	9

Process Control	9

Flow Control	10

Data / Memory	10

String Handling	11

Math	11

Debugging	12

Timing	13

In-line PASM	13

PASM Interfacing	13

PASM	13

Registers	13

Unary Operators	14

Binary Operators	14

Syntax Symbols	14

Spin Language Elements	15

Symbol Rules	15

Value Representations	15

Syntax Definitions	15

Code Examples	16

ABORT	17

About the Call Stack	17

Using ABORT	17

The Abort Trap ( \ )	18

Example Use Of Abort	18

An Abort without Abort Trap (\)	19

Zero or One Value Allowed	19

CASE / CASE_FAST	20

Indention is Critical	20

Using CASE	20

Using OTHER	20

Using CASE_FAST	21

Code Variations	21

IF / IFNOT	23

Indention is Critical	23

Simple IF Statement	23

Combining Conditions	24

Using IF with ELSE	24

Using IF with ELSEIF	24

Using IF with ELSEIF and ELSE	25

The ELSEIFNOT Condition	26

REPEAT	27

Indention is Critical	27

Infinite Loops (Syntax 1 w/o Count)	28

Simple Finite Loops (Syntax 1)	28

Counted Finite Loops (Syntax 2 & 3)	28

Conditional Loops (Syntax 3 and 4)	30

Other REPEAT Options	31

RETURN	32

About the Call Stack	32

Using RETURN	32

Multiple Return Values	33

Propeller 2 Reserved Words (Spin2 + PASM2)	34

_ (leading underscore)	34

A - B	34

C - D	34

E - F	34

G - H	34

I - J	34

L - M	34

N - O	34

P	35

Q - R	35

S - T	35

U, V, W	35

X, Y, Z	35

CHANGE LOG	37

PARALLAX INCORPORATED	37

# PREFACE

This manual is an in-depth description of the Spin 2 programming language of the Propeller 2 multicore microcontroller.

For additional documentation and resources, including programming tools, visit www.parallax.com/P2. The latest version of this manual, along with links to a commentable Google Doc version, are available from the Documentation section. In addition, there are links to more in-depth references for the Propeller 2 and its Spin2 and PASM2 languages, which may include commentable Google Docs.

# 2: Spin2 Language Reference

{tbw}

## Structure Of Propeller Applications

{draft}

The Propeller 2 supports programming in multiple languages.  This document focuses primarily on the Parallax Spin2 language but also introduces a little of its close companion, PASM2 (Propeller 2 Assembly Language).

Spin2 applications are built from one or more objects.  Objects are Spin2 text files describing executable code to perform a dedicated function.  Spin2 objects often include a combination of public and private methods, constants, variables, data, and references to other objects in order to do their job.

To form an application, its objects are assembled together into a top-level executable object with an internal hierarchy of sub-objects.  At run-time, each object instance gets its own set of variables, as defined by the object, to maintain its unique operating state.

The tables below show minimum and larger examples of complete object files.

###### Minimal Spin2 Object Example (single PUB block only)

PUB MinimalSpin2Program() 'first PUB method executes REPEAT PINWRITE(7..0, GETRND()) 'write a random pattern to P7..P0 WAITMS(100) 'wait 1/10th of a second, loop

###### Minimal PASM2 Object Example (single DAT block only)

DAT ORG 'start PASM at hub $00000 for cog $000 loop DRVRND #0 ADDPINS 7 'write a random pattern to P7..P0 WAITX ##clkfreq_/10 'wait 1/10th of a second, loop JMP #loop

###### Spin2 Object Example (all blocks used)

CON _clkfreq = 297_000_000 'set clock frequency OBJ vga : "VGA_640x480_text_80x40" 'instantiate vga object VAR time, i 'declare object-wide variables PUB go() 'this first public method executes, cog stops after vga.start(8) 'start vga on base pin 8 SEND := @vga.print 'establish SEND pointer SEND(4, $004040, 5, $00FFFF) 'set light cyan on dark cyan time := GETCT() 'capture time i := @text 'print file to vga screen REPEAT @textend-i SEND(byte[i++]) time := GETCT() - time 'capture time delta in clock cycles time := MULDIV64(time, 1_000_000, clkfreq) 'get time delta in microseconds SEND(12, "Time during printing was ", dec(time), " microseconds.") 'print time delta PRI dec(value) ¦ flag, place, digit 'private method prints decimals, three local variables flag~ 'reset digit-printed flag place := 1_000_000_000 'start at the one-billion's place and work downward REPEAT IF flag ¦¦= (digit := value / place // 10) ¦¦ place == 1 'print a digit? SEND("0" + digit) 'yes IF LOOKDOWN(place : 1_000_000_000, 1_000_000, 1_000) 'also print a comma? SEND(",") 'yes WHILE place /= 10 'next place, done? DAT text FILE "VGA_640x480_text_80x40.txt" 'include raw file data for printing textend

### Block Designators

Each object uses six different special-purpose blocks (CON, VAR, OBJ, PUB, PRI, and DAT) to organize its contents.  An object may include few or many of each block type but must include at least one PUB block or one DAT block; public method or data block, respectively.  If a PUB block is included, the object may contain both Spin2 and PASM2 source code.  If a DAT block (but no PUB block) is included, the object may contain PASM2 source code or just data and constants (no code at all).

| ###### Propeller 2 Object Blocks |  |  |  |
| --- | --- | --- | --- |
| Block Identifier | Block Contents | Spin2+PASM2 Applications | PASM2-only Applications |
| CON | Constant and data-structure declarations (CON is the initial/default block type) | Permitted | Permitted |
| VAR | Variable declarations | Permitted | Not Allowed |
| OBJ | Child-object declarations (creates compile-time object instantiations) | Permitted | Not Allowed |
| PUB | Public method for use by the parent object and within this object | Required | Not Allowed |
| PRI | Private method for use only within this object | Permitted | Not Allowed |
| DAT | Data declarations and/or PASM2 code | Permitted | Required |

#### CON

{rough}

CON blocks are used to declare symbolic constants and data structures which can be used throughout the object.  There may be zero or more CON blocks in an object and the order in which they appear does not matter.  Typically, objects have just one CON block, near the top of code, that defines every constant for that object; however, objects can be organized with multiple smaller CON blocks that each appear right above the code or data (PUB/PRI/DAT) that directly use those symbols.

Example CON Block:

CON

EnableFlow = 8	'single assignment

x = 5, y = -5, z = 1	'comma-separated assignments

HalfPi = 1.5707963268	'IEEE-754 single-precision float values

#0,a,b,c           	'enum; a=0, b=1, c=2  (start=0, step=1)

#1,e,f,g	'enum; e=1, f=2, g=3  (start=1, step=1)

#4[2],i,j,k	'enum; i=4, j=6, k=8  (start=4, step=2)

sPoint(x, y)	'structure; two longs

sLine(sPoint a, sPoint b, BYTE color)	'structure; two sPoints and a byte

#### VAR

{tbw}

#### OBJ

{tbw}

#### PUB/PRI

{tbw}

#### DAT

{tbw}

### Comments

{tbw}

## Categorical Listing Of Propeller 2 Spin Language (Spin2)

In the following category lists, styles indicate different types of elements.

- BLOCKS,  DIRECTIVES, and COMMANDS

- METHODS() and ACCESSORS[]

- CONSTANTS

- PSEUDO-CONSTANTS

- variables

### Block Designators

CON	Declare constant block

VAR	Declare variable block

OBJ	Declare object reference block

PUB	Declare public method block

PRI	Declare private method block

DAT	Declare data block

### Configuration

{draft}

_CLKFREQ	Set application (initial) clock frequency at compile-time

_CLKMODE	Set application (initial) clock mode at compile-time

_ERRFREQ	Set acceptable frequency variance (+/-) from _CLKFREQ at compile-time

CLKFREQ_	Compiled application clock frequency

CLKMODE_	Compiled application clock mode

clkfreq	Current clock frequency at run-time

clkmode	Current clock mode at run-time

CLKSET()	Safely set new clock frequency and mode at run-time

HUBSET()	Execute system configuration instruction

DOWNLOAD_BAUD	Set the download baud rate

DEBUG_BAUD	Set debug subsystem serial baud rate

DEBUG_COGS	Select cogs to debug

DEBUG_DELAY	Delay (in milliseconds) application waits before transmitting debug messages

DEBUG_PIN_TX	Select serial output pin for debug subsystem

DEBUG_PIN_RX	Select serial input pin for host interaction with debug subsystem

DEBUG_LEFT	Set left coordinate where the Debug Message window appears on the host screen

DEBUG_TOP	Set top coordinate where the Debug Message window appears on the host screen

DEBUG_WIDTH	Set the width of the Debug Message window on the host screen

DEBUG_HEIGHT	Set the height of the Debug Message window on the host screen

DEBUG_DISPLAY_LEFT	Set the overall left offset where any Debug displays appear on the host screen

DEBUG_DISPLAY_TOP	Set the overall top offset where any Debug displays appear on the host screen

DEBUG_WINDOWS_OFF	Prevent any debug windows from opening after downloading

DEBUG_TIMESTAMP	Enable message time-stamping

DEBUG_LOG_SIZE	Set the max size (in bytes) of the Debug log file which collects debug messages

DEBUG_COGINIT	Invoke cogs' PASM-level debugger when a COGINIT occurs

DEBUG_MAIN	Same as above plus ready to single-step through main (non-interrupt) code.

### Cog Control

{draft}

COGSPIN()	Start, or restart, a Spin2 cog by ID

COGINIT()	Start, or restart, a PASM2 cog by ID

COGSTOP()	Stop a cog by ID

COGID()	Get current cog’s ID

COGCHK()	Check if cog running, by ID

### I/O Control

{draft}

PINW() / PINWRITE()	Drive pin(s) high and/or low

PINL() / PINLOW()	Drive pin(s) low

PINH() / PINHIGH()	Drive pin(s) high

PINT() / PINTOGGLE()	Drive and toggle pin(s)

PINF() / PINFLOAT()	Float pin(s)

PINR() / PINREAD()	Read pin(s)

PINSTART()	Start smart pin(s)

PINCLEAR()	Clear smart pin(s)

WRPIN()	Write to Mode register of smart pin(s)

WXPIN()	Write to X register of smart pin(s)

WYPIN()	Write to Y register of smart pin(s)

AKPIN()	Acknowledge smart pin(s)

RDPIN()	Read and acknowledge smart pin

RQPIN()	Read, but don't acknowledge, smart pin

### Process Control

{draft}

LOCKNEW()	Check out a new lock

LOCKRET()	Release a lock

LOCKTRY()	Try to capture a lock

LOCKREL()	Release a lock

LOCKCHK()	Check a lock's state

COGATN()	Ask for a cog's attention

POLLATN()	Check if any cog needs our attention

WAITATN()	Wait for any cog to ask our attention

### Flow Control

IF / IFNOT	Conditionally execute block of code…

… ELSE	optionally execute another upon opposite condition,

… ELSEIF / ELSEIFNOT	optionally execute another upon opposite condition plus new condition

CASE / CASE_FAST	Evaluate expression and execute block of code matching a condition…

… OTHER	optionally execute block of code when no match found

REPEAT	Execute block of code repetitively an infinite or finite number of times with…

+ WITH	optional loop counter,

+ FROM… TO… STEP	optional loop counter plus intervals and steps, or…

+/… UNTIL / WHILE	optional exit and continue conditions

… NEXT	Skip rest of REPEAT block and jump to next loop iteration

… QUIT	Exit from REPEAT loop

RETURN	Exit PUB/PRI with normal status and optional return value

ABORT	Exit PUB/PRI with abort status and optional return value

### Data / Memory

{draft}

ALIGNW	Word-align variable or data to main memory

ALIGNL	Long-align variable or data to main memory

BYTE / BYTE[]	Declare byte-sized symbol, DAT data, or access byte of main memory

WORD / WORD[]	Declare word-sized symbol, DAT data,  or access word of main memory

LONG / LONG[]	Declare long-sized symbol, DAT data,  or access long of main memory

BYTEFIT	Declare byte-sized data (in DAT) limited to range -$80 to $FF

WORDFIT	Declare word-sized data (in DAT) limited to range -$8000 to $FFFF

FIELD[]	Access bit field within main memory byte/word/long or from register

REG[]	Access a cog register

BYTEFILL()	Fill bytes of main memory with a value

WORDFILL()	Fill words of main memory with a value

LONGFILL()	Fill longs of main memory with a value

BYTEMOVE()	Copy bytes from one region to another in main memory

WORDMOVE()	Copy words from one region to another in main memory

LONGMOVE()	Copy longs from one region to another in main memory

BYTESWAP()	Swap two ranges of bytes in main memory

WORDSWAP()	Swap two ranges of words in main memory

LONGSWAP()	Swap two ranges of longs in main memory

BYTECOMP()	Compare two ranges of bytes in main memory

WORDCOMP()	Compare two ranges of words in main memory

LONGCOMP()	Compare two ranges of longs in main memory

GETREGS()	Copy cog register longs to main memory

SETREGS()	Copy main memory longs to cog registers

FILL()	Fill structure with byte value

COPY()	Copy structure's contents to another structure

SWAP()	Swap structure's contents with another structure

COMP()	Compare contents of two structures

GETCRC()	Compute CRC (up to 32-bits) of bytes from main memory

LOOKUP()	Get value at index (1..N) from a list

LOOKUPZ()	Get value at zero-based index (0..N−1) from a list

LOOKDOWN()	Get index (1..N) of a matching value from a list

LOOKDOWNZ()	Get zero-based index (0..N−1) of a matching value from a list

SEND()	Special method pointer for efficient output of data

RECV()	Special method pointer for efficient input of data

FILE	Include binary file in DAT block

varbase	Main memory base for object; used by method-pointer calls

### String Handling

{draft}

BYTE()	Declare string of byte-sized values and return a pointer

WORD()	Declare string of word-sized values and return a pointer

LONG()	Declare string of long-sized values and return a pointer

STRING()	Declare compile-time in-line z-string expression and return a pointer

LSTRING()	Declare compile-time in-line l-string expression and return a pointer; zeros allowed

STRSIZE()	Get size of z-string in bytes

STRCOPY()	Copy z-string from one location to another; up to specified max

STRCOMP()	Compare contents of two z-strings

@""	Declare compile-time in-line text z-string and return a pointer

### Math

{draft}

FLOAT()	Declare compile-time floating-point expression

ROUND()	Round compile-time floating-point expression to integer

TRUNC()	Truncate compile-time floating-point expression at decimal

FSQRT	Get square root of floating-point number

NAN()	Check if floating-point value is a number

GETRND()	Get random value

ROTXY()	Rotate point by angle

POLXY()	Convert from polar to cartesian coordinate

XYPOL()	Convert from cartesian to polar coordinate

QSIN()	Rotate horizontal vector by step ratio; returns y

QCOS()	Rotate vertical vector by step ratio; returns x

MULDIV64()	Multiply 32-bit values then divide the 64-bit product

FALSE	Boolean false; same as 0 ($0000_0000)

TRUE	Boolean true; same as -1 ($FFFF_FFFF)

NEGX	Most-negative integer; -2_147_483_648 ($8000_0000)

POSX	Most-positive integer; +2_147_483_647 ($7FFF_FFFF)

PI	Single-precision floating-point value of Pi; 3.14159265

### Debugging

{draft}

Lower-case 'x' means multiple variations exist.

DEBUG_x	Configure system debug behaviors (see Configuration)

DEBUG	Invoke PASM-level debugger (no parentheses)

DEBUG()	Transmit debug message to host

IF() / IFNOT()	Conditionally execute next Debug Command(s)

DLY()	Delay (in milliseconds) after performing Debug's command(s)

PC_KEY()	Get the host's keyboard state

PC_MOUSE()	Get the host's mouse state

UDECx() / SDECx() / FDECx() *	Format as unsigned/signed/floating-point decimal value in debug message

UHEXx() / SHEXx() *	Format as unsigned/signed hexadecimal value in debug message

UBINx() / SBINx() *	Format as unsigned/signed binary value in debug message

ZSTRx() / LSTRx() *	Output zero-terminated/length-terminated string in debug message

BOOL() *	Format value as Boolean string ("TRUE"/"FALSE") in debug message

*  Alternate formatters exist (same name with "_" appended) for value-only output;  ex: BOOL_, ZSTR_, and UDEC_.

TERM	Create text terminal debug display

BITMAP	Create bitmap debug display

PLOT	Create general-purpose cartesian/polar plotter debug display

LOGIC	Create multi-channel logic analyzer debug display

SCOPE	Create multi-channel oscilloscope debug display

SCOPE_XY	Create multi-channel XY polar/log-scale oscilloscope debug display

FFT	Create multi-channel Fast Fourier Transform log-scale debug display

SPECTRO	Create spectrograph log-scale debug display

MIDI	Create piano keyboard debug display

TITLE, POS, SIZE, RANGE, RATE,	These display commands are used to configure graphical

SPACING, SAMPLES, ALT, SIGNED,	debug displays during their creation

CHANNEL, DEPTH, MAG, LOGSCALE,

TRACE, DOTSIZE, SPARSE,

LINESIZE, TEXTSIZE, COLOR,

BACKCOLOR, LUTCOLORS, BLACK,

WHITE, ORANGE, BLUE, GREEN,

CYAN, RED, MAGENTA, YELLOW,

GRAY, GREY, LUTx, LUMA8x, HSVx,

RGBx, POLAR, CARTESIAN,

HIDEXY, LONGS_xBIT, WORDS_xBIT,

BYTES_xBIT, UPDATE

OPACITY, PRECISE, ORIGIN,	These display commands are used to adjust graphical

SET, SCROLL, LINESIZE, DOT,	debug displays while feeding them data

LINE, CIRCLE, OVAL, BOX, OBOX,

COLOR, BACKCOLOR, LUTCOLORS,

BLACK, WHITE, ORANGE, BLUE,

GREEN, CYAN, RED, MAGENTA,

YELLOW, GRAY, GREY, LUTx, LUMA8x,

HSVx, RGBx, TEXTSTYLE,

TEXTANGLE, TEXT, SPRITEDEF,

SPRITE, TRACE, AUTO, TRIGGER,

HOLDOFF, CLEAR, SAVE, CLOSE

### Timing

{draft}

GETCT()	Get count from 32-bit system counter

POLLCT()	Check if system counter passed target

WAITCT()	Wait for system counter to pass target

WAITUS()	Wait for target microseconds

WAITMS()	Wait for target milliseconds

GETMS()	Get milliseconds since boot up

GETSEC()	Get seconds since boot up

### In-line PASM

{draft}

ORG	Start in-line PASM2 code; optionally positioned and limited

ORGF	Prefill cog registers with zeros up to specified address

RES	Reserve, but don't fill, one or more cog registers

FIT	Limit in-line PASM2 code to desired length

END	End in-line PASM2 code

### PASM Interfacing

{draft}

CALL()	Call register or main-resident PASM2 code from Spin2

REGEXEC()	Load and execute PASM2 code from register memory

REGLOAD()	Load, but don't execute, PASM2 code into register memory

### PASM

ASMCLK	(deprecated) Set clock mode for PASM2-only applications

### Registers

{draft}

PR0	Propeller register 0; general purpose Spin2/PASM2 use

PR1	Propeller register 1; general purpose Spin2/PASM2 use

PR2	Propeller register 2; general purpose Spin2/PASM2 use

PR3	Propeller register 3; general purpose Spin2/PASM2 use

PR4	Propeller register 4; general purpose Spin2/PASM2 use

PR5	Propeller register 5; general purpose Spin2/PASM2 use

PR6	Propeller register 6; general purpose Spin2/PASM2 use

PR7	Propeller register 7; general purpose Spin2/PASM2 use

IJMP3	Interrupt jump 3; for PASM2 interrupt routines

IRET3	Interrupt return 3; for PASM2 interrupt routines

IJMP2	Interrupt jump 2; for PASM2 interrupt routines

IRET2	Interrupt return 2; for PASM2 interrupt routines

IJMP1	Interrupt jump 1; for PASM2 interrupt routines

IRET1	Interrupt return 1; for PASM2 interrupt routines

PA	Pointer register A; for PASM2 indirect memory access

PB	Pointer register B; for PASM2 indirect memory access

PTRA	Data pointer passed from COGINIT; for PASM2

PTRB	Code pointer passed from COGINIT; for PASM2

DIRA	Direction register for 32-bit port A (P31:P0)

DIRB	Direction register for 32-bit port B (P63:P32)

OUTA	Output register for 32-bit port A (P31:P0)

OUTB	Output register for 32-bit port B (P63:P32)

INA	Input register for 32-bit port A (P31:P0) [read only]

INB	Input register for 32-bit port B (P63:P32) [read only]

### Unary Operators

{tbw}

### Binary Operators

{tbw}

### Syntax Symbols

{tbw}

## Spin Language Elements

{tbw}

### Symbol Rules

{tbw}

### Value Representations

{tbw}

### Syntax Definitions

In addition to detailed descriptions, the following pages contain syntax definitions that concisely demonstrate all the options of that element. The syntax definitions use special symbols to indicate when and how certain element features are to be used.

BOLDCAPS 	Items in bold uppercase should be typed in as shown.

Italics	Items in italics should be replaced by user text; symbols, operators, expressions, etc.

. .. : , #	Periods, double-periods, colons, commas, pound signs, pipes, back slashes, square brackets

| \ [ ] ( )	and parentheses should be typed in where shown.

{ }	Curly brace symbols enclose optional items. Enter the enclosed item if desired. Do not enter the curly braces.

⦅┆⦆	Hollow parentheses symbols enclose mutually exclusive items, each separated by a dash-bar. Enter one, and only one, of the encoded items. Do not enter the hollow parentheses or dash-bar.

…	Repetition symbol indicates that the previous item, or group, can be repeated numerous times. Repeat the last item(s) if desired. Do not enter the repetition symbol.

⇢	Indent symbol indicates following items should be intended by at least one space.

↪	New Line+Indent symbol indicates following items should appear on a new line, indented by at least one space.

Thin line	Separates various syntax structure options.

Heavy line	Separates instruction from the value it returns.

Since elements are limited to specific blocks, all syntax definitions begin with an indication of the type of block required. For example, the following syntax indicates that the BYTEFILL command and its parameters must appear in either a PUB or PRI block, but it may be one of many commands within that same block.

⦅PUB┆PRI⦆

BYTEFILL (StartAddress, Value, Count)

### Code Examples

{draft}

Much of this document's descriptions feature short code examples that follow these conventions:

normal code	Commands, directives, etc. appear in monospaced font to represent exact code

<pseudocode>	Items in angle brackets represent a concept, not literal executable code; it must be replaced with valid code before execution

bold code	Just like normal code, but marks start of block for better readability

highlighted code	Highlights emphasize the main topics to note

...	Ellipsis means "more code here" where it's important to know code belongs there but specifics are irrelevant to the example

PUB/PRI (implied)	If an example is a short snippet of Spin2 code, remember that it is implied to be within a PUB or PRI block that was left out for brevity

## ABORT

{draft}

Command: Exit with abort status from PUB/PRI method with optional return Value.

⦅PUB┆PRI⦆

ABORT {Value}

Returns: Either zero, or Value if provided.

- Value is an optional expression whose value is to be returned, with abort status, from the method.

Explanation

ABORT is one of two commands (ABORT and RETURN) that immediately terminates a PUB or PRI method.

ABORT causes the PUB or PRI method to exit with abort status— meaning it pops the call stack repeatedly from any depth of method calls until it either reaches a caller that includes an Abort Trap and delivers a value to that caller, or the call stack is emptied and the cog terminates.  All callers in-between are terminated without warning.

It may be easier to write that application using small, specialized methods that are called in a nested fashion, each meant to deal with a specific sub-event in the chain.  When one of the methods determines a course of action, or it recognizes a fatal error condition, it can issue an ABORT (and optionally a special value) that completely collapses the nested call chain and prevents all the intermediate methods from continuing.

When ABORT appears without the optional Value, it returns zero (FALSE).

#### About the Call Stack

When methods are called, a hidden mechanism keeps track of important information.  This mechanism is a “call stack;” an area of RAM used to store return addresses, return values, method parameters, and local variables for intermediate use.  As more methods are called, additional sets of information are stacked; the call stack grows taller.  As methods exit, this temporary information is removed and the call stack gets shorter.  This process is called "pushing” onto the stack and “popping” off of the stack, respectively.

The ABORT command repetitively pops data off the call stack until it reaches a caller with an Abort Trap (see below); returning to some higher-level caller potentially many steps up the nested call chain.  Any return points along the way between an aborting method and an abort trapping method are ignored and terminated.  In this way, ABORT allows code to back out of very deep and complicated logic to handle a serious issue at a high level.

#### Using ABORT

Any method can choose to issue an ABORT command in addition to a normal RETURN command.  Usually an abort is triggered by a special condition, so an IF or CASE construct is used.

if <bad condition>

abort	'If bad condition detected, abort without value

—or—

case <test value>

...

<bad condition>: abort <value>	'If bad condition detected, abort with value

...

<bad condition> is a test that determines the method should abort and <value> is a value to return upon aborting.

#### The Abort Trap ( \ )

It’s up to the caller to check for a method's abort status and handle it, otherwise that caller is eliminated in the event of an abort.

To trap an ABORT, the call to the abortable method must be preceded with the Abort Trap symbol, a backslash (\). For example, when a method named MayAbort might abort itself, or if it calls other methods that could abort, its caller can use the following approach to trap the abort:

\MayAbort	'Call MayAbort with trap; ignore abort value

<continue regardless>	'Perform rest of code regardless

—or—

if \MayAbort	'Call MayAbort with trap and, if non-zero...

<handle abort case>	'Process abort

When a method is called with an Abort Trap, a single abort value will always be returned; 0 (FALSE) by default.  If the method (MayAbort in this case) executes an ABORT command with a value, that value will be returned to the trapping caller.  Example:  ABORT True would return TRUE (-1) to the caller in the example above.

In the example above, if MayAbort used a RETURN instead, execution would still return to the direct, trapping caller, but the value would always be 0 (FALSE) regardless of what RETURN specified, since the Abort Trap always retrieves the special abort value.

#### Example Use Of Abort

This code demonstrates the ABORT command for a robot that avoids objects approaching it and beeps if its motors get stuck.  CheckSensors, Beep, and MotorStuck are assumed to be defined elsewhere.

CON

#0, None, Left, Right, Front, Back	'Direction Enumerations

PUB Main() | Direction

Direction := None

repeat

case CheckSensors	'Get active sensor

Left  : Direction := Right	'Object on left? Let's go right

Right : Direction := Left	'Object on right? Let's go left

Front : Direction := Back	'Object in front? Let's go back

Back  : Direction := Front	'Object in back? Let's go front

other : Direction := None	'Otherwise, stay still

if not \Move(Direction)	'Move robot

Beep	'We're stuck? Beep

PUB Move(Direction): result

result := TRUE	'Assume success

if Direction == None

return	'Return if no direction

repeat 1000

DriveMotors(Direction)	'Drive motor 1000 times

PUB DriveMotors(Direction)

<code to drive motors>

if MotorStuck

abort FALSE	'If motor is stuck, abort

<more code>

Three methods manage robot behavior:

- Main(): [high-level] - Decides movement based on sensor readings

- Move(): [mid-level] - Manages short-distance movement in a direction

- DriveMotors(): [low-level] - Handles motor control and stall detection

This approach, with ABORT, allows critical events in low-level code to be handled by high-level code without complex communication, even with nested mid-level methods.

In the Main() method, when an approaching object is sensed, it calls Move() with the Abort Trap symbol, \ , preceding it.  The Move() method itself never aborts; however, it calls DriveMotors() which can abort.  Move() always returns TRUE when done, but if DriveMotors() detects a stalled motor, it aborts with FALSE and pops everything from the stack back up to Main(), which handles the abort with a beep.  Note that Move() is oblivious to an abort; it ceases to exist when one happens.

#### An Abort without Abort Trap (\)

If the Abort Trap were missing in the example above, the abort action pops the whole stack and terminates the cog entirely.

#### Zero or One Value Allowed

Whenever a method is called with an Abort Trap (\), it will always have only one return value regardless of how many return values its declaration has.  This overriding return value is either zero (0; a.k.a FALSE) when ABORT is given with no Value specified, or is the specified Value.

## CASE / CASE_FAST

Command: Compare expression against matching expression(s) and execute code block if match found.

⦅PUB┆PRI⦆

CASE/CASE_FAST CaseExpression

⇢ MatchExpression : { ↪ } Code

{ ⇢ MatchExpression : { ↪ } Code }…

{ 	⇢ OTHER : { ↪ } Code }

- CaseExpression is the expression to compare.

- MatchExpression is a single or comma-delimited set of value-expressions and/or range-expressions, to compare CaseExpression against.  Each MatchExpression must be followed by a colon (:).

- Code is a block of one or more lines of code to execute when the CaseExpression matches the associated MatchExpression or OTHER.  The first, or only, statement in Code may appear to the right of the colon on the MatchExpression/OTHER line, or below it and indented from MatchExpression/OTHER itself.

Explanation

CASE and CASE_FAST are part of three flow control constructs (IF/IFNOT, CASE/CASE_FAST, and REPEAT) that conditionally executes a block of code.  CASE/CASE_FAST is the preferred structure to use, as opposed to IF..ELSEIF..ELSE, when you need to compare the equality of CaseExpression to a number of different values.

CASE/CASE_FAST compares CaseExpression against the values of each MatchExpression, in order, and if a match is found, executes the associated Code.  If no previous matches are found, the Code associated with the optional OTHER command are executed.

#### Indention is Critical

IMPORTANT: Indention is critical.  The Spin2 language relies on indention (of one space or more) on lines following flow control constructs to determine if they belong to that construct or not.  Your code editor may also  visually indicate such groupings.  If using the Propeller Tool, Ctrl + I toggles block-group indicators on/off.

#### Using CASE

CASE is handy where one of many actions needs to be performed depending on the value of an expression.  The following example assumes A, X, and Y are variables defined earlier.

case X+Y	'Test X+Y

10, 15: pintoggle(0)	'X+Y = 10 or 15? Toggle P0

A*2   : pintoggle(1)	'X+Y = A*2? Toggle P1

30..40: pintoggle(2)	'X+Y in 30 to 40? Toggle P2

X += 5	'Add 5 to X

Since the MatchExpression lines are indented from the CASE line, they belong to the CASE structure and are executed based on the CaseExpression comparison results.  The last line, X += 5, is not indented from CASE, so it is executed after, and regardless of, the CASE results.

This example compares the value of X+Y against 10 or 15, A*2, and the range 30 through 40.  If X+Y equals 10 or 15, P0 is toggled.  If X+Y equals A*2, P1 is toggled.  If X+Y is in the range 30 through 40, inclusive, then P2 is toggled.  Whether or not any match was found, the X += 5 line is executed next.

#### Using OTHER

The optional OTHER component of CASE is similar to the optional ELSE component of an IF structure.  It should appear at the end of the CASE block if there's an action needed when no other match is found.  For example:

case X+Y	'Test X+Y

10, 15: pintoggle(0)	'X+Y = 10 or 15? Toggle P0

25    : pintoggle(1)	'X+Y = 25? Toggle P1

20..30: pintoggle(2)	'X+Y in 20 to 30? Toggle P2

OTHER : pintoggle(3)	'Otherwise toggle P3

X += 5	'Add 5 to X

This example includes an OTHER statement.  If X+Y does not equal 10, 15, 25, or is not in the range 20 to 30, the Code block following OTHER is executed.  After the CASE construct, the X += 5 line is executed.

There is an important concept to note about this example: the MatchExpressions are evaluated in order, so the first valid match will execute and ignore the rest.  If we had arranged the last two MatchExpressions above OTHER so that the range of 20..30 is checked before 25, we’d have a bug in our code.  Here's similar code with that bug:

case X+Y	'Test X+Y

10, 15: pintoggle(0)	'X+Y = 10 or 15? Toggle P0

20..30: pintoggle(2)	'X+Y in 20 to 30? Toggle P2

25    : pintoggle(1)	'X+Y = 25? Toggle P1        <-- THIS NEVER RUNS

The above example contains an error because, while X+Y could be equal to 25, that exact match expression would never be tested since the previous one, range of 20..30 would be tested first and satisfies the case, so its block is executed and no further match expressions are checked.

#### Using CASE_FAST

CASE_FAST is like CASE, but rather than sequentially comparing the target to a list of possible matches, it uses an indexed jump table of up to 256 entries to immediately branch to the appropriate Code block, saving time at a possible cost of larger compiled code.  However, the resulting CASE_FAST code will be smaller than an equivalent CASE construct including several match values, if those values are all contiguous and don't include match ranges.

CASE_FAST MatchExpressions can contain single or comma-delimited values and/or range-expressions, values can be contiguous or not, but all values must all be unique constants that are within 255 of each other.  The order of MatchExpressions doesn't matter in CASE_FAST since, at run-time, CaseExpression feeds into an indexed jump table.

CASE_FAST chr

0:     BYTEFILL(@screen, " ", screen_size)

col := row := 0

1:     col := row := 0

2..7:  flag := chr

RETURN

8:     IF col

col--

9:     REPEAT

out(" ")

WHILE col & 7

10:    RETURN

11:    color := $00

12:    color := $80

13:    newline()

OTHER: out(chr)

#### Code Variations

Each Code block can be many lines and may also appear below, and indented from, the MatchExpression itself.  The following two examples show these variations.

case A	'Test A

4     : pintoggle(0)	'A = 4? Toggle P0

Z+1   : pintoggle(1)	'A = Z+1? Toggle P1

pintoggle(2)	'And toggle P2

10..15: pintoggle(3)	'A in 10 to 15? Toggle P3

case A	'Test A

4:	'A = 4?

pintoggle(0)	'Toggle P0

Z+1:	'A = Z+1?

pintoggle(1)	'Toggle P1

pintoggle(2)	'And toggle P2

10..15:	'A in 10 to 15?

pintoggle(3)	'Toggle P3

## IF / IFNOT

Command: Test condition(s) and execute a block of code if true (IF) or if false (IFNOT).

⦅PUB┆PRI⦆

IF/IFNOT Condition(s)

⇢ If/IfNot-Code

{ ELSEIF/ELSEIFNOT Condition(s)

⇢ ElseIf/ElseIfNot-Code }…

{ ELSE

⇢ Else-Code }

- Condition(s) is one or more Boolean expressions to test.

- If/IfNot-Code is a block of code to execute when the IF/IFNOT Condition(s) is valid.

- ElseIf/ElseIfNot-Code is an optional block of code to execute when all the previous Condition(s) are invalid and the ELSEIF/ELSEIFNOT Condition(s) is value.

- Else-Code is an optional block of code to execute when all the previous Condition(s) are invalid.

Explanation

IF and IFNOT are part of three flow control constructs (IF/IFNOT, CASE/CASE_FAST, and REPEAT) that conditionally executes a block of code.  IF/IFNOT can optionally be combined with one or more ELSEIF and  ELSEIFNOT commands and/or one ELSE command to form sophisticated conditional structures.

IF and IFNOT tests Condition(s) and, if valid, executes the indented If/IfNot-Code block immediately below it.  If Condition(s) is invalid, the following optional ELSEIF Condition(s) and/or ELSEIFNOT Condition(s) are tested, in order, until a valid condition is found, then the associated ElseIf/ElseIfNot-Code block is executed.  The optional Else-Code block is executed if no previous valid conditions are found.

A “valid” condition is one that evaluates to TRUE for a positive conditional statement (IF or ELSEIF) or evaluates to FALSE for a negative conditional statement (IFNOT or ELSEIFNOT).  The remaining explanation below often uses the positive conditional (IF or ELSEIF) but similar concepts apply to the negative conditional (IFNOT or ELSEIFNOT).

#### Indention is Critical

IMPORTANT: Indention is critical.  The Spin2 language relies on indention (of one space or more) on lines following flow control constructs to determine if they belong to that construct or not.  Your code editor may also  visually indicate such groupings.  If using the Propeller Tool, Ctrl + I toggles block-group indicators on/off.

#### Simple IF Statement

The most common form of the IF command performs an action if, and only if, a condition is true.  This is written as an IF statement followed by one or more indented lines of code.  For example:

if X > 10	'If X is greater than 10

pintoggle(0)	'Toggle P0

pintoggle(1)	'Toggle P1

This example tests if X is greater than 10; if it is, I/O pin 0 is toggled.  Whether or not the IF condition was true, I/O pin P1 is toggled next.

Since the pintoggle(0) line is indented from the IF line, it belongs to the If-Code block and is executed only if the IF condition is true.  The next line, pintoggle(1), is not indented from the IF line, so it is executed next whether or not the IF’s Condition(s) was true.  Here’s another version of the same example:

if X > 10	'If X is greater than 10

pintoggle(0)	'Toggle P0

pintoggle(1)	'Toggle P1

waitms(1000)	'Wait for 1 second

This example is very similar to the first, except there are now two lines of code indented from the IF statement.  In this case, if X is greater than 10, P0 is toggled, then P1 is toggled, and finally the waitms line is executed.  If, however, X was not greater than 10, the pintoggle(0) and pintoggle(1) lines are skipped (since they are indented and part of the If-Code block) and the waitms line is executed (since it is not indented; it is not part of the If-Code block).

#### Combining Conditions

The Condition(s) field is evaluated as one single Boolean condition, but it can be made up of multiple expressions by combining them with the AND and OR operators.  For example:

if X > 10 AND X < 100	'If X greater than 10 and less than 100

This IF statement would be true if, and only if, X is greater than 10 and X is also less than 100.  In other words, it’s true if X is in the range 11 to 99.  Sometimes statements like these can be a little difficult to read.  To make it easier to read, parentheses may be used to group each sub-condition, such as with the following.

if (X > 10) AND (X < 100)	'If X greater than 10 and less than 100

#### Using IF with ELSE

The next common form of the IF command performs an action if a condition is true or a different action if that condition is false.  This is written as an IF statement followed by its If-Code block, then an ELSE followed by its Else-Code block, as shown below:

if X > 100	'If X is greater than 100

pintoggle(0)	'Toggle P0

else	'Else, X <= 100

pintoggle(1)	'Toggle P1

Here, if X is greater than 100 then I/O pin 0 is toggled, otherwise X must be less than or equal to 100 and I/O pin 1 is toggled.  This IF...ELSE construct, as written, always performs either a toggle on P0 or a toggle on P1; never both, and never neither.

Remember, the code that logically belongs to the If-Code or the Else-Code blocks must be indented from the IF or the ELSE, respectively, by at least one space.  Also note that the ELSE must be lined up horizontally with the IF statement; they must both begin on the same column so the compiler knows that the ELSE goes with that IF.

For every IF/IFNOT statement, there can be zero or one ELSE component.  ELSE, if included, must be the last component in an IF/IFNOT statement, appearing after all potential ELSEIF/ELSEIFNOTs.

#### Using IF with ELSEIF

IF can perform some action if a condition is true or a different action if that condition is false but another condition is true.  This is written as an IF statement followed by its If-Code block, then one or more ELSEIF statements followed by their respective ElseIf-Code blocks.  Here’s an example:

if X > 100	'If X is greater than 100

pintoggle(0)	'Toggle P0

elseif X == 90	'Else If X = 90

pintoggle(1)	'Toggle P1

Here, if X is greater than 100 then I/O pin 0 is toggled, otherwise if X equals 90 then I/O pin 1 is toggled, and if neither of those conditions are true, neither P0 nor P1 is toggled.  This is a slightly shorter way of writing the following code:

if X > 100	'If X is greater than 100

pintoggle(0)	'Toggle P0

else	'Otherwise,

if X == 90	'If X = 90

pintoggle(1)	'Toggle P1

Both of these examples perform the same actions, but the first is shorter and easier to read. Note that the ELSEIF, just like the ELSE, must be lined up with the same column as the IF that it is associated with.

Each IF conditional statement can have zero or more ELSEIF statements associated with it.  Look at the following:

if X > 100	'If X is greater than 100

pintoggle(0)	'Toggle P0

elseif X == 90	'Else If X = 90

pintoggle(1)	'Toggle P1

elseif X > 50	'Else If X > 50

pintoggle(2)	'Toggle P2

We have three conditions and three possible actions here.  Just like the previous example, if X is greater than 100, P0 is toggled, otherwise, if X equals 90, P1 is toggled, but if neither of those conditions are true and X is greater than 50, P2 is toggled.  If none of those conditions are true, then none of those actions would occur.

There is an important concept to note about this example: the conditions are evaluated in order, so a valid condition will execute and ignore the rest.  If we had arranged the two ELSEIFs so that the “X > 50” were checked first, we’d have a bug in our code.  Here's the code with that bug:

if X > 100	'If X is greater than 100

pintoggle(0)	'Toggle P0

elseif X > 50	'Else If X > 50

pintoggle(2)	'Toggle P2

elseif X == 90	'Else If X = 90 <-- ERROR, ABOVE CONDITION

pintoggle(1)	'Toggle P1      <-- SUPERSEDES THIS AND

'                   THIS CODE NEVER RUNS

The above example contains an error.  While X could be equal to 90, the elseif X == 90 statement would never be tested since the previous one, elseif X > 50, is tested first and is also true, so its block would be  executed and no further conditions of that IF structure are tested.  Also, if X were 50 or less, the last ELSEIF condition is tested, but of course, it will never be true.

#### Using IF with ELSEIF and ELSE

An enhancement to the above IF conditional structures performs an alternate action if none of the previous conditions were true. This is written as an IF, one or more ELSEIFs, and finally an ELSE.  Here’s an example:

if X > 100	'If X is greater than 100

pintoggle(0)	'Toggle P0

elseif X == 90	'Else If X = 90

pintoggle(1)	'Toggle P1

elseif X > 50	'Else If X > 50

pintoggle(2)	'Toggle P2

else	'Otherwise,

pintoggle(3)	'Toggle P3

In the above example, P3 is toggled if none of the prior IF or ELSEIF conditions are true.

#### The ELSEIFNOT Condition

The ELSEIFNOT condition behaves exactly like ELSEIF except that it uses negative logic; it executes its Elseif not-Code block only if its Condition(s) evaluates to FALSE.  Multiple ELSEIFNOT and ELSEIF conditions can be combined in a single IF conditional construct, in any order, between the IF and the optional ELSE.

## REPEAT

Command: Execute code block repetitively.

⦅PUB┆PRI⦆

REPEAT {Count}

{ ⇢ Code }

⦅PUB┆PRI⦆

REPEAT Count WITH Variable

{ ⇢ Code }

⦅PUB┆PRI⦆

REPEAT Variable FROM Start TO Finish {STEP Delta}

{ ⇢ Code }

⦅PUB┆PRI⦆

REPEAT ⦅UNTIL┆WHILE⦆ Condition(s)

{ ⇢ Code }

⦅PUB┆PRI⦆

REPEAT

{ ⇢ Code }

⦅UNTIL┆WHILE⦆ Condition(s)

- Count is an optional expression indicating the finite number of times to execute Code.  Count should be a positive value.  If Count is omitted, syntax 1 creates an infinite loop made up of Code.

- Code is an optional block of one or more lines of code to execute repeatedly.  Omitting Code is rare, but may be useful for an endless pause (syntax 1 without Count) to keep I/O pins active, or in syntax 4 and 5 when Condition(s) alone achieve the needed effects.  Indention is required for all statements in the block.

- Variable is a user-defined variable that will be iterated from 0 to Count-1 (syntax 2) or from Start to Finish and optionally by Delta units per iteration (syntax 3).  Variable can be used in Code to determine or utilize the iteration count.

- Start is an expression that determines the starting value of Variable in syntax 3.  If Start is less than Finish, Variable will be incremented each iteration; it will be decremented otherwise.

- Finish is an expression that determines the ending value of Variable in syntax 3.  If Finish is greater than Start, Variable will be incremented each iteration; it will be decremented otherwise.

- Delta is an optional expression that determines the units in which to increment/decrement Variable each iteration (syntax 3).  If omitted, Variable is incremented/decremented by 1 each iteration.

- Condition(s) is one or more Boolean expression(s) used by syntax 4 and 5 to continue or terminate the loop.  When preceded by UNTIL, Condition(s) terminates the loop when true.  When preceded by WHILE, Conditions(s) terminates the loop when false.

Explanation

REPEAT is the very flexible looping structure for Spin2 code.  It can be used to create any type of loop, including: infinite, finite, with/without loop counter, and conditional zero-to-many/one-to-many loops.

#### Indention is Critical

IMPORTANT: Indention is critical.  The Spin2 language relies on indention (of one space or more) on lines following flow control constructs to determine if they belong to that construct or not.  Your code editor may also  visually indicate such groupings.  If using the Propeller Tool, Ctrl + I toggles block-group indicators on/off.

#### Infinite Loops (Syntax 1 w/o Count)

Most forms of REPEAT can be infinite loops, but the form used most often for this purpose is syntax 1 without the Count field.  For example:

repeat	'Repeat endlessly

pintoggle(25)	'Toggle P25

waitms(1000)	'Pause for 1,000 ms

This code repeats the pintoggle(25) and waitms(1000) lines endlessly.  Both lines are indented from the REPEAT so they belong to the REPEAT loop.

Since Code is really an optional part of REPEAT, the REPEAT command by itself can be used as an endless loop that does nothing but keep the cog active— including its influence on I/O pins.  This can be intentional, but may be unintentional due to improper indentation.  For example:

repeat	'Repeat endlessly

pintoggle(25)	'Toggle P25       <-- This is never run

The above example is erroneous; the last line is never executed because the REPEAT above it is an endless loop that has no Code— there is nothing indented immediately below it, so the cog simply sits in an endless loop at the REPEAT line that does nothing but keep the cog active and consuming power.

#### Simple Finite Loops (Syntax 1)

Most loops are finite in nature; they execute a limited number of iterations only.  The simplest form is syntax 1 with the Count field included.

For example:

repeat 10	'Repeat 10 times

pintoggle(25)	'Toggle P25

waitms(500)	'Pause for 500 ms

byte[$7000]++	'Increment RAM location $7000

This code toggles P25 ten times, waiting 500 ms in-between, then increments the value in RAM location $7000.

Note that Count should be a positive value; negatives will yield unwanted results.  Also, the Count field may be any numeric expression but the expression is evaluated only once, the first time the loop is entered.  This means that any changes to the expression’s variables within the loop will not affect the number of iterations of the loop. The next example assumes the variable Index was created previously.

Index := 10	'Set loop to repeat 10 times

repeat Index	'Repeat Index times

pintoggle(25)	'Toggle P25

waitms(500)	'Pause for 500 ms

Index := 20	'Change Index to 20

In the above example, Index is 10 upon entering the REPEAT loop the first time.  Each time through the loop, however, Index is set to 20, but the loop continues to execute only 10 times.

#### Counted Finite Loops (Syntax 2 & 3)

Quite often it is necessary to count the loop iterations so the loop’s code can perform differently based on that count.  The REPEAT command makes it easy to do this with syntax 2 and 3.  The next example assumes the variable Index was created previously.

repeat 10 with Index	'Repeat 10 times using Index as the counter

byte[$7000][Index]++	'Increment RAM locations $7000 to $7009

The code above loops 10 times, adjusting the value in the variable Index on each iteration; starting at 0 and ending after 9.  The first time through the loop, Index will be 0 and each iteration afterwards Index will be 1 greater than the previous iteration: ..1, 2, 3…9.  After the tenth iteration, Index will be incremented to 10 and the loop will terminate, causing the next code following the REPEAT loop structure to execute, if any exists.  The code in the loop uses Index as an offset to affect memory, byte[$7000][Index]++; in this case it is incrementing each of the byte-sized values in RAM locations $7000 to $7009 by 1, one at a time.

Note that Count should be a positive value; negatives will yield unwanted results.

Syntax 3 serves a similar function as the example above, though with added flexibility.

repeat Index from 1 to 10	'Repeat 10 times

byte[$7000][Index]++	'Increment RAM locations $7000 to $7009

Like the previous example, the code above loops 10 times, adjusting the variable Index.  The difference is that syntax 3 allows starting at any arbitrary value; 1 in this case.  The first time through the loop, Index will be 1 (as indicated by the “from 1”) and continues iterating until Index equals 11.  Be careful of Start and Finish values; this example affects RAM locations $7001 to $7010 instead of the previous example's $7000 to $7009.

The REPEAT command automatically determines whether the range suggested by Start and Finish is increasing or decreasing.  Since the above example used 1 to 10, the range is an increasing range; adjusting Index by +1 every time.  To get the count to go backwards, simply reverse the Start and Finish values.  Here's an example:

repeat Index from 9 to 0	'Repeat 10 times

byte[$7000][Index]++	'Increment RAM $7009 down through $7000

This example also loops 10 times, but counts with Index from 9 down to 0; adjusting Index by -1 each time.  The contents of the loop still increments the values in RAM, but from locations $7009 down to $7000.  After the tenth iteration, Index will equal -1.

Since the Start and Finish fields can be expressions, they can contain variables.  The next example assumes that S and F are variables created previously.

S := 0

F := 9

repeat 2	'Repeat twice

repeat Index from S to F	'Repeat 10 times

byte[$7000][Index]++	'Increment RAM locations 7000..$7009

S := 9

F := 0

The above example uses a nested loop.  The outer loop (the first one) repeats 2 times.  The inner loop repeats with Index from S to F, which were previously set to 0 and 9, respectively.  The inner loop increments the values in RAM locations $7000 to $7009, in that order, because the inner loop is counting iterations from 0 to 9.  Then, the inner loop terminates (with Index being set to 10) and the last two lines set S to 9 and F to 0, effectively swapping the Start and Finish values.  Since this is still inside the outer loop, the outer loop then executes its contents again (for the second time) causing the inner loop to repeat with Index from 9 down to 0.  The inner loop increments the values in RAM locations $7009 to $7000, in that order (reverse of the previous time) and terminates with Index equaling -1.  The last two lines set S and F again, but the outer loop does not repeat a third time.

REPEAT loops don’t have to be limited to incrementing or decrementing by 1 either.  If the REPEAT command uses the optional STEP Delta syntax, it will increment or decrement the Variable by the Delta amount.  In the syntax 2 form, REPEAT is actually always using a Delta value, but when the “STEP Delta” component is omitted, it uses either +1 or -1 by default, depending on the range of Start and Finish.  The following example includes the optional Delta value to increment by 2.

repeat Index from 0 to 8 step 2	'Repeat 5 times

byte[$7000][Index]++	'Increment even RAM $7000 to $7008

Here, REPEAT loops five times, with Index set to 0, 2, 4, 6, and 8, respectively.  This code effectively increments every other RAM location (the even numbered locations) from $7000 to $7008 and terminates with Index equaling 10.

The Delta field can be positive or negative, regardless of the natural ascending/descending range of the Start and Finish values, and can even be adjusted within the loop to achieve interesting effects.  For example, assuming Index and D are previously defined variables, the following code sets Index to the following sequence: 5, 6, 6, 5, 3.

D := 2

repeat Index from 5 to 10 step D

--D

This loop started out with Index at 5 and a Delta (D) of +2.  But each iteration of the loop decrements D by one, so at the end of iteration 1, Index = 5 and D = +1.  Iteration 2 has Index = 6 and D = 0.  Iteration 3 has Index = 6 and D = -1.  Iteration 4 has Index = 5 and D = -2.  Iteration 5 has Index = 3 and D = -3.  The loop then terminates because Index plus Delta (3 + -3) is outside the range of Start to Finish (5 to 10).

#### Conditional Loops (Syntax 3 and 4)

The final forms of REPEAT, syntax 3 and 4, are finite loops with conditional exits and have flexible options allowing for the use of either positive or negative logic and the creation of zero-to-many or one-to-many iteration loops.  These two forms of REPEAT are usually referred to as “repeat while” or “repeat until” loops.

Let’s look at the REPEAT form described by syntax 3.  It consists of the REPEAT command followed immediately by either WHILE or UNTIL then Condition(s) and finally, on the lines below it, optional Code.  Since this form tests Condition(s) at the start of every iteration, it creates a zero-to-many loop; the Code block will execute zero or more times, depending on the Condition(s).  For example, assume that X is a variable created earlier:

X := 0

repeat while X < 10	'Repeat while X is less than 10

byte[$7000][X] := 0	'Increment RAM value

X++	'Increment X

This example first sets X to 0, then repeats the loop while X is less than 10.  The code inside the loop clears RAM locations based on X (starting at location $7000) and increments X.

After the 10th iteration of the loop, X equals 10, making the condition while X < 10 false and the loop terminates.

This loop is said to use “positive” logic because it continues “WHILE” a condition is true.  It could also be written with “negative” logic using UNTIL, instead.  Such as:

X := 0

repeat until X > 9	'Repeat until X is greater than 9

byte[$7000][X] := 0	'Increment RAM value

X++	'Increment X

The above example performs the same way as the previous, but the REPEAT loop uses negative logic because it continues “UNTIL” a condition is true; i.e: it continues while a condition is false.

In either example, if X was equal to 10 or higher before the first iteration of the REPEAT loop, the condition would cause the loop to never execute at all, which is why we call it a zero-to-many loop.

The REPEAT form described by syntax 4 is very similar to syntax 3, but the condition is tested at the end of every iteration, making it a one-to-many loop.  For example:

X := 0

repeat

byte[$7000][X] := 0	'Increment RAM value

X++	'Increment X

while X < 10	'Repeat while X is less than 10

This works the same as the previous examples, looping 10 times, except that the condition is not tested until the end of each iteration.  However, unlike the previous examples, even if X was equal to 10 or higher before the first iteration, the loop would run once then terminate, which is why we call it a one-to-many loop.

#### Other REPEAT Options

There are two other commands that affect the behavior of REPEAT loops: NEXT and QUIT.  See their respective descriptions for more information.

## RETURN

Command: Exit with normal status from PUB/PRI method with optional return Value(s).

⦅PUB┆PRI⦆

RETURN {Value(s)}

Returns: Either the method's current defined results, or Value(s) if provided.

- Value(s) is an optional set of one or more expressions whose values are to be returned from the method.  Up to 15 comma-delimited values can be provided, matching the defined set of PUB/PRI method results.

Explanation

RETURN is one of two commands (ABORT and RETURN) that immediately terminates a PUB or PRI method.

RETURN cau the PUB or PRI method to exit with normal status— meaning it pops the call stack once and returns to the direct caller of the method, optionally delivering one or more "return" values in the process.

Every PUB or PRI method has an implied RETURN at its end, so there's no need to manually place one; however, RETURN can be used in multiple places within the method to create multiple exit points— such as in IF or CASE statements.

When RETURN appears without the optional Value(s), it returns the current value(s) of the PUB/PRI's defined Result(s), which can be set or changed in many places within the method.  If Value(s) are provided with the RETURN, they are returned to the caller instead of the current Result(s).

#### About the Call Stack

When methods are called, a hidden mechanism keeps track of important information.  This mechanism is a “call stack;” an area of RAM used to store return addresses, return values, method parameters, and local variables for intermediate use.  As more methods are called, additional sets of information are stacked; the call stack grows taller.  As methods exit, this temporary information is removed and the call stack gets shorter.  This process is called "pushing” onto the stack and “popping” off of the stack, respectively.

The RETURN command pops the most recent data off the call stack to facilitate returning to the immediate caller; the one who directly called the method that just returned.

#### Using RETURN

The following example demonstrates two uses of RETURN.  Assume that DisplayDivByZeroError is a method defined elsewhere.

PUB Add(Num1, Num2): Sum

Sum:= Num1 + Num2	'Add Num1 + Num2

return

PUB Divide(Dividend, Divisor): Result

if Divisor == 0	'Check if Divisor = 0

DisplayDivByZeroError	'If so, display error

return 0	'and return with 0

return Dividend / Divisor	'Otherwise return quotient

The Add() method sets its Sum variable equal to Num1 plus Num2, then executes RETURN.  The RETURN causes Add() to return the value of Sum to the caller.  Note that this RETURN was not really required because the Compiler automatically adds it to the end of methods.

The Divide() method uses RETURN in a different way.  If Divisor equals zero, it calls a DisplayDivByZeroError() method and then executes return 0, so the method returns to the caller with the single value 0.  Otherwise, it executes return Dividend / Divisor, returning to the caller with the result of the division.  In this case, the last RETURN was used to perform the calculation and return the result all in one step rather than separately affecting the declared Result variable beforehand.

#### Multiple Return Values

PUB and PRI methods will return multiple values just by defining their method that way.  For example:

PUB Divide(Dividend, Divisor): Quotient, Remainder

Quotient := Dividend / Divisor	'Calculate quotient

Remainder := Dividend // Divisor	'Calculate remainder

Here, the Divide() method defines two results, Quotient and Remainder, and sets their values during internal division calculations.  The resulting values are automatically returned to the caller upon method exit.  However, the Divide method could also calculate and return the results in one RETURN statement, without ever explicitly setting the Quotient and Remainder result variables:

PUB Divide(Dividend, Divisor): Quotient, Remainder

return Dividend/Divisor, Dividend//Divisor	'Return quotient & remainder

Either way, all results will be returned in their defined order, and all must be specified if any are provided via the RETURN statement.  The caller can either ignore them all or receive them all.

Below is an example of a caller.  The Display() method calls Divide() and retrieves the two results by providing a comma-delimited list of local receiving variables in the proper order, Q, R := Divide(...).

PUB Display() | Q, R

Q, R := Divide(1023, 16)	'Call Divide() to get whole and part

debug(udec(Q), udec(R))	'Display results

When combined with one of the above and executed, Display() will send "Q = 63, R = 15" to the Debug Output window.

# Propeller 2 Reserved Words (Spin2 + PASM2)

Predefined symbols recognized by the compiler to have special meaning.

## _ (leading underscore)

| _C _C_AND_NZ _C_AND_Z _C_EQ_Z _C_NE_Z | _C_OR_NZ _C_OR_Z _CLKFREQ _CLKMODE _CLR | _E _GE _GT _LE _LT | _NC _NC_AND_NZ _NC_AND_Z _NC_OR_NZ _NC_OR_Z | _NE _NZ _NZ_AND_C _NZ_AND_NC _NZ_OR_C | _NZ_OR_NC _RET_ _SET _Z _Z_AND_C | _Z_AND_NC _Z_EQ_C _Z_NE_C _Z_OR_C _Z_OR_NC |
| --- | --- | --- | --- | --- | --- | --- |

## A - B

| ABORT ABS ADD ADDBITS ADDCT1 ADDCT2 | ADDCT3 ADDPINS ADDPIX ADDS ADDSX ADDX | AKPIN ALIGNL ALIGNW ALLOWI ALT ALTB | ALTD ALTGB ALTGN ALTGW ALTI ALTR | ALTS ALTSB ALTSN ALTSW AND ANDC | ANDN ANDZ ARCHIVE ASMCLK AUGD AUGS | AUTO BACKCOLOR BITC BITH BITL BITMAP | BITNC BITNOT BITNZ BITRND BITZ BLACK | BLNPIX BLUE BMASK BOOL BOOL_ BOX | BRK BYTE BYTECOMP BYTEFILL BYTEFIT BYTEMOVE | BYTESWAP BYTES_1BIT BYTES_2BIT BYTES_4BIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## C - D

| CALL CALLA CALLB CALLD CALLPA CALLPB CARTESIAN CASE CASE_FAST CHANNEL CIRCLE CLEAR | CLKFREQ CLKFREQ_ CLKMODE CLKMODE_ CLKSET CLOSE CMP CMPM CMPR CMPS CMPSUB CMPSX | CMPX COGATN COGBRK COGCHK COGEXEC COGEXEC_NEW COGEXEC_NEW_PAIR COGID COGINIT COGSPIN COGSTOP COLOR | COMP CON COPY CRCBIT CRCNIB CYAN DAT DEBUG DEBUG_BAUD DEBUG_COGINIT DEBUG_COGS DEBUG_DELAY | DEBUG_DISPLAY_LEFT DEBUG_DISPLAY_TOP DEBUG_HEIGHT DEBUG_LEFT DEBUG_LOG_SIZE DEBUG_MAIN DEBUG_PIN_TX DEBUG_PIN_RX DEBUG_TIMESTAMP DEBUG_TOP DEBUG_WIDTH DEBUG_WINDOWS_OFF | DECMOD DECOD DEPTH DEV DIRA DIRB DIRC DIRH DIRL DIRNC DIRNOT DIRNZ | DIRRND DIRZ DJF DJNF DJNZ DJZ DLY DOT DOTSIZE DOWNLOAD_BAUD DRVC DRVH | DRVL DRVNC DRVNOT DRVRND DRVZ |
| --- | --- | --- | --- | --- | --- | --- | --- |

## E - F

| ELSE ELSEIF ELSEIFNOT ENCOD END | EVENT_ATN EVENT_CT1 EVENT_CT2 EVENT_CT3 EVENT_FBW | EVENT_INT EVENT_PAT EVENT_QMT EVENT_SE1 EVENT_SE2 | EVENT_SE3 EVENT_SE4 EVENT_XFI EVENT_XMT EVENT_XRL | EVENT_XRO EXECF FABS FALSE FBLOCK | FDEC FDEC_ FDEC_ARRAY FDEC_ARRAY_ FDEC_REG_ARRAY | FDEC_REG_ARRAY_ FFT FGE FGES FILE | FIT FIELD FILL FLE FLES | FLOAT FLTC FLTH FLTL FLTNC | FLTNOT FLTNZ FLTRND FLTZ FRAC | FROM FSQRT FVAR FVARS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## G - H

| GETBRK GETBYTE GETCRC | GETCT GETMS GETNIB | GETPTR GETQX GETQY | GETREGS GETRND GETSCP | GETSEC GETWORD GETXACC | GRAY GREEN GREY | HIDEXY HOLDOFF HSV16 | HSV16W HSV16X HSV8 | HSV8W HSV8X HUBEXEC | HUBEXEC_NEW HUBEXEC_NEW_PAIR HUBSET |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## I - J

| IF IF_00 IF_0000 IF_0001 IF_0010 IF_0011 IF_01 IF_0100 IF_0101 IF_0110 IF_0111 IF_0X | IF_10 IF_1000 IF_1001 IF_1010 IF_1011 IF_11 IF_1100 IF_1101 IF_1110 IF_1111 IF_1X IF_A | IF_AE IF_ALWAYS IF_B IF_BE IF_C IF_C_AND_NZ IF_C_AND_Z IF_C_EQ_Z IF_C_NE_Z IF_C_OR_NZ IF_C_OR_Z IF_DIFF | IF_E IF_GE IF_GT IF_LE IF_LT IF_NC IF_NC_AND_NZ IF_NC_AND_Z IF_NC_OR_NZ IF_NC_OR_Z IF_NE IF_NOT_00 | IF_NOT_01 IF_NOT_10 IF_NOT_11 IF_NZ IF_NZ_AND_C IF_NZ_AND_NC IF_NZ_OR_C IF_NZ_OR_NC IF_SAME IF_X0 IF_X1 IF_Z | IF_Z_AND_C IF_Z_AND_NC IF_Z_EQ_C IF_Z_NE_C IF_Z_OR_C IF_Z_OR_NC IFNOT IJMP1 IJMP2 IJMP3 IJNZ IJZ | INA INB INCMOD INT_OFF IRET1 IRET2 IRET3 JATN JCT1 JCT2 JCT3 JFBW | JINT JMP JMPREL JNATN JNCT1 JNCT2 JNCT3 JNFBW JNINT JNPAT JNQMT JNSE1 | JNSE2 JNSE3 JNSE4 JNXFI JNXMT JNXRL JNXRO JPAT JQMT JSE1 JSE2 JSE3 | JSE4 JXFI JXMT JXRL JXRO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## L - M

| LINE LINESIZE LOC LOCKCHK LOCKNEW LOCKREL | LOCKRET LOCKTRY LOGIC LOGSCALE LONG LONGCOMP | LONGFILL LONGMOVE LONGSWAP LONGS_16BIT LONGS_1BIT LONGS_2BIT | LONGS_4BIT LONGS_8BIT LOOKDOWN LOOKDOWNZ LOOKUP LOOKUPZ | LSTR LSTR_ LSTRING LUMA8 LUMA8W LUMA8X | LUT1 LUT2 LUT4 LUT8 LUTCOLORS MAG | MAGENTA MERGEB MERGEW MIDI MIXPIX MODC | MODCZ MODZ MOV MOVBYTS MUL MULDIV64 | MULPIX MULS MUXC MUXNC MUXNIBS MUXNITS | MUXNZ MUXQ MUXZ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## N - O

| NAN NEG NEGC NEGNC | NEGNZ NEGX NEGZ NEWCOG | NEXT NIXINT1 NIXINT2 NIXINT3 | NOP NOT OBJ OBOX | ONES OPACITY OR ORANGE | ORC ORG ORGF ORGH | ORIGIN ORZ OTHER OUTA | OUTB OUTC OUTH OUTL | OUTNC OUTNOT OUTNZ OUTRND | OUTZ OVAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## P

| P_ADC P_ADC_100X P_ADC_10X P_ADC_1X P_ADC_30X P_ADC_3X P_ADC_EXT P_ADC_FLOAT P_ADC_GIO P_ADC_SCOPE P_ADC_VIO P_AND_AB P_ASYNC_IO P_ASYNC_RX P_ASYNC_TX P_BITDAC P_CHANNEL P_COMPARE_AB P_COMPARE_AB_FB P_COUNT_HIGHS | P_COUNT_RISES P_COUNTER_HIGHS P_COUNTER_PERIODS P_COUNTER_TICKS P_DAC_124R_3V P_DAC_600R_2V P_DAC_75R_2V P_DAC_990R_3V P_DAC_DITHER_PWM P_DAC_DITHER_RND P_DAC_NOISE P_EVENTS_TICKS P_FILT0_AB P_FILT1_AB P_FILT2_AB P_FILT3_AB P_HIGH_100UA P_HIGH_10UA P_HIGH_150K P_HIGH_15K | P_HIGH_1K5 P_HIGH_1MA P_HIGH_FAST P_HIGH_FLOAT P_HIGH_TICKS P_INVERT_A P_INVERT_B P_INVERT_IN P_INVERT_OUT P_INVERT_OUTPUT P_LEVEL_A P_LEVEL_A_FBN P_LEVEL_B_FBN P_LEVEL_B_FBP P_LOCAL_A P_LOCAL_B P_LOGIC_A P_LOGIC_A_FB P_LOGIC_B_FB P_LOW_100UA | P_LOW_10UA P_LOW_150K P_LOW_15K P_LOW_1K5 P_LOW_1MA P_LOW_FAST P_LOW_FLOAT P_MINUS1_A P_MINUS1_B P_MINUS2_A P_MINUS2_B P_MINUS3_A P_MINUS3_B P_NCO_DUTY P_NCO_FREQ P_NORMAL P_OE P_OR_AB P_OUTBIT_A P_OUTBIT_B | P_PASS_AB P_PERIODS_HIGHS P_PERIODS_TICKS P_PLUS1_A P_PLUS1_B P_PLUS2_A P_PLUS2_B P_PLUS3_A P_PLUS3_B P_PULSE P_PWM_SAWTOOTH P_PWM_SMPS P_PWM_TRIANGLE P_QUADRATURE P_REG_UP P_REG_UP_DOWN P_REPOSITORY P_SCHMITT_A P_SCHMITT_A_FB P_SCHMITT_B_FB | P_STATE_TICKS P_SYNC_IO P_SYNC_RX P_SYNC_TX P_TRANSITION P_TRUE_A P_TRUE_B P_TRUE_IN P_TRUE_OUT P_TRUE_OUTPUT P_TT_00 P_TT_01 P_TT_10 P_TT_11 P_USB_PAIR P_XOR_AB PA PB PC_KEY PC_MOUSE | PI PINCLEAR PINF PINFLOAT PINH PINHIGH PINL PINLOW PINR PINREAD PINSTART PINT PINTOGGLE PINW PINWRITE PLOT POLAR POLLATN POLLCT POLLCT1 | POLLCT2 POLLCT3 POLLFBW POLLINT POLLPAT POLLQMT POLLSE1 POLLSE2 POLLSE3 POLLSE4 POLLXFI POLLXMT POLLXRL POLLXRO POLXY POP POPA POPB POS POSX | PR0 PR1 PR2 PR3 PR4 PR5 PR6 PR7 PRECISE PRECOMPILE PRI PTRA PTRB PUB PUSH PUSHA PUSHB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Q - R

| QCOS QDIV QEXP QFRAC QLOG QMUL | QROTATE QSIN QSQRT QUIT QVECTOR RANGE | RATE RCL RCR RCZL RCZR RDBYTE | RDFAST RDLONG RDLUT RDPIN RDWORD RECV | RED REG REGEXEC REGLOAD REP REPEAT | RES RESI0 RESI1 RESI2 RESI3 RET | RETA RETB RETI0 RETI1 RETI2 RETI3 | RETURN REV RFBYTE RFLONG RFVAR RFVARS | RFWORD RGB16 RGB24 RGB8 RGBEXP RGBI8 | RGBI8W RGBI8X RGBSQZ ROL ROLBYTE ROLNIB | ROLWORD ROR ROTXY ROUND RQPIN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## S - T

| SAL SAMPLES SAR SAVE SBIN SBIN_ SBIN_BYTE SBIN_BYTE_ SBIN_BYTE_ARRAY SBIN_BYTE_ARRAY_ SBIN_LONG SBIN_LONG_ SBIN_LONG_ARRAY SBIN_LONG_ARRAY_ SBIN_REG_ARRAY SBIN_REG_ARRAY_ SBIN_WORD SBIN_WORD_ SBIN_WORD_ARRAY SBIN_WORD_ARRAY_ | SCA SCAS SCOPE SCOPE_XY SCROLL SDEC SDEC_ SDEC_BYTE SDEC_BYTE_ SDEC_BYTE_ARRAY SDEC_BYTE_ARRAY_ SDEC_LONG SDEC_LONG_ SDEC_LONG_ARRAY SDEC_LONG_ARRAY_ SDEC_REG_ARRAY SDEC_REG_ARRAY_ SDEC_WORD SDEC_WORD_ SDEC_WORD_ARRAY | SDEC_WORD_ARRAY_ SEND SET SETBYTE SETCFRQ SETCI SETCMOD SETCQ SETCY SETD SETDACS SETINT1 SETINT2 SETINT3 SETLUTS SETNIB SETPAT SETPIV SETPIX SETQ | SETQ2 SETR SETREGS SETS SETSCP SETSE1 SETSE2 SETSE3 SETSE4 SETWORD SETXFRQ SEUSSF SEUSSR SHEX SHEX_ SHEX_BYTE SHEX_BYTE_ SHEX_BYTE_ARRAY SHEX_BYTE_ARRAY_ SHEX_LONG | SHEX_LONG_ SHEX_LONG_ARRAY SHEX_LONG_ARRAY_ SHEX_REG_ARRAY SHEX_REG_ARRAY_ SHEX_WORD SHEX_WORD_ SHEX_WORD_ARRAY SHEX_WORD_ARRAY_ SHL SHR SIGNED SIGNX SIZE SKIP SKIPF SPACING SPARSE SPECTRO SPLITB | SPLITW SPRITE SPRITEDEF SQRT STALLI STEP STRCOPY STRCOMP STRING STRSIZE SUB SUBR SUBS SUBSX SUBX SUMC SUMNC SUMNZ SUMZ SWAP | TERM TEST TESTB TESTBN TESTN TESTP TESTPN TEXT TEXTANGLE TEXTSIZE TEXTSTYLE TITLE TJF TJNF TJNS TJNZ TJS TJV TJZ TO | TRACE TRGINT1 TRGINT2 TRGINT3 TRIGGER TRUE TRUNC |
| --- | --- | --- | --- | --- | --- | --- | --- |

## U, V, W

| UBIN UBIN_ UBIN_BYTE UBIN_BYTE_ UBIN_BYTE_ARRAY UBIN_BYTE_ARRAY_ UBIN_LONG UBIN_LONG_ UBIN_LONG_ARRAY UBIN_LONG_ARRAY_ UBIN_REG_ARRAY UBIN_REG_ARRAY_ UBIN_WORD UBIN_WORD_ UBIN_WORD_ARRAY | UBIN_WORD_ARRAY_ UDEC UDEC_ UDEC_BYTE UDEC_BYTE_ UDEC_BYTE_ARRAY UDEC_BYTE_ARRAY_ UDEC_LONG UDEC_LONG_ UDEC_LONG_ARRAY UDEC_LONG_ARRAY_ UDEC_REG_ARRAY UDEC_REG_ARRAY_ UDEC_WORD UDEC_WORD_ | UDEC_WORD_ARRAY UDEC_WORD_ARRAY_ UHEX UHEX_ UHEX_BYTE UHEX_BYTE_ UHEX_BYTE_ARRAY UHEX_BYTE_ARRAY_ UHEX_LONG UHEX_LONG_ UHEX_LONG_ARRAY UHEX_LONG_ARRAY_ UHEX_REG_ARRAY UHEX_REG_ARRAY_ UHEX_WORD | UHEX_WORD_ UHEX_WORD_ARRAY UHEX_WORD_ARRAY_ UNTIL UPDATE VAR VARBASE WAITATN WAITCT WAITCT1 WAITCT2 WAITCT3 WAITFBW WAITINT WAITMS | WAITPAT WAITSE1 WAITSE2 WAITSE3 WAITSE4 WAITUS WAITX WAITXFI WAITXMT WAITXRL WAITXRO WC WCZ WFBYTE WFLONG | WFWORD WHILE WHITE WINDOW WITH WMLONG WORD WORDCOMP WORDFILL WORDSWAP WORDFIT WORDMOVE WORDS_1BIT WORDS_2BIT WORDS_4BIT | WORDS_8BIT WRBYTE WRC WRFAST WRLONG WRLUT WRNC WRNZ WRPIN WRWORD WRZ WXPIN WYPIN WZ |
| --- | --- | --- | --- | --- | --- | --- |

## X, Y, Z

| X_16P_2DAC8_WFWORD X_16P_4DAC4_WFWORD X_1ADC8_0P_1DAC8_WFBYTE X_1ADC8_8P_2DAC8_WFWORD X_1P_1DAC1_WFBYTE X_2ADC8_0P_2DAC8_WFWORD X_2ADC8_16P_4DAC8_WFLONG X_2P_1DAC2_WFBYTE X_2P_2DAC1_WFBYTE X_32P_4DAC8_WFLONG X_4ADC8_0P_4DAC8_WFLONG X_4P_1DAC4_WFBYTE X_4P_2DAC2_WFBYTE X_4P_4DAC1_WFBYTE X_8P_1DAC8_WFBYTE X_8P_2DAC4_WFBYTE | X_8P_4DAC2_WFBYTE X_ALT_OFF X_ALT_ON X_DACS_0_0_0_0 X_DACS_0_0_X_X X_DACS_0_X_X_X X_DACS_0N0_0N0 X_DACS_0N0_X_X X_DACS_1_0_1_0 X_DACS_1_0_X_X X_DACS_1N1_0N0 X_DACS_3_2_1_0 X_DACS_OFF X_DACS_X_0_X_X X_DACS_X_X_0_0 X_DACS_X_X_0_X | X_DACS_X_X_0N0 X_DACS_X_X_1_0 X_DACS_X_X_X_0 X_DDS_GOERTZEL_SINC1 X_DDS_GOERTZEL_SINC2 X_IMM_16X2_1DAC2 X_IMM_16X2_2DAC1 X_IMM_16X2_LUT X_IMM_1X32_4DAC8 X_IMM_2X16_2DAC8 X_IMM_2X16_4DAC4 X_IMM_32X1_1DAC1 X_IMM_32X1_LUT X_IMM_4X8_1DAC8 X_IMM_4X8_2DAC4 X_IMM_4X8_4DAC2 | X_IMM_4X8_LUT X_IMM_8X4_1DAC4 X_IMM_8X4_2DAC2 X_IMM_8X4_4DAC1 X_IMM_8X4_LUT X_PINS_OFF X_PINS_ON X_RFBYTE_1P_1DAC1 X_RFBYTE_2P_1DAC2 X_RFBYTE_2P_2DAC1 X_RFBYTE_4P_1DAC4 X_RFBYTE_4P_2DAC2 X_RFBYTE_4P_4DAC1 X_RFBYTE_8P_1DAC8 X_RFBYTE_8P_2DAC4 X_RFBYTE_8P_4DAC2 | X_RFBYTE_LUMA8 X_RFBYTE_RGB8 X_RFBYTE_RGBI8 X_RFLONG_16X2_LUT X_RFLONG_32P_4DAC8 X_RFLONG_32X1_LUT X_RFLONG_4X8_LUT X_RFLONG_8X4_LUT X_RFLONG_RGB24 X_RFWORD_16P_2DAC8 X_RFWORD_16P_4DAC4 X_RFWORD_RGB16 X_WRITE_OFF X_WRITE_ON XCONT XINIT | XOR XORC XORO32 XORZ XSTOP XYPOL XZERO YELLOW ZEROX ZSTR ZSTR_ |
| --- | --- | --- | --- | --- | --- |

# CHANGE LOG

| Date | Notes |
| --- | --- |
| 6/7/2024 | First public release. |

# PARALLAX INCORPORATED

| Parallax Inc. 599 Menlo Drive, Suite 100 Rocklin, CA 95765 USA | Office: +1 916-624-8333 Toll Free US: 888-512-1024 | sales@parallax.com support@parallax.com | www.parallax.com/p2 forums.parallax.com |
| --- | --- | --- | --- |
| Purchase of the P2X8C4M64P does not include any license to emulate any other device nor to communicate via any specific proprietary protocol; P2X8C4M64P connectivity objects and code examples provided or referenced by Parallax, Inc. are NOT licensed and are provided for research and development purposes only; end users must seek permission to use licensed protocols for their applications and products from the protocol license holders. Parallax Inc. makes no warranty, representation or guarantee regarding the suitability of its products for any particular purpose, nor does Parallax Inc. assume any liability arising out of the application or use of any product, and specifically disclaims any and all liability, including without limitation consequential or incidental damages even if Parallax Inc. has been advised of the possibility of such damages. Copyright © 2024 Parallax Inc. All rights are reserved. Parallax, the Parallax logo, the P2 logo, and Propeller are trademarks of Parallax Inc. |  |  |  |