#### COPYRIGHTS AND TRADEMARKS

This documentation is copyright © 2022 by Parallax Inc. By downloading or obtaining a printed copy of this documentation or software you agree that it is to be used with, or with products containing, the Parallax Propeller 2 P2X8C4M64P microcontroller. Any other uses are not permitted and may represent a violation of Parallax copyrights, legally punishable according to Federal copyright or intellectual property laws. Any duplication of this documentation for commercial uses is expressly prohibited by Parallax Inc. Duplication for educational use is permitted, subject to the following Conditions of Duplication: Parallax Inc. grants the user a conditional right to download, duplicate, and distribute this text without Parallax's permission. This right is based on the following conditions: the text, or any portion thereof, may not be duplicated for commercial use; it may be duplicated only for educational purposes when used solely in conjunction with Parallax products, and the user may recover from the student only the cost of duplication.

Parallax, Propeller Spin, and the Parallax logos are trademarks of Parallax Inc. If you decide to use any trademarks of Parallax Inc. on your web page or in printed material, you must state that (trademark) is a trademark of Parallax Inc.” upon the first appearance of the trademark name in each printed document or web page. Other brand and product names herein are trademarks or registered trademarks of their respective holders.

#### DISCLAIMER OF LIABILITY

Parallax, Inc. makes no warranty, representation or guarantee regarding the suitability of its products for any particular purpose, nor does Parallax, Inc. assume any liability arising out of the application or use of any product, and specifically disclaims any and all liability, including without limitation consequential or incidental damages even if Parallax, Inc. has been advised of the possibility of such damages.

#### INTERNET DISCUSSION LISTS

We maintain active web-based discussion forums for people interested in Parallax Propeller products, at forums.parallax.com.

#### ERRATA

While great effort is made to assure the accuracy of our texts, errors may still exist. If you find an error, please let us know by commenting/suggesting on live documentation, or by sending an email to editor@parallax.com. We continually strive to improve all of our educational materials and documentation, and frequently revise our texts. Occasionally, an errata sheet with a list of known errors and corrections for a given text will be posted to our website, www.parallax.com. Please check the individual product page’s free downloads for an errata file.

#### SUPPORTED HARDWARE AND FIRMWARE

This manual is valid with the following hardware and firmware versions:

| ###### Hardware | ###### Firmware |
| --- | --- |
| P2X8C4M64P | Rev B/C |

#### CREDITS

Authorship: Jeff Martin • Format & Editing: Stephanie Lindsay • Technical Graphics: Michael Mulholland

With many thanks to everyone in the Propeller Community and staff at Parallax Inc.

# TABLE OF CONTENTS

PREFACE	10

CONVENTIONS	10

ASSEMBLY LANGUAGE REFERENCE	11

Multi-Long ADD/SUB/CMP Operations	11

Adding Two Multi-Long Values	11

Subtracting Two Multi-Long Values	12

Comparing Two Multi-Long Values	14

Categorical Listing Of Propeller 2 Assembly Language	15

Directives	15

Conditions	15

Effects	16

Flag Modification	17

Augmentation	18

Indirection	18

Configuration	18

Cog Control	18

Process Control	18

Flow Control	19

Lookup Table (LUT) Memory Access	19

Hub Memory Access	20

Streamer	20

I/O Pins	20

Math	21

Timing	23

Event Handling	23

Interrupts	25

Bit Operations	25

Color Manipulation	26

Miscellaneous	27

Registers	27

Constants	27

Operators	28

Assembly Language Elements	29

Term Definitions	29

Opcode Tables	30

Propeller Assembly Instruction Master Table	31

ABS	31

ADD	32

ADDCT1/2/3	32

ADDPIX	33

ADDS	33

ADDSX	34

ADDX	34

AKPIN	35

ALIGNL	35

ALIGNW	36

ALLOWI	38

ALTB	38

ALTD	39

ALTGB	40

ALTGN	41

ALTGW	42

ALTI	43

ALTR	45

ALTS	46

ALTSB	46

ALTSN	47

ALTSW	48

AND / ANDN	49

AUGD	50

AUGS	51

BITC / BITNC	52

BITH / BITL	52

BITNOT	53

BITRND	54

BITZ / BITNZ	54

BMASK	55

BRK	55

CALL	56

CALLA / CALLB	57

CALLD	58

CALLPA / CALLPB	59

CMP	59

CMPM	60

CMPR	60

CMPS	61

CMPSUB	61

CMPSX	62

CMPX	62

COGATN	63

COGBRK	64

COGID	64

COGINIT	64

COGSTOP	67

Conditions ( IF_x )	67

DECMOD	69

DECOD	69

DIRC / DIRNC	70

DIRH / DIRL	71

DIRNOT	72

DIRRND	72

DIRZ / DIRNZ	73

DJF / DJNF	74

DJZ / DJNZ	74

DRVC / DRVNC	75

DRVH / DRVL	76

DRVNOT	77

DRVRND	77

DRVZ / DRVNZ	78

ENCOD	79

Effects ( WC, WZ, WCZ, ANDC, etc. )	80

FGE	81

FGES	81

FLE	82

FLES	82

FLTC / FLTNC	83

FLTH / FLTL	84

FLTNOT	85

FLTRND	85

FLTZ / FLTNZ	86

GETBYTE	87

GETNIB	88

GETWORD	88

IF_x (Conditions)	89

IJZ / IJNZ	89

INCMOD	89

JATN / JNATN	90

JCT1/2/3 / JNCT1/2/3	91

JFBW / JNFBW	91

JINT / JNINT	92

JPAT / JNPAT	93

JQMT / JNQMT	93

JSE1/2/3/4 / JNSE1/2/3/4	94

JXFI / JNXFI	95

JXMT / JNXMT	95

JXRL / JNXRL	96

JXRO / JNXRO	96

MODC / MODZ / MODCZ	97

MOV	99

MUL	99

MULS	100

MUXC / MUXNC	100

MUXNIBS	101

MUXNITS	101

MUXQ	101

MUXZ / MUXNZ	102

NEG	102

NEGC	103

NEGNC	103

NEGNZ	104

NEGZ	105

NOP	105

NOT	105

OR	106

ONES	106

OUTC / OUTNC	107

OUTH / OUTL	108

OUTNOT	108

OUTRND	109

OUTZ / OUTNZ	110

POLLATN	111

POLLCT1/2/3	111

POLLFBW	112

POLLINT	112

POLLPAT	113

POLLQMT	113

POLLSE1/2/3/4	114

POLLXFI	114

POLLXMT	115

POLLXRL	115

POLLXRO	116

RCL	116

RCR	117

RCZL	117

RCZR	118

Registers	118

REV	119

ROL	119

ROLBYTE	119

ROLNIB	120

ROLWORD	121

ROR	121

SAL	122

SAR	122

SCA	123

SCAS	123

SETBYTE	124

SETD	124

SETNIB	125

SETR	126

SETS	126

SETWORD	127

SHL	128

SHR	128

SIGNX	129

STALLI	129

SUB	130

SUBR	130

SUBS	131

SUBSX	131

SUBX	132

SUMC / SUMNC	132

SUMZ / SUMNZ	133

TEST	134

TESTB / TESTBN	134

TESTN	135

TESTP / TESTPN	136

TJF / TJNF	136

TJS / TJNS	137

TJV	138

TJZ / TJNZ	138

WAITATN	139

WAITCT1/2/3	139

WAITFBW	140

WAITINT	141

WAITPAT	141

WAITSE1/2/3/4	142

WAITXFI	143

WAITXMT	143

WAITXRL	144

WAITXRO	145

WRC / WRNC	145

WRZ / WRNZ	146

XOR	146

ZEROX	146

PROPELLER 2 ASSEMBLY LANGUAGE (PASM2) IN BRIEF	148

PROPELLER 2 RESERVED WORDS (SPIN2 + PASM2)	160

CHANGE LOG	162

PARALLAX INCORPORATED	162

# PREFACE

This guide is an in-depth description of the Propeller 2 PASM2 language (Propeller 2 Assembly Language).  It serves as a full reference to the language and techniques for its use.

For additional documentation and resources, including programming tools, visit www.parallax.com/P2. The latest version of this guide, along with links to a commentable Google Doc version, are available from the Documentation section. In addition, there are links to other references for the Propeller 2 and its Spin2 language, which may include commentable Google Docs.

# CONVENTIONS

- % - indicates a binary number (containing the digits 0 and 1, and underscore "_" characters)

  - ex: %0101 and %11000111

- $ - indicates either a hexadecimal number (containing the digits 0–9, A–F, and underscore "_" characters) or the current address (the address of the current instruction)

  - ex: $2AF and $0D816

  - ex: $ (in an instruction's destination or source field)

- _ - (underscore) is a visual separator in numbers and in certain symbols.  Occasionally they are used as the leading or trailing character of a symbol to make it unique while keeping it similar to a same-named symbol that may already exist, or may be used in a syntax description to separate words of an all-lowercase phrase meant to be replaced by the reader when typing.  In numbers (i.e. decimal, hexadecimal, and binary values) they are group indicators that may separate natural boundaries (like groups of 3 digits in decimal or 8 bits in binary) or may separate context-specific fields of a value (like smart mode bits and drive level bits in an I/O pin configuration value)

  - ex: %00111010_01011101  and  $4C1F_0D816  and  2_328_476

  - ex: <low_byte>  and  IF_C_AND_Z  and  ZSTR_

- x - indicates a group of symbols where the x-part can vary

  - ex: INx means both INA and INB

  - ex: RDxxxx / WRxxx means RDBYTE, RDWORD, etc. and WRBYTE, WRWORD, etc.

-- or --

x - indicates a don't care bit in a binary number (a bit that can be 0 or 1 without affecting the execution context nor the explanation)

  - ex: %x_1_xxx0 and %xxxx10

- ALL CAPS or ALL_CAPS - indicates the item has special meaning, such as a reference to a label in a diagram, or a component in the Propeller 2, or is a predefined symbol (reserved word) such as a command, condition, register name, etc.

- <all_lowercase> - indicates an item meant to be completely replaced by the user when typing code; the phrase in brackets "<>" describes the intent of the specific item.

- {item} or { item1 | item2 | item3 } - denotes an optional item or items.  Either zero or one of the items can exist at the noted place; don't type the brackets "{}" or pipe symbol "|".

- item1 | item2 | item3 - denotes a single item, chosen from the list, that must be at the noted place; don't type the pipe symbol "|".

# ASSEMBLY LANGUAGE REFERENCE

## Multi-Long ADD/SUB/CMP Operations

Many useful integer quantities fit within 32 bits of space (0 to 4,294,967,295 or -2,147,483,648 to +2,147,483,647).  This wide range means the majority of integer add/subtract/compare operations are achieved in just one step using a single 32-bit signed or unsigned math instruction; a single-long operation.  However, if any such operation will use or create values larger than 32 bits, multiple math instructions must be chained together to form a multi-long "extended" operation; two instructions for 64-bits, three instructions for 96-bits, etc.

### Adding Two Multi-Long Values

To add two 64-bit values, use an unsigned ADD instruction (regardless of the values' unsigned or signed nature) followed by either an unsigned-extended ADDX or signed-extended ADDSX instruction, for unsigned or signed numbers, respectively.  If the two values are larger than 64 bits in size (regardless of their signed/unsigned format) insert one ADDX instruction per extra 32-bits in-between the first and last instruction.

Make sure to also use the WC or WCZ effect on the leading ADD and ADDX instructions so that the final ADDX or ADDSX instruction works properly.

#### 64-bit Unsigned Add

An unsigned double-long (64-bit) addition looks like this:

add   XLow,  YLow    wcz	'Add low longs; save C (required) and Z (optional)

addx  XHigh, YHigh   wcz	'Add high longs; save C and Z (both optional)

Note: the optional flag effects are only required if the intent is to monitor for zero or an unsigned overflow.

After executing the above, the full double-long (64-bit) result is in the registers XHigh:XLow. If XHigh:XLow started out as $0000_0000:FFFF_FFFF (4,294,967,295) and YHigh:YLow was $0000_0000:0000_0001 (1), the result in XHigh:XLow would be $0000_0001:0000_0000 (4,294,967,296). This is demonstrated below.

Hexadecimal                  Decimal

(high)     (low)

(XHigh:XLow)     $0000_0000:FFFF_FFFF           4,294,967,295

+ (YHigh:YLow)   + $0000_0000:0000_0001         +             1

───────────────────        ──────────────

= $0000_0001:0000_0000         = 4,294,967,296

An unsigned triple-long (96-bit) addition would look similar but with another ADDX instruction inserted between the ADD and ADDX instructions:

add   XLow,  YLow    wcz	'Add low longs; save C (required) and Z (optional)

addx  XMid,  YMid    wcz	'Add middle longs; save C (required) and Z (optional)

addx  XHigh, YHigh   wcz	'Add high longs; save C and Z (both optional)

During this multi-step operation, the Z flag always indicates if the result is turning out to be zero, but the C flag indicates unsigned carries until the final ADDX instruction, in which it indicates unsigned overflow (if any).

#### 64-bit Signed Add

In comparison to the above, a signed double-long (64-bit) addition uses an ADDSX instead of ADDX as the last instruction:

add   XLow,  YLow    wcz	'Add low longs; save C (required) and Z (optional)

addsx XHigh, YHigh   wcz	'Add high longs; save C and Z (both optional)

Note: the optional flag effects are only required if the intent is to monitor for zero or the final sign.

After executing the above, the full double-long (64-bit) result is in the registers XHigh:XLow.  If XHigh:XLow started out as $0000_0001:0000_0000 (4,294,967,296) and YHigh:YLow was $FFFF_FFFF:FFFF_FFFF (-1), the result in XHigh:XLow would be $0000_0000:FFFF_FFFF (4,294,967,295). This is demonstrated below.

Hexadecimal                  Decimal

(high)     (low)

(XHigh:XLow)     $0000_0001:0000_0000           4,294,967,296

+ (YHigh:YLow)   + $FFFF_FFFF:FFFF_FFFF         +            -1

───────────────────        ──────────────

= $0000_0000:FFFF_FFFF         = 4,294,967,295

A signed triple-long (96-bit) addition is similar, but with an ADDX instruction inserted between the ADD and ADDSX instructions:

add   XLow,  YLow    wcz	'Add low longs; save C (required) and Z (optional)

addx  XMid,  YMid    wcz	'Add middle longs; save C (required) and Z (optional)

addsx XHigh, YHigh   wcz	'Add high longs; save C and Z (both optional)

During this multi-step operation, the Z flag always indicates if the result is turning out to be zero, but the C flag indicates unsigned carries until the final instruction, ADDSX, in which it indicates the final result's sign.

### Subtracting Two Multi-Long Values

To subtract two 64-bit values, use an unsigned SUB instruction (regardless of the values' unsigned or signed nature) followed by either an unsigned-extended SUBX or signed-extended SUBSX instruction, for unsigned or signed numbers, respectively.  If the two values are larger than 64 bits in size (regardless of their signed/unsigned format) insert one SUBX instruction per extra 32-bits in-between the first and last instruction.

Make sure to also use the WC or WCZ effect on the leading SUB and SUBX instructions so that the final SUBX or SUBSX instruction works properly.

#### 64-bit Unsigned Subtraction

An unsigned double-long (64-bit) subtraction looks like this:

sub   XLow,  YLow    wcz	'Subtract low longs; save C (required) and Z (optional)

subx  XHigh, YHigh   wcz	'Subtract high longs; save C and Z (both optional)

Note: the optional flag effects are only required if the intent is to monitor for zero or an unsigned underflow.

After executing the above, the full double-long (64-bit) result is in the registers XHigh:XLow. If XHigh:XLow started out as $0000_0001:0000_0000 (4,294,967,296) and YHigh:YLow was $0000_0000:0000_0001 (1), the result in XHigh:XLow would be $0000_0000:FFFF_FFFF (4,294,967,295). This is demonstrated below.

Hexadecimal                  Decimal

(high)     (low)

(XHigh:XLow)     $0000_0001:0000_0000           4,294,967,296

- (YHigh:YLow)   - $0000_0000:0000_0001         -             1

───────────────────        ──────────────

= $0000_0000:FFFF_FFFF         = 4,294,967,295

An unsigned triple-long (96-bit) subtraction is similar, but with another SUBX instruction inserted between the SUB and SUBX instructions:

sub   XLow,  YLow    wcz	'Subtract low longs; save C (required) and Z (optional)

subx  XMid,  YMid    wcz	'Subtract middle longs; save C (required) and Z (optional)

subx  XHigh, YHigh   wcz	'Subtract high longs; save C and Z (both optional)

During this multi-step operation, the Z flag always indicates if the result is turning out to be zero, but the C flag indicates unsigned borrows until the final SUBX instruction, in which it indicates unsigned underflow (if any).

#### 64-bit Signed Subtraction

In comparison to the above, a signed double-long (64-bit) subtraction uses a SUBSX instead of SUBX as the last instruction:

sub   XLow,  YLow    wcz	'Subtract low longs; save C (required) and Z (optional)

subsx XHigh, YHigh   wcz	'Subtract high longs; save C and Z (both optional)

Note: the optional flag effects are only required if the intent is to monitor for zero or the final sign.

After executing the above, the full double-long (64-bit) result is in the registers XHigh:XLow.  If XHigh:XLow started out as $0000_0000:0000_0001 (1) and YHigh:YLow was $0000_0000:0000_0002 (2), the result in XHigh:XLow would be $FFFF_FFFF:FFFF_FFFF (-1). This is demonstrated below.

Hexadecimal                  Decimal

(high)     (low)

(XHigh:XLow)     $0000_0000:0000_0001                       1

- (YHigh:YLow)   - $0000_0000:0000_0002         -             2

───────────────────        ──────────────

= $FFFF_FFFF:FFFF_FFFF         =            -1

A signed triple-long (96-bit) subtraction would look similar but with a SUBX instruction inserted between the SUB and SUBSX instructions:

sub   XLow,  YLow    wcz	'Subtract low longs; save C (required) and Z (optional)

subx  XMid,  YMid    wcz	'Subtract middle longs; save C (required) and Z (optional)

subsx XHigh, YHigh   wcz	'Subtract high longs; save C and Z (both optional)

During this multi-step operation, the Z flag always indicates if the result is turning out to be zero, but the C flag indicates unsigned borrows until the final instruction, SUBSX, in which it indicates the final result's sign.

### Comparing Two Multi-Long Values

The process of comparing multi-long values is similar to subtracting them.  To compare two 64-bit values, use an unsigned CMP instruction (regardless of their unsigned or signed form) followed by either an unsigned-extended CMPX or signed-extended CMPSX instruction, for unsigned or signed numbers, respectively.  If the two values are larger than 64 bits in size (regardless of their signed/unsigned format) insert one CMPX instruction per extra 32-bits in-between the first and last instruction.

Make sure to also use the WC or WCZ effect on the leading CMP and CMPX instructions so the final CMPX or CMPSX instruction works properly.

#### 64-bit Unsigned Compare

An unsigned double-long (64-bit) compare looks like this:

cmp   XLow,  YLow    wcz	'Compare low longs; save C and Z (required)

cmpx  XHigh, YHigh   wcz	'Compare high longs; save C and Z (required)

Note: both C and Z flags are required to discern the possible outcomes of the comparison; X < Y, X = Y, or X > Y.

After executing the above, the C and Z flags will indicate the relationship between the two unsigned double-long (64-bit) values.  If XHigh:XLow started as $0000_0001:0000_0000 (4,294,967,296) and YHigh:YLow was $0000_0000:0000_0001 (1) the resulting flags would be: Z = 0 and C = 0; (X > Y). This is demonstrated below. Note that the comparison is really just a subtraction with the result not written and the Z and C flags updated.

Hexadecimal                  Decimal           Flags

(high)     (low)

(XHigh:XLow)     $0000_0001:0000_0000           4,294,967,296         n/a

- (YHigh:YLow)   - $0000_0000:0000_0001         -             1         n/a

───────────────────        ──────────────     ───────

= $0000_0000:FFFF_FFFF         = 4,294,967,295      Z=0, C=0

#### 64-bit Signed Compare

In comparison to the above, a signed double-long (64-bit) comparison uses a CMPSX instead of CMPX as the last instruction:

cmp   XLow,  YLow    wcz	'Compare low longs; save C and Z (required)

cmpsx XHigh, YHigh   wcz	'Compare high longs; save C and Z (required)

Note: both C and Z flags are required to discern the possible outcomes of the comparison; X < Y, X = Y, or X > Y.

After executing the above, the C and Z flags indicate the relationship between the two signed double-long (64-bit) values.  If XHigh:XLow started as $FFFF_FFFF:FFFF_FFFF (-1) and YHigh:YLow was $0000_0000:0000_0001 (1) the resulting flags would be: Z = 0 and C = 1; (Value1 < Value2). This is demonstrated below.  Note that the comparison is really just a subtraction with the result not written and the Z and C flags updated.

Hexadecimal                  Decimal           Flags

(high)     (low)

(XHigh:XLow)     $FFFF_FFFF:FFFF_FFFF                      -1         n/a

- (YHigh:YLow)   - $0000_0000:0000_0001         -             1         n/a

───────────────────        ──────────────     ───────

= $FFFF_FFFF:FFFF_FFFE         =            -2      Z=0, C=1

A signed triple-long (96-bit) comparison would look similar but with a CMPX  instruction inserted between the CMP and CMPSX instructions:

cmp   XLow,  YLow    wcz	'Compare low longs; save C and Z (required)

cmpx  XMid,  YMid    wcz	'Compare middle longs; save C and Z (required)

cmpsx XHigh, YHigh   wcz	'Compare high longs; save C and Z (required)

## Categorical Listing Of Propeller 2 Assembly Language

The following is a list of all elements of the PASM2 language grouped by category and ordered by type or frequency-of-use within each category.

### Directives

PASM2 directives instruct the compiler how to configure assembly code.

ALIGNW	Align to next word in Hub

ALIGNL	Align to next long in Hub

BYTE	Insert byte data

WORD	Insert word data

LONG	Insert long data

ORG	Set code for Cog RAM

ORGH	Set code for Hub RAM

ORGF	Fill Cog RAM with zeros

FIT	Validate that code fits within Cog RAM or Hub RAM

RES	Reserve long registers for symbol

### Conditions

Conditions are placed in front of instructions ( {Label}  {Condition}  Instruction  Operands  {Effect} ) to conditionally execute or exclude that instruction based on flag settings at runtime.  Conditions are optional; omitting the condition means "always execute" the instruction (the default behavior).

IF_E	If comparison/subtraction was equal (Z = 1)

IF_NE	If comparison/subtraction was not equal (Z = 0)

IF_A	If comparison/subtraction was above (C = 0 and Z = 0)

IF_AE	If comparison/subtraction was above or equal (C = 0)

IF_B	If comparison/subtraction was below (C = 1)

IF_BE	If comparison/subtraction was below or equal (C = 1 or Z = 1)

IF_GT	If comparison/subtraction was greater than (C = 0 and Z = 0)

IF_GE	If comparison/subtraction was greater than or equal (C = 0)

IF_LT	If comparison/subtraction was less than (C = 1)

IF_LE	If comparison/subtraction was less than or equal (C = 1 or Z = 1)

IF_C	If C set (C = 1)

IF_NC	If C clear (C = 0)

IF_Z	If Z set (Z = 1)

IF_NZ	If Z clear (Z = 0)

IF_C_EQ_Z	If C equal to Z (C = 0 and Z = 0 --or-- C = 1 and Z = 1)

IF_C_NE_Z	If C not equal to Z (C = 0 and Z = 1 --or-- C = 1 and Z = 0)

IF_C_AND_Z	If C set and Z set (C = 1 and Z = 1)

IF_C_AND_NZ	If C set and Z clear (C = 1 and Z = 0)

IF_NC_AND_Z	If C clear and Z set (C = 0 and Z = 1)

IF_NC_AND_NZ	If C clear and Z clear (C = 0 and Z = 0)

IF_C_OR_Z	If C set or Z set (C = 1 or Z = 1)

IF_C_OR_NZ	If C set or Z clear (C = 1 or Z = 0)

IF_NC_OR_Z	If C clear or Z set (C = 0 or Z = 1)

IF_NC_OR_NZ	If C clear or Z clear (C = 1 or Z = 0)

IF_Z_EQ_C	If Z equal to C (Z = 0 and C = 0 --or-- Z = 1 and C = 1)

IF_Z_NE_C	If Z not equal to C (Z = 0 and C = 1 --or-- Z = 1 and C = 0)

IF_Z_AND_C	If Z set and C set (Z = 1 and C = 1)

IF_Z_AND_NC	If Z set and C clear (Z = 1 and C = 0)

IF_NZ_AND_C	If Z clear and C set (Z = 0 and C = 1)

IF_NZ_AND_NC	If Z clear and C clear (Z = 0 and C = 0)

IF_Z_OR_C	If Z set or C set (Z = 1 or C = 1)

IF_Z_OR_NC	If Z set or C clear (Z = 1 or C = 0)

IF_NZ_OR_C	If Z clear or C set (Z = 0 or C = 1)

IF_NZ_OR_NC	If Z clear or C clear (Z = 0 or C = 0)

IF_00	If C clear and Z clear (C = 0 and Z = 0)

IF_01	If C clear and Z set (C = 0 and Z = 1)

IF_10	If C set and Z clear (C = 1 and Z = 0)

IF_11	If C set and Z set (C = 1 and Z = 1)

IF_X0	If Z clear (Z = 0)

IF_X1	If Z set (Z = 1)

IF_0X	If C clear (C = 0)

IF_1X	If C set (C = 1)

IF_NOT_00	If C clear and Z clear (C = 0 and Z = 0)

IF_NOT_01	If C set or Z clear (C = 1 or Z = 0)

IF_NOT_10	If C clear or Z set (C = 0 or Z = 1)

IF_NOT_11	If C clear or Z clear (C = 0 or Z = 0)

IF_DIFF	If C not equal to Z (C = 0 and Z = 1 --or-- C = 1 and Z = 0)

IF_SAME	If C equal to Z (C = 0 and Z = 0 --or-- C = 1 and Z = 1)

_RET_	Always execute instruction then return if no branch; no context restore

### Effects

Nearly half of PASM2 instructions feature optional effects to modify the C and/or Z flags.  Effects are placed at the end of such instructions ( {Label}  {Condition}  Instruction  Operands  {Effect} ) to affect flags, or omitted to leave flags as-is.

ANDC	AND tested bit/pin into current C

ANDZ	AND tested bit/pin into current Z

ORC	OR tested bit/pin into current C

ORZ	OR tested bit/pin into current Z

XORC	XOR tested bit/pin into current C

XORZ	XOR tested bit/pin into current Z

WC	Write C flag

WCZ	Write both C and Z flags

WZ	Write Z flag

### Flag Modification

Flag Modification commands (MODxx) alter the state of the C and/or Z flag according to the given modifier

( {Label}  {Condition}  MOD_Instruction  Modifier  {Effect} ).

MOD_Instruction

MODC	Modify C according to modifier

MODZ	Modify Z according to modifier

MODCZ	Modify C and Z according to modifier

Modifier

_CLR	Clear C/Z (C == 0 and/or Z == 0)

_E	Set C/Z if comparison/subtraction was equal (C == Z and/or Z == Z)

_NE	Set C/Z if comparison/subtraction was not equal (C == !Z and/or Z == !Z)

_GT	Set C/Z if comparison/subtraction was greater than (C == !C AND !Z and/or Z == !C AND !Z)

_GE	Set C/Z if comparison/subtraction was greater than or equal (C == !C and/or Z == !C)

_LT	Set C/Z if comparison/subtraction was less than (C == C and/or Z == C)

_LE	Set C/Z if comparison/subtraction was less than or equal (C == C OR Z and/or Z == C OR Z)

_C	Set C/Z to C (C == C and/or Z == C)

_NC	Set C/Z to inverse of C (C == !C and/or Z == !C)

_Z	Set C/Z to Z (C == Z and/or Z == Z)

_NZ	Set C/Z to inverse of Z (C == !Z and/or Z == !Z)

_C_EQ_Z	Set C/Z if C equal to Z (C == C = Z and/or Z == C = Z)

_C_NE_Z	Set C/Z if C not equal to Z (C == C <> Z and/or Z == C <> Z)

_C_AND_Z	Set C/Z to C AND Z (C == C AND Z and/or Z == C AND Z)

_C_AND_NZ	Set C/Z to C AND NOT Z (C == C AND !Z and/or Z == C AND !Z)

_NC_AND_Z	Set C/Z to NOT C AND Z (C == !C AND Z and/or Z == !C AND Z)

_NC_AND_NZ	Set C/Z to NOT C AND NOT Z (C == !C AND !Z and/or Z == !C AND !Z)

_C_OR_Z	Set C/Z to C OR Z (C == C OR Z and/or Z == C OR Z)

_C_OR_NZ	Set C/Z to C OR NOT Z (C == C OR !Z and/or Z == C OR !Z)

_NC_OR_Z	Set C/Z to NOT C OR Z (C == !C OR Z and/or Z == !C OR Z)

_NC_OR_NZ	Set C/Z to NOT C OR NOT Z (C == !C OR !Z and/or Z == !C OR !Z)

_Z_EQ_C	Set C/Z if Z equal to C (C == Z = C and/or Z == Z = C)

_Z_NE_C	Set C/Z if Z not equal to C (C == Z <> C and/or Z == Z <> C)

_Z_AND_C	Set C/Z to Z AND C (C == Z AND C and/or Z == Z AND C)

_Z_AND_NC	Set C/Z to Z AND NOT C (C == Z AND !C and/or Z == Z AND !C)

_NZ_AND_C	Set C/Z to NOT Z AND C (C == !Z AND C and/or Z == !Z AND C)

_NZ_AND_NC	Set C/Z to NOT Z AND NOT C (C == !Z AND !C and/or Z == !Z AND !C)

_Z_OR_C	Set C/Z to Z OR C (C == Z OR C and/or Z == Z OR C)

_Z_OR_NC	Set C/Z to Z OR NOT C (C == Z OR !C and/or Z == Z OR !C)

_NZ_OR_C	Set C/Z to NOT Z OR C (C == !Z OR C and/or Z == !Z OR C)

_NZ_OR_NC	Set C/Z to NOT Z OR NOT C (C == !Z OR !C and/or Z == !Z OR !C)

_SET	Set C/Z (C == 1 and/or Z == 1)

### Augmentation

Augmentation instructions change or enhance the next instruction's behavior.

SETQ	Set Q register for companion instruction

SETQ2	Set Q register for LUT RAM companion instruction

AUGS	Augment next literal Src to 32-bits

AUGD	Augment next literal Dest to 32-bits

### Indirection

Indirection instructions modify the next instruction's target addresses.

ALTB	Alter subsequent BITxxx instruction

ALTSN	Alter subsequent SETNIB instruction

ALTGN	Alter subsequent GETNIB / ROLNIB instruction

ALTSB	Alter subsequent SETBYTE instruction

ALTGB	Alter subsequent GETBYTE / ROLBYTE instruction

ALTSW	Alter subsequent SETWORD instruction

ALTGW	Alter subsequent GETWORD / ROLWORD instruction

ALTD	Alter D field of next instruction

ALTS	Alter S field of next instruction

ALTR	Alter Result register address of next instruction

SETD	Set template D field for ALTI

SETS	Set template S field for ALTI

SETR	Set template Result field for ALTI

ALTI	Substitute next instruction's field values from template, per configuration

### Configuration

The HUBSET configuration instruction sets clock, write-protect, debug, filter, and PRNG configuration or initiates a software reset of the Propeller.

HUBSET	Set global configuration, or software reset

### Cog Control

Cog control instructions monitor or alter current cog activity.

COGID	Get current cog’s ID or any cog's status by ID

COGINIT	Start an available cog, or restart a cog by ID

COGSTOP	Stop a cog by ID

### Process Control

Process control instructions coordinate exclusive actions between multiple cogs.  See also the Event Handling instructions like xxxATN and xxxSE1.

LOCKNEW	Request a new lock

LOCKRET	Return a lock

LOCKTRY	Attempt ownership of a lock

LOCKREL	Release ownership of a lock

### Flow Control

Flow control instructions direct execution to another part of the application.  They are used for looping, subroutine, interrupt, or instruction skipping purposes.  Also see the Event Handling > Branch category for event-based flow control.

CALL	Call a subroutine; store return context on the stack

CALLA	Call a subroutine; store return context in the Hub long at PTRA++

CALLB	Call a subroutine; store return context in the Hub long at PTRB++

CALLD	Call a subroutine; store return context in PA/PB/PTRA/PTRB/D

CALLPA	Call a subroutine; store return context on the stack and copy D into PA

CALLPB	Call a subroutine; store return context on the stack and copy D into PB

RET	Return from subroutine by popping stack; optional context restore

RETA	Return from subroutine by reading Hub long at --PTRA; restore context

RETB	Return from subroutine by reading Hub long at --PTRB; restore context

JMP	Jump

JMPREL	Jump relative (forward/back)

DJZ	Decrement value and jump if zero

DJNZ	Decrement value and jump if not zero

DJF	Decrement value and jump if full (-1; $FFFF_FFFF)

DJNF	Decrement value and jump if not full (<> -1; <> $FFFF_FFFF)

IJZ	Increment value and jump if zero

IJNZ	Increment value and jump if not zero

TJZ	Test value and jump if zero

TJNZ	Test value and jump if not zero

TJF	Test value and jump if full (-1; $FFFF_FFFF)

TJNF	Test value and jump if not full (<> -1; <> $FFFF_FFFF)

TJS	Test value and jump if signed

TJNS	Test value and jump if not signed

TJV	Test value and jump if overflowed

REP	Repeat next instructions

SKIP	Skip next instructions by pattern

SKIPF	Skip next instructions fast by pattern

EXECF	Execute fast; jump to Cog RAM and set SKIPF pattern

RETI0	Return from interrupt 0

RETI1	Return from interrupt 1

RETI2	Return from interrupt 2

RETI3	Return from interrupt 3

RESI0	Resume from interrupt 0

RESI1	Resume from interrupt 1

RESI2	Resume from interrupt 2

RESI3	Resume from interrupt 3

### Lookup Table (LUT) Memory Access

Instructions that manipulate or share the cog's Lookup RAM.

RDLUT	Read long from Lookup RAM

WRLUT	Write long to Lookup RAM

SETLUTS	Enable/disable Lookup RAM sharing

### Hub Memory Access

Instructions for manipulating Hub RAM.

Random Access

RDBYTE	Read byte of Hub RAM

RDWORD	Read word of Hub RAM

RDLONG	Read long of Hub RAM

WRBYTE	Write byte to Hub RAM

WRWORD	Write word to Hub RAM

WRLONG	Write long to Hub RAM

WMLONG	Write non-zero bytes of long to Hub RAM

PUSHA	Write long to Hub RAM at PTRA++

PUSHB	Write long to Hub RAM at PTRB++

POPA	Read long of Hub RAM at --PRTA

POPB	Read long of Hub RAM at --PRTB

FIFO Access

GETPTR	Get current FIFO Hub pointer

RDFAST	Start new fast FIFO Hub read

WRFAST	Start new fast FIFO Hub write

FBLOCK	Set next block for FIFO

RFBYTE	Read next byte from FIFO

RFWORD	Read next word from FIFO

RFLONG	Read next long from FIFO

WFBYTE	Write next byte to FIFO

WFWORD	Write next word to FIFO

WFLONG	Write next long to FIFO

RFVAR	Read variable-size (1 to 4 byte) value from FIFO

RFVARS	Read variable-size (1 to 4 byte) sign-extended value from FIFO

### Streamer

Instructions to manipulate the cog's streamer.

SETXFRQ	Set streamer NCO frequency

XINIT	Issue streamer command immediately and zero the phase

XSTOP	Stop streamer immediately

XZERO	Buffer new streamer command; issued on final NCO rollover and zero the phase

XCONT	Buffer new streamer command; issued on final NCO rollover and continue the phase

GETXACC	Get the streamer's Goertzel accumulator results and clear the accumulators

### I/O Pins

Instructions for manipulating Core and Smart I/O pin features.  In addition to these instructions, the DIRA / DIRB / OUTA / OUTB / INA / INB registers may be used as operands of instructions to affect or read I/O pins.

Core Logic

DIRL	Set pin(s) direction to input (0)

DIRH	Set pin(s) direction to output (1)

DIRC	Set pin(s) direction to input/output according to C

DIRNC	Set pin(s) direction to input/output according to !C

DIRZ	Set pin(s) direction to input/output according to Z

DIRNZ	Set pin(s) direction to input/output according to !Z

DIRRND	Set pin(s) direction to random input/output

DIRNOT	Toggle pin(s) to the opposite direction

OUTL	Set pin(s) output level low (0)

OUTH	Set pin(s) output level high (1)

OUTC	Set pin(s) output level to low/high according to C

OUTNC	Set pin(s) output level to low/high according to !C

OUTZ	Set pin(s) output level to low/high according to Z

OUTNZ	Set pin(s) output level to low/high according to !Z

OUTRND	Set pin(s) output level to random low/high

OUTNOT	Toggle pin(s) to the opposite output level

DRVL	Set pin(s) direction to output and output level low (0)

DRVH	Set pin(s) direction to output and output level high (1)

DRVC	Set pin(s) direction to output and output level to low/high according to C

DRVNC	Set pin(s) direction to output and output level to low/high according to !C

DRVZ	Set pin(s) direction to output and output level to low/high according to Z

DRVNZ	Set pin(s) direction to output and output level to low/high according to !Z

DRVRND	Set pin(s) direction to output and output level to random low/high

DRVNOT	Set pin(s) direction to output and toggle to the opposite output level

FLTL	Set pin(s) direction to input and to an output level of low (0)

FLTH	Set pin(s) direction to input and to an output level of high (1)

FLTC	Set pin(s) direction to input and to an output level of low/high according to C

FLTNC	Set pin(s) direction to input and to an output level of low/high according to !C

FLTZ	Set pin(s) direction to input and to an output level of low/high according to Z

FLTNZ	Set pin(s) direction to input and to an output level of low/high according to !Z

FLTRND	Set pin(s) direction to input and to an output level of random low/high

FLTNOT	Set pin(s) direction to input and toggle to the opposite output level

TESTP	Test pin and either store, AND, OR, or XOR the result into C/Z

TESTPN	Test pin and either store, AND, OR, or XOR inverse result into C/Z

Smart Logic

AKPIN	Acknowledge smart pin(s)

RQPIN	Read smart pin result and mode flag; no acknowledge (quiet)

RDPIN	Read smart pin result and mode flag, then acknowledge

WRPIN	Set smart pin(s) mode and acknowledge

WXPIN	Set smart pin(s) "X" value and acknowledge

WYPIN	Set smart pin(s) "Y" value and acknowledge

SETDACS	Set smart pin digital to analog converters (DACs)

SETSCP	Enable/disable four-channel oscilloscope pins

GETSCP	Get four-channel oscilloscope samples

### Math

ALU circuit and CORDIC Solver math instructions.  The ALU (Arithmetic Logic Unit) instructions perform common math operations in just 2 clock cycles each.  The CORDIC (COordinate Rotation DIgital Computer) instructions perform more complicated math operations in 54 clock cycles each.

ALU Circuit

ABS	Get the absolute value of a number

NEG	Negate a value

NEGC	Negate value according to C

NEGNC	Negate value according to !C

NEGZ	Negate value according to Z

NEGNZ	Negate value according to !Z

ZEROX	Zero-extend value beyond designated bit

SIGNX	Sign-extend value beyond designated bit

FGE	Force unsigned value to be greater than or equal to another

FGES	Force signed value to be greater than or equal to another

FLE	Force unsigned value to be less than or equal to another

FLES	Force signed value to be less than or equal to another

SUMC	Adjust signed value by other C-negated value

SUMNC	Adjust signed value by other !C-negated value

SUMZ	Adjust signed value by other Z-negated value

SUMNZ	Adjust signed value by other !Z-negated value

ADD	Add two unsigned values

ADDX	Add two unsigned extended values

ADDS	Add two signed values

ADDSX	Add two signed extended values

INCMOD	Increment with modulus

DECMOD	Decrement with modulus

SUB	Subtract one unsigned value from another

SUBX	Subtract one unsigned extended value from another

SUBS	Subtract one signed value from another

SUBSX	Subtract one signed extended value from another

SUBR	Subtract one unsigned value from another (in reverse order to SUB)

CMPSUB	Compare two unsigned values and subtract the second if it is lesser or equal

MUL	Multiply unsigned 16-bit x 16-bit values

MULS	Multiply signed 16-bit x 16-bit values

SHL	Multiply 32-bit integer by power-of-two (shift left)

SHR	Divide unsigned 32-bit integer by power-of-two (shift right)

SAR	Divide signed 32-bit integer by power-of-two (shift arithmetic right)

SCA	Create unsigned 16-bit scale value for next instruction's S value

SCAS	Create signed 18-bit scale value for next instruction's S value

CMP	Compare two unsigned values

CMPX	Compare two unsigned values plus carry flag

CMPS	Compare two signed values

CMPSX	Compare two signed values plus carry flag

CMPR	Compare two unsigned values (in reverse order to CMP)

CMPM	Compare two unsigned values, get MSB of difference

CORDIC Solver

QLOG	Start CORDIC number-to-logarithm conversion

QEXP	Start CORDIC logarithm-to-number conversion

QMUL	Start CORDIC unsigned multiplication

QDIV	Start CORDIC unsigned division

QFRAC	Start CORDIC unsigned division

QSQRT	Start CORDIC square root

QROTATE	Start CORDIC rotation or polar-to-cartesian conversion

QVECTOR	Start CORDIC cartesian-to-polar conversion

GETQX	Get lower long, quotient, root, X, length, logarithm, or integer CORDIC result

GETQY	Get upper long, remainder, Y, or angle CORDIC result

### Timing

Instructions to retrieve the current clock value or wait for a future clock value.  Also see xxxCT event instructions for handling timed events.

GETCT	Get lower/upper 32-bits of System Counter

WAITX	Wait for fixed or range-limited random number of clocks

### Event Handling

Event instructions create, configure, monitor, or branch on various events.

Create/Configure

COGATN	Strobe the attention signal of one or more cogs

ADDCT1	Set counter event 1 trigger time

ADDCT2	Set counter event 2 trigger time

ADDCT3	Set counter event 3 trigger time

SETPAT	Set pin pattern event

SETSE1	Set selectable event 1 configuration

SETSE2	Set selectable event 2 configuration

SETSE3	Set selectable event 3 configuration

SETSE4	Set selectable event 4 configuration

Monitor

POLLATN	Retrieve and clear attention flag

POLLCT1	Retrieve and clear counter event 1 flag

POLLCT2	Retrieve and clear counter event 2 flag

POLLCT3	Retrieve and clear counter event 3 flag

POLLFBW	Retrieve and clear FIFO-interface-block-wrap event flag

POLLINT	Retrieve and clear interrupt-occurred event flag

POLLPAT	Retrieve and clear pin-pattern-detected event flag

POLLQMT	Retrieve and clear CORDIC-read-but-empty event flag

POLLSE1	Retrieve and clear selectable event 1 flag

POLLSE2	Retrieve and clear selectable event 2 flag

POLLSE3	Retrieve and clear selectable event 3 flag

POLLSE4	Retrieve and clear selectable event 4 flag

POLLXMT	Retrieve and clear streamer-empty event flag

POLLXFI	Retrieve and clear streamer-finished event flag

POLLXRO	Retrieve and clear streamer-NCO-rollover event flag

POLLXRL	Retrieve and clear streamer-LUT-RAM-rollover event flag

WAITATN	Wait for and clear attention flag

WAITCT1	Wait for and clear counter event 1 flag

WAITCT2	Wait for and clear counter event 2 flag

WAITCT3	Wait for and clear counter event 3 flag

WAITFBW	Wait for and clear FIFO-interface-block-wrap event flag

WAITINT	Wait for and clear interrupt-occurred event flag

WAITPAT	Wait for and clear pin-pattern-detected event flag

WAITSE1	Wait for and clear selectable event 1 flag

WAITSE2	Wait for and clear selectable event 2 flag

WAITSE3	Wait for and clear selectable event 3 flag

WAITSE4	Wait for and clear selectable event 4 flag

WAITXMT	Wait for and clear streamer-empty event flag

WAITXFI	Wait for and clear streamer-finished event flag

WAITXRO	Wait for and clear streamer-NCO-rollover event flag

WAITXRL	Wait for and clear streamer-LUT-RAM-rollover event flag

Branch

JATN	Jump if attention flag set

JNATN	Jump if attention flag clear

JCT1	Jump if counter 1 event flag set

JNCT1	Jump if counter 1 event flag clear

JCT2	Jump if counter 2 event flag set

JNCT2	Jump if counter 2 event flag clear

JCT3	Jump if counter 3 event flag set

JNCT3	Jump if counter 3 event flag clear

JFBW	Jump if FIFO interface block wrap event flag set

JNFBW	Jump if FIFO interface block wrap event flag clear

JINT	Jump if interrupt-occurred event flag set

JNINT	Jump if interrupt-occurred event flag clear

JPAT	Jump if pin pattern event flag set

JNPAT	Jump if pin pattern event flag clear

JSE1	Jump if selectable event 1 flag set

JNSE1	Jump if selectable event 1 flag clear

JSE2	Jump if selectable event 2 flag set

JNSE2	Jump if selectable event 2 flag clear

JSE3	Jump if selectable event 3 flag set

JNSE3	Jump if selectable event 3 flag clear

JSE4	Jump if selectable event 4 flag set

JNSE4	Jump if selectable event 4 flag clear

JQMT	Jump if CORDIC-read-but-empty event flag set

JNQMT	Jump if CORDIC-read-but-empty event flag clear

JXMT	Jump if streamer empty event flag set

JNXMT	Jump if streamer empty event flag clear

JXFI	Jump if streamer finished event flag set

JNXFI	Jump if streamer finished event flag clear

JXRO	Jump if streamer NCO rollover event flag set

JNXRO	Jump if streamer NCO rollover event flag clear

JXRL	Jump if streamer LUT RAM rollover event flag set

JNXRL	Jump if streamer LUT RAM rollover event flag clear

### Interrupts

Instructions to manage interrupts.

ALLOWI	Allow interrupts (default)

STALLI	Stall Interrupts

TRGINT1	Trigger interrupt 1; regardless of STALLI mode

TRGINT2	Trigger interrupt 2; regardless of STALLI mode

TRGINT3	Trigger interrupt 3; regardless of STALLI mode

NIXINT1	Cancel interrupt 1

NIXINT2	Cancel interrupt 2

NIXINT3	Cancel interrupt 3

SETINT1	Set interrupt 1 source

SETINT2	Set interrupt 2 source

SETINT3	Set interrupt 3 source

BRK	Trigger breakpoint in current cog

COGBRK	Trigger breakpoint in specified cog

GETBRK	Get breakpoint and cog status

### Bit Operations

Bit manipulation instructions.

AND	Bitwise AND two values

ANDN	Bitwise AND a value with the NOT of another

OR	Bitwise OR two values

XOR	Bitwise XOR two values

NOT	Bitwise NOT a value

REV	Reverse bits of value

SHR	Shift bits right

SHL	Shift bits left

SAR	Shift bits right, extending the MSB

SAL	Shift bits left, extending the LSB

ROR	Rotate bits right

ROL	Rotate bits left

RCR	Rotate carry flag right into value

RCL	Rotate carry flag left into value

RCZR	Rotate carry and zero flags right (2-bit rotate right)

RCZL	Rotate carry and zero flags left (2-bit rotate left)

BITL	Set bit(s) low (0)

BITH	Set bit(s) high (1)

BITC	Set bit(s) low/high according to C

BITNC	Set bit(s) low/high according to !C

BITZ	Set bit(s) low/high according to Z

BITNZ	Set bit(s) low/high according to !Z

BITRND	Set bit(s) random low/high

BITNOT	Toggle bit(s) to opposite state

MUXC	Set discrete bits to C

MUXNC	Set discrete bits to !C

MUXZ	Set discrete bits to Z

MUXNZ	Set discrete bits to !Z

MUXNITS	Set discrete bit pairs of a value to non-zero bit pair states of another

MUXNIBS	Set discrete nibbles of a value to non-zero nibble states of another

MUXQ	Set discrete bits of a value to that of another

WRC	Write C to register

WRNC	Write !C to register

WRZ	Write Z to register

WRNZ	Write !Z to register

TESTB	Test bit of D and either store, AND, OR, or XOR the result into flags

TESTBN	Test bit of !D and either store, AND, OR, or XOR the result into flags

TEST	Test D, or bitwise AND D with S, to affect flags only

TESTN	Test D by bitwise ANDing with !S to affect flags only

GETNIB	Get a nibble from a value

GETBYTE	Get a byte from a value

GETWORD	Get a word from a value

SETNIB	Set a nibble to new value

SETBYTE	Set a byte to new value

SETWORD	Set a word to new value

ROLNIB	Rotate a nibble left into a value

ROLBYTE	Rotate a byte left into a value

ROLWORD	Rotate a word left into a value

MOV	Move a value to a register

MOVBYTS	Move bytes within D, described by S

ENCOD	Get bit position of top-most 1 of Src or Dest into Dest

DECOD	Decode value (0—31) into single-high-bit long

BMASK	Get 1..32-bit mask into Dest

ONES	Get number of 1s from Dest or Src into Dest

RGBSQZ	Squeeze 8:8:8 RGB value into word

RGBEXP	Squeeze word RGB value into 8:8:8

SPLITB	Split every 4th bit of D into bytes

MERGEB	Merge bits of bytes in D

SPLITW	Split odd/even bits of D into words

MERGEW	Merge bits of words in D

SEUSSF	Forward relocate and periodically invert bits within D

SEUSSR	Reverse relocate and periodically invert bits within D

### Color Manipulation

Color value mixing and control instructions.

Pixel Mixer

ADDPIX	Add RGB colors with full saturation

MULPIX	Multiply colors ($00—$FF = 0.0—1.0)

BLNPIX	Alpha-blend colors using SETPIV value

MIXPIX	Mix colors using SETPIX and SETPIV values

SETPIV	Set blend factor for BLNPIX / MIXPIX

SETPIX	Set mode for MIXPIX

Color Space Converter

SETCY	Set colorspace luma (Y) value

SETCI	Set colorspace chrominance in-phase (I) value

SETCQ	Set colorspace chrominance quadrature (Q) value

SETCFRQ	Set colorspace frequency

SETCMOD	Set colorspace mode

### Miscellaneous

Miscellaneous instructions.

GETRND	Get Xoroshiro128** random value

XORO32	Create Xoroshiro32+ random value from D and store in next instruction's S value

CRCBIT	Iterate CRC value with 1-bit injection against 32-bit polynomial

CRCNIB	Iterate CRC value with 4-bit injection against 32-bit polynomial

PUSH	Place value onto hardware stack

POP	Remove value from hardware stack

LOC	Create relative or absolutable address for relocatable code

NOP	No operation, just elapse two cycles

### Registers

Special register symbols for direct feature access.

DIRA	Output enables for P31..P0

DIRB	Output enables for P63..P32

OUTA	Output states for P31..P0

OUTB	Output states for P63..P32

INA	Input states for P31..P0

INB	Input states for P63..P32

PA	CALLD-imm return, CALLPA parameter, or LOC address

PB	CALLD-imm return, CALLPB parameter, or LOC address

PTRA	Pointer A to Hub RAM

PTRB	Pointer B to Hub RAM

IJMP1	Interrupt call address for INT1

IJMP2	Interrupt call address for INT2

IJMP3	Interrupt call address for INT3

IRET1	Interrupt return address for INT1

IRET2	Interrupt return address for INT2

IRET3	Interrupt return address for INT3

PR0..PR7	PASM2 to Spin2 shared registers

### Constants

Constants that represent special numeric values and modes.

Numerics

TRUE	Logical true: -1 ($FFFFFFFF)

FALSE	Logical false: 0 ($00000000)

POSX	Maximum positive integer: 2,147,483,647 ($7FFFFFFF)

NEGX	Maximum negative integer: -2,147,483,648 ($80000000)

PI	Floating-point value for PI: ~3.141593 ($40490FDB)

Cog Initialization

COGEXEC	Start a cog by ID in Cog Execution mode

COGEXEC_NEW	Start the next available cog in Cog Execution mode

COGEXEC_NEW_PAIR	Start the next available cog pair in Cog Execution mode with LUT RAM sharing

HUBEXEC	Start a cog by ID in Hub Execution mode

HUBEXEC_NEW	Start the next available cog in Hub Execution mode

HUBEXEC_NEW_PAIR	Start the next available cog pair in Hub Execution mode with LUT RAM sharing

Smart Pins

P_ADC..P_XOR_AB	116 smart pin mode constants for WRPIN instruction

Streamer

X_16P_2DAC8_WFWORD..

X_WRITE_ON	78 streamer mode constants for XINIT / XZERO / XCONT instructions

Event/Interrupt

EVENT_ATN..INT_OFF	17 event/interrupt mode constants for SETINTx and GETBRK instructions

### Operators

Operators for constant-expressions in PASM2 operands.

Unary

!	Bitwise: NOT

+	Positive (+X) unary form of Add

-	Negate (−X); unary form of Subtract

Binary

>>	Bitwise: Shift right

<<	Bitwise: Shift left

&	Bitwise: AND

|	Bitwise: OR

^	Bitwise: XOR

+	Add

-	Subtract

*	Multiply and return lower 32 bits (signed)

/	Divide and return quotient (signed)

+/	Divide and return quotient (unsigned)

//	Divide and return remainder (signed)

+//	Divide and return remainder (unsigned)

#>	Limit minimum (signed)

<#	Limit maximum (signed)

<	Boolean: Is less than (signed)

+<	Boolean: Is less than (unsigned)

>	Boolean: Is greater than (signed)

+>	Boolean: Is greater than (unsigned)

<=	Boolean: Is less than or equal (signed)

+<=	Boolean: Is less than or equal (unsigned)

>=	Boolean: Is greater than or equal (signed)

+>=	Boolean: Is greater than or equal (unsigned)

==	Boolean: Is equal

<>	Boolean: Is not equal

!!	Boolean: NOT

&&	Boolean: AND

||	Boolean: OR

^^	Boolean: XOR

<=>	Signed comparison (<, =, > returns -1, 0, 1)

Ternary

? :	Ternary: return 2nd or 3rd value based on 1st

## Assembly Language Elements

### Term Definitions

The Propeller 2 Assembly Language syntax, tables, and descriptions use the following terms extensively.

A / Addr: A 20-bit relative or absolute value used to change PC (the program counter).

C / Carry Flag: A 1-bit persistent "flag" value representing a special state prior to or after an instruction executes.  The C flag is traditionally output by a processor's adder circuit (ALU) to indicate that the previous mathematical operation resulted in a "carry" or a "borrow."  In addition to this use, the Propeller has many instruction-specific meanings for the C flag (both for instruction input and output).  When C appears in an instruction opcode, it indicates that it optionally writes to the C flag at the end of its execution, governed by the WC or WCZ effect.

D / Dest / Destination: The target register that an instruction ultimately affects.  Usually the Destination operand is a 9-bit register address, but may be a 32-bit augmented value (a 9-bit value extended to 32-bits by a previous AUGD instruction).  Often the value in the Destination register is read, manipulated, and overwritten by the execution of the instruction.  The final value is also referred to as Result (or R).  Some instructions use Destination for both the source value and the result's landing place.

H / Hub Long: Hub RAM long (4 bytes) used to store subroutine calling context states (C and Z flags and return address).

I: Immediate "literal" flag.  1 = Src operand value is an immediate (literal) value; 0 = Src operand value is a register address.

K / Stack: The 8-level hardware stack used for subroutine calls (stores C, Z, and PC) and temporary 32-bit push/pop storage.

L: Literal flag.  1 = Dest operand value is a literal (immediate) value; 0 = Dest operand value is a register address.

N: Index number.  0–1, 0–3, or 0–7 = An index value for instructions with a third operand.

PC: Program counter that determines the next instruction to be read by the cog's pipeline.  This is a hidden, dedicated register whose value automatically increments by 1 or 4 (Cog/LUT execution or Hub execution) unless altered by a branching instruction such as DJNZ, CALL, or JMP.

R: Relative flag.  1 = address is relative; 0 = address is absolute.

Result: Value written at the end of instruction execution; often the Destination (i.e. D or Dest) operand.

S / Src / Source: The origin value that many instructions operate with.  Often, the Source operand is a 9-bit literal value, but can also be the address of a register containing the source value, or may be a 32-bit augmented value (a 9-bit literal that is extended to 32-bits by a previous AUGS instruction or the convenient ## prefix).

W: Register to write.  00–11 = index of special register to write (PA, PB, PTRA, or PTRB).

Z / Zero Flag: A 1-bit persistent "flag" value representing a special state prior to or after an instruction executes.  The Z flag is traditionally output by a processor's adder circuit (ALU) to indicate that the previous mathematical operation resulted in a result of "zero."  In addition to this use, the Propeller has other instruction-specific uses for the Z flag (for instruction input and output).  When Z appears in an instruction opcode, it indicates that it optionally writes to the Z flag at the end of its execution, governed by the WZ or WCZ effect.

### Opcode Tables

Most syntax definitions include an opcode table similar to the one below. This table lists the instruction’s 32-bit opcode encoding, register and flag outputs, and execution time in clock cycles.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0001010 | CZI | DDDDDDDDD | SSSSSSSSS | D | sign of (D + S) | Result = 0 | 2 |

The above example is from the ADDS instruction which has one syntax form.  For instructions with multiple syntax forms, the opcode table will include a separate row for each, like this from the CALL instruction:

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101101 | RAA | AAAAAAAAA | AAAAAAAAA | PC | — | — | 4 / 13–20 |
| EEEE | 1101011 | CZ0 | DDDDDDDDD | 000101101 | PC | D[31] | D[30] | 4 / 13–20 |

The table’s first column describes the PASM2 instruction's opcode, consisting of the following fields:

- COND (bits 31:28) - Indicates the condition in which to execute the instruction.

- INSTR (bits 27:21) - Indicates the instruction or class of instruction.

- FX (bits 20:18) - Indicates the instruction’s effect status and SRC field meaning.

- DEST (bits 17:9) - Contains the 9-bit destination register address.

- SRC (bits 8:0) - Contains the 9-bit source register address or literal value.

The bits of the COND field default to all ones (1111) if no condition is specified on the instruction line, as is common; meaning "execute always."  The bit pattern will be different if a condition is specified, according to that shown for the IF_x (Conditions).

The INSTR field uniquely identifies the specific instruction, or instruction class.  Some instructions have aliases (different forms of the same class of instruction) which are further uniquely identified by the way the FX, DEST, and SRC fields are encoded.

Each bit of the FX (effects) field is high or low (1 or 0) usually to describe the ‘C’ flag, ‘Z’ flag, and ‘I’mmediate nature of the instruction; noted as the encoding "CZI" in that field.  The C and Z bits are set (1) when the instruction line specifies a WC, WZ, or WCZ effect, meaning write (update) the C and/or Z flag at the end of the instruction execution.  The "I" bit is set (1) when the instruction's Src operand is prefixed by a "#" sign, meaning the SRC field contains an immediate literal value, rather than a register address where the value is located.  Certain instructions have fixed values (0 or 1) in some or all of these bits when the normal use (C, Z, or I) has no purpose or when the DEST or SRC fields have special meaning.  A bit marked "L" is set (1) when the instruction's Dest operand is prefixed by a "#" sign, meaning the DEST field contains an immediate literal value, rather than a register address where the value is located.  A bit marked "R" is set (1) when the instruction's Address operand is not prefixed by a "\" sign, meaning the address is relative rather than absolute.

The DEST and SRC fields show the encoding of the instruction's Destination and Source operands, often "D" and "S" to indicate 9-bits from the given destination and source symbol/value on the instruction line.

The Write column indicates what register (if any) is written by the instruction.  This is most often D (the destination register indicated by the address in the D operand) but may be PC (the program counter) or I/O registers. A "—" means nothing is written.

The Z Flag and C Flag columns indicate what value (if any) is written to the flag by the instruction.  This only applies when the instruction line includes a WC, WZ, or WCZ effect, otherwise the flag is left as-is.  A "—" means the flag is never updated.

The Clocks column indicates the number of clock cycles the instruction takes to execute.  A single value or range (# --or-- #–#) is the number of clocks the instruction takes regardless of its location in memory (Reg/LUT/Hub).  Two values or ranges separated by a slash "/" (# / #–#  --or--  #–# / #–#) is the number of clock cycles if the instruction is executing from Reg or LUT RAM (left of the slash) and the number of clock cycles if executing from Hub RAM (right of the slash).  A value shown as "# or #" is the number of clock cycles if not branching (left of the "or") or if branching (right of the "or").

### Propeller Assembly Instruction Master Table

All elements (symbols) of the PASM2 language are listed below either in alphabetical order or grouped within a category that fits alphabetically.  To find or learn about elements by category, use the main categorical listing and follow its links to reach the details below.

## ABS

Absolute

Math Instruction - Get the absolute value of a number.

ABS Dest, {#}Src	{WC|WZ|WCZ}

ABS Dest	{WC|WZ|WCZ}

Result: Absolute Src (or Dest) value is stored in Dest.

- Dest is the register in which to write the absolute value of Dest or Src.

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose absolute value is written to Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0110010 | CZI | DDDDDDDDD | SSSSSSSSS | D | S[31] | Result = 0 | 2 |
| EEEE | 0110010 | CZ0 | DDDDDDDDD | DDDDDDDDD | D | D[31] | Result = 0 | 2 |

Related: NEG

Explanation:

ABS determines the absolute value of Src or Dest and writes the result into Dest.

If the WC or WCZ effect is specified, the C flag is set (1) if the original Src or Dest value was negative, or is cleared (0) if it was positive.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

Literal Src values are zero-extended, so ABS is really best used with register Src (or augmented Src) values.

## ADD

Add

Math Instruction - Add two unsigned values.

ADD Dest, {#}Src {WC|WZ|WCZ}

Result: Sum of unsigned Src and unsigned Dest is stored in Dest.

- Dest is a register containing the value to add Src to, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is added into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0001000 | CZI | DDDDDDDDD | SSSSSSSSS | D | carry of (D + S) | Result = 0 | 2 |

Related: ADDX, ADDS, ADDSX, and SUB

Explanation:

ADD sums the two unsigned values of Dest and Src together and stores the result into the Dest register.

If the WC or WCZ effect is specified, the C flag is set (1) if the summation results in a 32-bit overflow (unsigned carry), or is cleared (0) if no overflow.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result of Dest + Src is zero, or is cleared (0) if it is non-zero.

To add unsigned, multi-long values, use ADD followed by ADDX as described in Adding Two Multi-Long Values.  ADD and ADDX are also used in adding signed, multi-long values with ADDSX ending the sequence.

## ADDCT1/2/3

Add counter 1/2/3

Event Handling Instruction - Set one of three counter events' trigger time.

ADDCT1 Dest, {#}Src

ADDCT2 Dest, {#}Src

ADDCT3 Dest, {#}Src

Result: The Src value is added into Dest and the result is also stored in the hidden CT1, CT2, or CT3 event trigger register.  The event triggers when System Counter (CT) = original Dest + Src.

- Dest is a register containing the value to add Src to, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is added into Dest.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1010011 | 00I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1010011 | 01I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1010011 | 10I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |

Related: POLLCTx, WAITCTx, JCTx, and JNCTx

Explanation:

ADDCT1, ADDCT2, or ADDCT3 sets the hidden CT1, CT2, or CT3 event trigger register (respectively) to the value of Dest + Src.  The result is also written to Dest.

Use the xxxxCTx Event Handling instructions to process time-based events.

## ADDPIX

Add pixels

Color Manipulation Instruction - Add RGB colors with full saturation.

ADDPIX Dest, {#}Src

Result: Src color value bytes are added into Dest color value bytes with full saturation.

- Dest is a register containing the RGB color value to add Src to, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose RGB color value bytes are added into Dest.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1010010 | 00I | DDDDDDDDD | SSSSSSSSS | D | — | — | 7 |

Explanation:

ADDPIX sums individual RGB (reg, green, blue) color values of Src into that of Dest and stores the result in the Dest register.

## ADDS

Add signed

Math Instruction - Add two signed values.

ADDS Dest, {#}Src {WC|WZ|WCZ}

Result: Sum of signed Src and signed Dest is stored in Dest.

- Dest is a register containing the value to add Src to, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is added into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0001010 | CZI | DDDDDDDDD | SSSSSSSSS | D | sign of (D + S) | Result = 0 | 2 |

Related: ADD, ADDX, ADDSX, and SUBS

Explanation:

ADDS sums the two signed values of Dest and Src together and stores the result into the Dest register.  If Src is a 9-bit literal, its value is interpreted as positive (0-511; it is not sign-extended) — use ##Value (or insert a prior AUGS instruction) for a 32-bit signed value; negative or positive.

If the WC or WCZ effect is specified, the C flag is set (1) if the summation results in a signed overflow (signed carry), or is cleared (0) if no overflow.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result of Dest + Src is zero, or is cleared (0) if it is non-zero.

To add signed, multi-long values, use ADD (not ADDS) followed possibly by ADDX, and finally ADDSX as described in Adding Two Multi-Long Values.

## ADDSX

Add signed, extended

Math Instruction - Add two signed extended values.

ADDSX Dest, {#}Src {WC|WZ|WCZ}

Result: Sum of signed Src plus C and signed Dest is stored in Dest.

- Dest is a register containing the value to add Src pluc C to, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value plus C is added into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0001011 | CZI | DDDDDDDDD | SSSSSSSSS | D | sign of (D+S+C) | Z AND (Result = 0) | 2 |

Related: ADD, ADDX, ADDSX, and SUBSX

Explanation:

ADDSX sums the signed values of Dest and Src plus C together and stores the result into the Dest register.  The ADDSX instruction is used to perform signed multi-long (extended) addition, such as 64-bit addition.

If the WC or WCZ effect is specified, the C flag is set (1) if the result is negative (Result[31] = 1), or is cleared (0) if positive.  Use WC or WCZ on preceding ADD and ADDX instructions for proper final C flag.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Z was previously set and the result of Dest + Src + C is zero, or it is cleared (0) if non-zero.  Use WZ or WCZ on preceding ADD and ADDX instructions for proper final Z flag.

To add signed multi-long values, use ADD (not ADDS) followed possibly by ADDX, and finally ADDSX as described in Adding Two Multi-Long Values.

## ADDX

Add extended

Math Instruction - Add two unsigned extended values.

ADDX Dest, {#}Src {WC|WZ|WCZ}

Result: Sum of unsigned Src plus C and unsigned Dest is stored in Dest.

- Dest is a register containing the value to add Src pluc C to, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value plus C is added into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0001001 | CZI | DDDDDDDDD | SSSSSSSSS | D | carry of (D+S+C) | Z AND (Result = 0) | 2 |

Related: ADD, ADDSX, and SUBX

Explanation:

ADDX sums the unsigned values of Dest and Src plus C together and stores the result into the Dest register.  The ADDX instruction is used to perform unsigned multi-long (extended) addition, such as 64-bit addition.

If the WC or WCZ effect is specified, the C flag is set (1) if the summation resulted in an unsigned carry, or is cleared (0) if no carry.  Use WC or WCZ on preceding ADD and ADDX instructions for proper final C flag.  If C is set after the last ADDX in a multi-long addition, it indicates unsigned overflow.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Z was previously set and the result of Dest + Src + C is zero, or it is cleared (0) if non-zero.  Use WZ or WCZ on preceding ADD and ADDX instructions for proper final Z flag.

To add unsigned multi-long values, use ADD followed by one or more ADDX instructions as described in Adding Two Multi-Long Values.

## AKPIN

Acknowledge pin

I/O Pin (Smart Logic) Instruction - Acknowledge smart pin(s).

AKPIN {#}Src

Result: One or more Smart Pins is acknowledged; lowering their corresponding IN signal(s).

- Src is a register, 9-bit literal, or 11-bit augmented literal whose value identifies the Smart Pin(s) to acknowledge.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1100000 | 01I | 000000001 | SSSSSSSSS | Ack Bus | — | — | 2 |

Explanation:

AKPIN acknowledges the Smart Pin(s) designated by Src.  This lowers the corresponding IN signal(s) so that future Smart Pin events may raise them again later.

Src[5:0] indicates the pin number (0–63).  For a range of Smart Pins, Src[5:0] indicates the first pin number (0–63) and Src[10:6] indicates how many contiguous pins beyond the first should be affected (1–31).

A 9-bit literal Src is enough to express the starting pin (Src[5:0]) and a range of up to 8 contiguous pins (Src[8:6]).  If needed, use the augmented literal feature (##Src) to augment Src to the required 11-bit literal value— this inserts an AUGS instruction prior.

When Src is a register, the register's value bits [10:0] are used as-is to form the 11-bit Smart Pin range, unless a SETQ instruction immediately precedes the AKPIN instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for AKPIN's use.

The range calculation (from Src[5:0] up to Src[5:0]+Src[10:6]) will wrap within the same 32-pin group (DIRA or DIRB); it will not cross the port boundary.

## ALIGNL

Align long

Directive - Align to next long in Hub RAM.

DAT

code_and_data_statements

ALIGNL

data_statements

Result: The next data element is long-aligned in Hub RAM by emitting up to three bytes (each $00) prior.

- Code_and_data_statements are leading program code and/or data.

- Data_statements begin long-aligned in Hub RAM.

Explanation:

ALIGNL aligns the next data element to the beginning of the next long of Hub RAM.  ALIGNL is important to use when code requires certain data to begin on a long boundary (for access convenience and speed).

ALIGNL is only allowed in DAT blocks, not in in-line PASM.

Example

The following creates a data table of a byte ($11), a word ($2222), and a long ($33333333) meant for access from Hub RAM.

DAT

T1  BYTE $11

T2  WORD $2222

LONG $33333333

This data may be emitted into the Hub memory image like below; the actual data start and alignment will vary depending on the code and data that precede it.  The L#, W#, and B# labels denote contiguous long, word, and byte boundaries.

| L0 |  |  |  | L1 |  |  |  | L2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W0 |  | W1 |  | W2 |  | W3 |  | W4 |  | W5 |  |
| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| $11 | $22 | $22 | $33 | $33 | $33 | $33 | -- | -- | -- | -- | -- |

Notice how each data element, regardless of size, is packed right next to the data before it.  If the code that is meant to access Table T2 expects it to align with a long boundary (i.e. for convenience or speed), the ALIGNL directive achieves this, as follows.

DAT

T1  BYTE $11

ALIGNL

T2  WORD $2222

LONG $33333333

In comparison, this data will be emitted as follows:

| L0 |  |  |  | L1 |  |  |  | L2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W0 |  | W1 |  | W2 |  | W3 |  | W4 |  | W5 |  |
| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| $11 | $00 | $00 | $00 | $22 | $22 | $33 | $33 | $33 | $33 | -- | -- |

In this case, the ALIGNL instruction causes three zero ($00) bytes to emit after Table T1 to automatically pad and align the start of Table T2 to the boundary of L1.  Note that the second element (a long) of Table T2 is still packed right after the first element (a word) which may require further attention depending on the needs of the code accessing it.

## ALIGNW

Align word

Directive - Align to next word in Hub RAM.

DAT

code_and_data_statements

ALIGNW

data_statements

Result: The next data element is word-aligned in Hub RAM by emitting zero or one byte ($00) prior.

- Code_and_data_statements are leading program code and/or data.

- Data_statements begin word-aligned in Hub RAM.

Explanation:

ALIGNW aligns the next data element to the beginning of the next word of Hub RAM.  ALIGNW is important to use when code requires certain data to begin on a word boundary (for access convenience and speed).

ALIGNW is only allowed in DAT blocks, not in in-line PASM.

Example

The following creates a data table of a byte ($11), a word ($2222), and a long ($33333333) meant for access from Hub RAM.

DAT

T1  BYTE $11

T2  WORD $2222

LONG $33333333

This data may be emitted into the Hub memory image like below; the actual data start and alignment will vary depending on the code and data that precede it.  The L#, W#, and B# labels denote contiguous long, word, and byte boundaries.

| L0 |  |  |  | L1 |  |  |  | L2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W0 |  | W1 |  | W2 |  | W3 |  | W4 |  | W5 |  |
| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| $11 | $22 | $22 | $33 | $33 | $33 | $33 | -- | -- | -- | -- | -- |

Notice how each data element, regardless of size, is packed right next to the data before it.  If the code that is meant to access Table T2 expects it to align with a word boundary (i.e. for convenience or speed), the ALIGNW directive achieves this, as follows.

DAT

T1  BYTE $11

ALIGNW

T2  WORD $2222

LONG $33333333

In comparison, this data will be emitted as follows:

| L0 |  |  |  | L1 |  |  |  | L2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W0 |  | W1 |  | W2 |  | W3 |  | W4 |  | W5 |  |
| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| $11 | $00 | $22 | $22 | $33 | $33 | $33 | $33 | -- | -- | -- | -- |

In this case, the ALIGNW instruction causes one zero ($00) byte to emit after Table T1 to automatically pad and align the start of Table T2 to the boundary of W1.

## ALLOWI

Allow interrupts

Interrupt Instruction - Allow interrupts.

ALLOWI

Result: Any stalled and future interrupts are allowed.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | 000 | 000100000 | 000100100 | — | — | — | 2 |

Related: STALLI

Explanation:

ALLOWI re-enables interrupt branching; the default on cog start.  ALLOWI is the complement of the STALLI instruction— both are used to protect short, vital sections of main code from timing jitter or state loss caused by asynchronous interrupt handling.

## ALTB

Alter bit

Indirection Instruction - Alter subsequent BITxxx instruction.

ALTB Dest, {#}Src

ALTB Dest

Result: The next instruction's pipelined Dest value is altered to be (Src + Dest[13:5]) & $1FF, or just Dest[13:5] for syntax 2.

- Dest is the register whose 14-bit value is the index, or the full bit address, for the BITxxx instruction to operate on.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base long address (Src[8:0]; added to index (Dest[13:5]) for BITxxx) and also an optional auto-indexer value (Src[17:9]; added to Dest at the end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001100 | 11I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001100 | 111 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTB should be followed by a BITxxx instruction— it modifies the BITxxx instruction's Dest value, enabling code to iterate through multiple bits of data across a range of Reg RAM.  BITxxx's Dest value is changed to (Src + Dest[13:5]) & $1FF (for syntax 1), or to Dest[13:5] (for syntax 2).

Dest[13:5] corresponds to the target long register's 9-bit address and Dest[4:0] is the bit ID within it; values of 0–31 identify individual bits, by position, in least-significant bit order.  Iteratively executing ALTB followed by a BITxxx instruction, and each time incrementing ALTB's 14-bit Dest value by one, effectively writes a stream of bit values to Reg RAM as if it were all made of bit-sized registers.

Warning: BITxxx instructions optionally operate on a range of bits, encoded in the Src value— they don't limit themselves to only reading Src[4:0] for the bit number.  For this reason, care must be taken when using ALTB with BITxxx or the index value (often used for the Src of the altered instruction) will be misinterpreted as multiple bits to affect.  One way to solve this is to use a SETQ #0 followed by the ALTB then BITxxx instructions to force BITxxx's Src[9:5] bits to 0; i.e. no extra bits beyond the single bit described by Src[4:0].

In syntax 1, Src consists of two 9-bit fields; a base address (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base is the Reg RAM address where the series of bits begins.  ALTB adds the long index (Dest[13:5]) to the base (Src[8:0]) to locate the register holding the target bit.  The bit ID (Dest[4:0]) identifies the bit's position within that long register.

- At the end of ALTB execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the 14-bit index (Dest) for a future ALTB+BITxxx iteration.

In syntax 2, Dest serves as the full bit address— it's the same format as in syntax 1, but represents the target long's absolute address and its bit index instead of the long's relative index (to add to a base) and bit index.

Notes:

- The instruction following ALTB is shielded from interrupt

- ALTB alters the next instruction regardless of its kind— the intention is for it to be a BITxxx

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTD

Alter destination

Indirection Instruction - Alter D field of next instruction.

ALTD Dest, {#}Src

ALTD Dest

Result: The next instruction's pipelined Dest value is altered to be (Src + Dest) & $1FF, or just Dest[8:0] in syntax 2.

- Dest is the register whose 9-bit value is the offset, or the full value, for the next instruction to operate on.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base (Src[8:0]; added to offset (Dest) for the next instruction) and also an optional auto-indexer value (Src[17:9]; added to Dest at the end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001100 | 01I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001100 | 011 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTD modifies the next instruction's Dest value to be (Src + Dest) & $1FF (for syntax 1), or to Dest[8:0] (for syntax 2).

In syntax 1, Src consists of two 9-bit fields; a base value (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base represents a starting point.  ALTD adds the offset (Dest[8:0]) to the base (Src[8:0]) to determine the next instruction's Dest value.

- At the end of ALTD execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the offset (Dest) for a future ALTD+instruction iteration.

In syntax 2, Dest serves as the full value— it is used as-is for the next instruction's substitute Dest value.

Notes:

- The instruction following ALTD is shielded from interrupt

- ALTD alters the next instruction regardless of its kind

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTGB

Alter get byte

Indirection Instruction - Alter subsequent GETBYTE / ROLBYTE instruction.

ALTGB Dest, {#}Src

ALTGB Dest

Result: The next instruction's pipelined Src and Num fields are altered to be (Src + Dest[10:2]) & $1FF, or just Dest[10:2] for syntax 2, and Dest[1:0], respectively.

- Dest is the register whose 11-bit value is the index, or the full byte address, for the GETBYTE / ROLBYTE instruction to read.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base long address (Src[8:0]; added to index (Dest[10:2]) for GETBYTE / ROLBYTE) and also an optional auto-indexer value (Src[17:9]; added to Dest at end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001011 | 01I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001011 | 011 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTGB should be followed by GETBYTE or ROLBYTE— it modifies the GETBYTE / ROLBYTE instruction's Src and Num values, enabling code to iterate through multiple bytes of data across a range of Reg RAM.  GETBYTE / ROLBYTE's Src value is changed to (Src + Dest[10:2]) & $1FF (for syntax 1), or to Dest[10:2] (for syntax 2), and its Num value is changed to Dest[1:0].

Dest[10:2] corresponds to the target long register's 9-bit address and Dest[1:0] is the byte ID within it; values of 0–3 identify individual bytes, by position, in least-significant byte order.  Iteratively executing ALTGB followed by GETBYTE or ROLBYTE, and each time incrementing ALTGB's 11-bit Dest value by one, effectively reads a stream of byte values from Reg RAM as if it were all made of byte-sized registers.

In syntax 1, Src consists of two 9-bit fields; a base address (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base is the Reg RAM address where the series of bytes begins.  ALTGB adds the long index (Dest[10:2]) to the base (Src[8:0]) to locate the register holding the target byte.  The byte ID (Dest[1:0]) identifies the byte's position within that long register.

- At the end of ALTGB execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the 11-bit index (Dest) for a future ALTGB+GETBYTE or ROLBYTE iteration.

In syntax 2, Dest serves as the full byte address— it's the same format as in syntax 1, but represents the target long's absolute address and its byte index instead of the long's relative index (to add to a base) and byte index.

Notes:

- The instruction following ALTGB is shielded from interrupt

- ALTGB alters the next instruction regardless of its kind— the intention is for it to be a GETBYTE / ROLBYTE

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTGN

Alter get nibble

Indirection Instruction - Alter subsequent GETNIB / ROLNIB instruction.

ALTGN Dest, {#}Src

ALTGN Dest

Result: The next instruction's pipelined Src and Num values are altered to be (Src + Dest[11:3]) & $1FF, or just Dest[11:3] for syntax 2, and Dest[2:0], respectively.

- Dest is the register whose 12-bit value is the index, or the full nibble address, for the next GETNIB / ROLNIB instruction to read.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base long address (Src[8:0]; added to index (Dest[11:3]) for GETNIB / ROLNIB) and also an optional auto-indexer value (Src[17:9]; added to Dest at end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001010 | 11I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001010 | 111 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTGN should be followed by GETNIB or ROLNIB— it modifies the GETNIB / ROLNIB instruction's  Src and Num values, enabling code to iterate through multiple nibbles of data across a range of Reg RAM.  GETNIB / ROLNIB's Src value is changed to (Src + Dest[11:3]) & $1FF (for syntax 1), or to Dest[11:3] (for syntax 2), and its Num value is changed to Dest[2:0].

Dest[11:3] corresponds to the target long register's 9-bit address and Dest[2:0] is the nibble ID within it; values of 0–7 identify individual nibbles, by position, in least-significant nibble order.  Iteratively executing ALTGN followed by GETNIB or ROLNIB, and each time incrementing ALTGN's 12-bit Dest value by one, effectively reads a stream of nibble values from Reg RAM as if it were all made of nibble-sized registers.

In syntax 1, Src consists of two 9-bit fields; a base address (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base is the Reg RAM address where the series of nibbles begins.  ALTGN adds the long index (Dest[11:3]) to the base (Src[8:0]) to locate the register holding the target nibble.  The nibble ID (Dest[2:0]) identifies the nibble's position within that long register.

- At the end of ALTGN execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the 12-bit index (Dest) for a future ALTGN+GETNIB or ROLNIB iteration.

In syntax 2, Dest serves as the full nibble address— it's the same format as in syntax 1, but represents the target long's absolute address and its nibble index instead of the long's relative index (to add to a base) and nibble index.

Notes:

- The instruction following ALTGN is shielded from interrupt

- ALTGN alters the next instruction regardless of its kind— the intention is for it to be a GETNIB / ROLNIB

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTGW

Alter get word

Indirection Instruction - Alter subsequent GETWORD / ROLWORD instruction.

ALTGW Dest, {#}Src

ALTGW Dest

Result: The next instruction's pipelined Src and Num fields are altered to be (Src + Dest[9:1]) & $1FF, or just Dest[9:1] for syntax 2, and Dest[0], respectively.

- Dest is the register whose 10-bit value is the index, or the full word address for the GETWORD / ROLWORD instruction to read.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base long address (Src[8:0]; added to index (Dest[9:1]) for GETWORD / ROLWORD) and also an optional auto-indexer value (Src[17:9]; added to Dest at end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001011 | 11I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001011 | 111 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTGW should be followed by GETWORD or ROLWORD— it modifies the GETWORD / ROLWORD instruction's Src and Num values, enabling code to iterate through multiple words of data across a range of Reg RAM.  GETWORD / ROLWORD's Src value is changed to (Src + Dest[9:1]) & $1FF (for syntax 1), or to Dest[9:1] (for syntax 2), and its Num value is changed to Dest[0].

Dest[9:1] corresponds to the target long register's 9-bit address and Dest[0] is the word ID within it; values of 0–1 identify individual words, by position, in least-significant word order.  Iteratively executing ALTGW followed by GETWORD or ROLWORD, and each time incrementing ALTGW's 10-bit Dest value by one, effectively reads a stream of word values from Reg RAM as if it were all made of word-sized registers.

In syntax 1, Src consists of two 9-bit fields; a base address (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base is the Reg RAM address where the series of words begins.  ALTGW adds the long index (Dest[9:1]) to the base (Src[8:0]) to locate the register holding the target word.  The word ID (Dest[0]) identifies the word's position within that long register.

- At the end of ALTGW execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the 10-bit index (Dest) for a future ALTGW+GETWORD or ROLWORD iteration.

In syntax 2, Dest serves as the full word address— it's the same format as in syntax 1, but represents the target long's absolute address and its word index instead of the long's relative index (to add to a base) and word index.

Notes:

- The instruction following ALTGW is shielded from interrupt

- ALTGW alters the next instruction regardless of its kind— the intention is for it to be a GETWORD / ROLWORD

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTI

Alter instruction

Indirection Instruction - Substitute next instruction's field values from template, per configuration.

ALTI Dest, {#}Src

ALTI Dest

Result: The next instruction's pipelined field value values are substituted from the Dest template, and Dest is modified per Src configuration (syntax 1), or the entire Dest opcode (instruction) is executed in place of the next instruction (syntax 2).

- Dest is the register whose value contains one or more of the next instruction's field substitutes or an entire 32-bit opcode for full substitution.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value describes the substitutions and Dest modifications to perform.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001101 | 00I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1001101 | 001 | DDDDDDDDD | 101100100 | — | — | — | 2 |

Related: SETD, SETS, SETR, ALTD, ALTS, and ALTR

Explanation:

ALTI substitutes fields from Dest for one or more of the next instruction's pipelined Dest, Src, Result, Instr, FX, and/or Cond values, and ALTI's Dest is then modified per Src configuration (syntax 1), or the entire Dest opcode (instruction) is executed in place of the next instruction (syntax 2).

The Dest register contains the ALTI template; a 32-bit value with the following format similar to an opcode:

| Bits | 31:28 (4 bits) | 27:19 (9 bits) | 18 | 17:9 (9 bits) | 8:0 (9 bits) |
| --- | --- | --- | --- | --- | --- |
| Field Description | Condition Field | Result Field | Indirect "I" Field | Dest "D" Field | Source "S" Field |

In syntax 1, Src consists of the following six 3-bit fields that describe field substitution and Dest modification.

# %rrr_ddd_sss_RRR_DDD_SSS

- %rrr = Result field increment/decrement mask size

- %ddd = Dest field increment/decrement mask size

- %sss = Src field increment/decrement mask size

- %RRR = Result/Instruction field control

- %DDD = Dest field control

- %SSS = Src field control

| ###### Field Increment/Decrement Mask Size |  |
| --- | --- |
| %rrr %ddd %sss | Mask Selection |
| 000 | Unlimited Increment/Decrement of 9 bits (default, full 512-register Reg RAM span) |
| 001 | Limit Increment/Decrement to 8 LSBs (256-register Reg RAM looped buffer) |
| 010 | Limit Increment/Decrement to 7 LSBs (128-register Reg RAM looped buffer) |
| 011 | Limit Increment/Decrement to 6 LSBs (64-register Reg RAM looped buffer) |
| 100 | Limit Increment/Decrement to 5 LSBs (32-register Reg RAM looped buffer) |
| 101 | Limit Increment/Decrement to 4 LSBs (16-register Reg RAM looped buffer) |
| 110 | Limit Increment/Decrement to 3 LSBs (8-register Reg RAM looped buffer) |
| 111 | Limit Increment/Decrement to 2 LSBs (4-register Reg RAM looped buffer) |

| ###### Result/Instruction Field Control |  |  |  |
| --- | --- | --- | --- |
| %RRR | Substitution | Dest[27:19] Adjustment | Notes |
| 000 | None | None |  |
| 001 | None | None | Cancel next instruction's Result write |
| 010 | None | Decrements per %rrr |  |
| 011 | None | Increments per %rrr |  |
| 100 | Dest[27:19] ⇨ next instruction's Result field | None |  |
| 101 | Dest[31:18] ⇨ next instruction's [31:18] | None | Cond+Instr+Fx substitution1, 2 |
| 110 | Dest[27:19] ⇨ next instruction's Result field | Decrements per %rrr |  |
| 111 | Dest[27:19] ⇨ next instruction's Result field | Increments per %rrr |  |

1 Substitutes for next instruction's conditional execution mode, instruction class, and/or effects (WC/WZ, I, etc.)

2 Can use with %DDD=1xx and %SSS=1xx for full instruction substitution; i.e. execute Dest opcode in place of next instruction

| ###### Dest Field Control |  |  |  |
| --- | --- | --- | --- |
| %DDD | Substitution | Dest[17:9] Adjustment | Notes |
| 000 | None | None |  |
| 001 | None | None | Same as %000 |
| 010 | None | Decrements per %ddd |  |
| 011 | None | Increments per %ddd |  |
| 100 | Dest[17:9] ⇨ next instruction's Dest field | None | Execute Dest w/ RRR = %101, SSS = %1xx |
| 101 | Dest[17:9] ⇨ next instruction's Dest field | None | Execute Dest w/ RRR = %101, SSS = %1xx |
| 110 | Dest[17:9] ⇨ next instruction's Dest field | Decrements per %ddd | Execute Dest w/ RRR = %101, SSS = %1xx |
| 111 | Dest[17:9] ⇨ next instruction's Dest field | Increments per %ddd | Execute Dest w/ RRR = %101, SSS = %1xx |

| ###### Src Field Control |  |  |  |
| --- | --- | --- | --- |
| %SSS | Substitution | Src[8:0] Adjustment | Notes |
| 000 | None | None |  |
| 001 | None | None | Same as %000 |
| 010 | None | Decrements per %sss |  |
| 011 | None | Increments per %sss |  |
| 100 | Dest[8:0] ⇨ next instruction's Src field | None | Execute Dest w/ RRR = %101, DDD = %1xx |
| 101 | Dest[8:0] ⇨ next instruction's Src field | None | Execute Dest w/ RRR = %101, DDD = %1xx |
| 110 | Dest[8:0] ⇨ next instruction's Src field | Decrements per %sss | Execute Dest w/ RRR = %101, DDD = %1xx |
| 111 | Dest[8:0] ⇨ next instruction's Src field | Increments per %sss | Execute Dest w/ RRR = %101, DDD = %1xx |

In syntax 2, Dest serves as the full opcode value— it is executed as-is in place of the next instruction and Dest remains unaltered afterward.

Examples:

ALTI    ptrs,#%111_111	'set next D and S fields, increment ptrs[17:9] and ptrs[8:0]

ADD     0,0	'add registers

ALTI    inst	'execute inst; same as ALTI inst, #%101_100_100

NOP	'NOP becomes inst

Notes:

- The instruction following ALTI is shielded from interrupt

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTR

Alter result

Indirection Instruction - Alter Result register address of next instruction.

ALTR Dest, {#}Src

ALTR Dest

Result: The next instruction's pipelined Result address (Dest address by default) is altered to be (Src + Dest) & $1FF, or just Dest[8:0] in syntax 2.

- Dest is the register whose 9-bit value is the offset, or the full value, for the next instruction to operate on.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base (Src[8:0]; added to offset (Dest) for the next instruction) and also an optional auto-indexer value (Src[17:9]; added to Dest at the end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001100 | 00I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001100 | 001 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTR modifies the next instruction's Result address to be (Src + Dest) & $1FF (for syntax 1), or to Dest[8:0] (for syntax 2).

The Result address is the Dest address by default— it identifies where the result value from the instruction's execution is written at the end of execution.  During execution, the pipeline holds an instruction's Dest address and the Result address as two separate entities, normally set to the same location.  ALTR causes the next instruction's Result to redirect to a different address; changing an instruction from a destructive (operand overwriting) operation to a non-destructive (operand preserving) operation.

In syntax 1, Src consists of two 9-bit fields; a base value (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base represents a starting point.  ALTR adds the offset (Dest[8:0]) to the base (Src[8:0]) to determine the next instruction's Result address.

- At the end of ALTR execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the offset (Dest) for a future ALTR+instruction iteration.

In syntax 2, Dest serves as the full value— it is used as-is for the next instruction's substitute Result address.

Notes:

- The instruction following ALTR is shielded from interrupt

- ALTR alters the next instruction regardless of its kind

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTS

Alter source

Indirection Instruction - Alter S field of next instruction.

ALTS Dest, {#}Src

ALTS Dest

Result: The next instruction's pipelined Src value is altered to be (Src + Dest) & $1FF, or just Dest[8:0] in syntax 2.

- Dest is the register whose 9-bit value is the offset, or the full value, for the next instruction to operate on.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base (Src[8:0]; added to offset (Dest) for the next instruction) and also an optional auto-indexer value (Src[17:9]; added to Dest at the end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001100 | 10I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001100 | 101 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTS modifies the next instruction's Src value to be (Src + Dest) & $1FF (for syntax 1), or to Dest[8:0] (for syntax 2).

In syntax 1, Src consists of two 9-bit fields; a base value (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base represents a starting point.  ALTS adds the offset (Dest[8:0]) to the base (Src[8:0]) to determine the next instruction's Src value.

- At the end of ALTS execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the offset (Dest) for a future ALTS+instruction iteration.

In syntax 2, Dest serves as the full value— it is used as-is for the next instruction's substitute Src value.

Notes:

- The instruction following ALTS is shielded from interrupt

- ALTS alters the next instruction regardless of its kind

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTSB

Alter set byte

Indirection Instruction - Alter subsequent SETBYTE instruction.

ALTSB Dest, {#}Src

ALTSB Dest

Result: The next instruction's pipelined Dest and Num values are altered to be (Src + Dest[10:2]) & $1FF, or just Dest[10:2] for syntax 2, and Dest[1:0], respectively.

- Dest is the register whose 11-bit value is the index, or the full byte address, for the SETBYTE instruction to operate on.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base long address (Src[8:0]; added to index (Dest[10:2]) for SETBYTE) and also an optional auto-indexer value (Src[17:9]; added to Dest at end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001011 | 00I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001011 | 001 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTSB should be followed by SETBYTE— it modifies the SETBYTE instruction's Dest and Num values, enabling code to iterate through multiple bytes of data across a range of Reg RAM.  SETBYTE's Dest value is changed to (Src + Dest[10:2]) & $1FF (for syntax 1), or to Dest[10:2] (for syntax 2), and its Num value is changed to Dest[1:0].

Dest[10:2] corresponds to the target long register's 9-bit address and Dest[1:0] is the byte ID within it; values of 0–3 identify individual bytes, by position, in least-significant byte order.  Iteratively executing ALTSB followed by SETBYTE, and each time incrementing ALTSB's 11-bit Dest value by one, effectively writes a stream of byte values to Reg RAM as if it were all made of byte-sized registers.

In syntax 1, Src consists of two 9-bit fields; a base address (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base is the Reg RAM address where the series of bytes begins.  ALTSB adds the long index (Dest[10:2]) to the base (Src[8:0]) to locate the register holding the target byte.  The byte ID (Dest[1:0]) identifies the byte's position within that long register.

- At the end of ALTSB execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the 11-bit index (Dest) for a future ALTSB+SETBYTE iteration.

In syntax 2, Dest serves as the full byte address— it's the same format as in syntax 1, but represents the target long's absolute address and its byte index instead of the long's relative index (to add to a base) and byte index.

Notes:

- The instruction following ALTSB is shielded from interrupt

- ALTSB alters the next instruction regardless of its kind— the intention is for it to be a SETBYTE

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTSN

Alter set nibble

Indirection Instruction - Alter subsequent SETNIB instruction.

ALTSN Dest, {#}Src

ALTSN Dest

Result: The next instruction's pipelined Dest and Num values are altered to be (Src + Dest[11:3]) & $1FF, or just Dest[11:3] for syntax 2, and Dest[2:0], respectively.

- Dest is the register whose 12-bit value is the index, or the full nibble address, for the SETNIB instruction to operate on.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base long address (Src[8:0]; added to index (Dest[11:3]) for SETNIB) and also an optional auto-indexer value (Src[17:9]; added to Dest at the end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001010 | 10I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001010 | 101 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTSN should be followed by SETNIB— it modifies the SETNIB instruction's Dest and Num values, enabling code to iterate through multiple nibbles of data across a range of Reg RAM.  SETNIB's Dest value is changed to (Src + Dest[11:3]) & $1FF (for syntax 1), or to Dest[11:3] (for syntax 2), and its Num value is changed to Dest[2:0].

Dest[11:3] corresponds to the target long register's 9-bit address and Dest[2:0] is the nibble ID within it; values of 0–7 identify individual nibbles, by position, in least-significant nibble order.  Iteratively executing ALTSN followed by SETNIB, and each time incrementing ALTSN's 12-bit Dest value by one, effectively writes a stream of nibble values to Reg RAM as if it were all made of nibble-sized registers.

In syntax 1, Src consists of two 9-bit fields; a base address (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base is the Reg RAM address where the series of nibbles begins.  ALTSN adds the long index (Dest[11:3]) to the base (Src[8:0]) to locate the register holding the target nibble.  The nibble ID (Dest[2:0]) identifies the nibble's position within that long register.

- At the end of ALTSN execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the 12-bit index (Dest) for a future ALTSN+SETNIB iteration.

In syntax 2, Dest serves as the full nibble address— it's the same format as in syntax 1, but represents the target long's absolute address and its nibble index instead of the long's relative index (to add to a base) and nibble index.

Notes:

- The instruction following ALTSN is shielded from interrupt

- ALTSN alters the next instruction regardless of its kind— the intention is for it to be a SETNIB

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## ALTSW

Alter set word

Indirection Instruction - Alter subsequent SETWORD instruction.

ALTSW Dest, {#}Src

ALTSW Dest

Result: The next instruction's pipelined Dest and Num fields are altered to be (Src + Dest[9:1]) & $1FF, or just Dest[9:1] for syntax 2, and Dest[0], respectively.

- Dest is the register whose 10-bit value is the index, or the full word address, for the SETWORD instruction to operate on.

- Src is an optional register, 9-bit literal, or 18-bit augmented literal whose value contains a base long address (Src[8:0]; added to index (Dest[9:1]) for SETWORD) and also an optional auto-indexer value (Src[17:9]; added to Dest at end of execution).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001011 | 10I | DDDDDDDDD | SSSSSSSSS | D1 | — | — | 2 |
| EEEE | 1001011 | 101 | DDDDDDDDD | 000000000 | D1 | — | — | 2 |

1 Dest is post-adjusted by the auto-indexer value; the sign-extended Src[17:9].  In syntax 2, the auto-indexer value is 0.

Explanation:

ALTSW should be followed by SETWORD— it modifies the SETWORD instruction's Dest and Num values, enabling code to iterate through multiple words of data across a range of Reg RAM.  SETWORD's Dest value is changed to (Src + Dest[9:1]) & $1FF (for syntax 1), or to Dest[9:1] (for syntax 2), and its Num value is changed to Dest[0].

Dest[9:1] corresponds to the target long register's 9-bit address and Dest[0] is the word ID within it; values of 0–1 identify individual words, by position, in least-significant word order.  Iteratively executing ALTSW followed by SETWORD, and each time incrementing ALTSW's 10-bit Dest value by one, effectively writes a stream of word values to Reg RAM as if it were all made of word-sized registers.

In syntax 1, Src consists of two 9-bit fields; a base address (Src[8:0]) and a signed auto-indexer (Src[17:9]).

- The base is the Reg RAM address where the series of words begins.  ALTSW adds the long index (Dest[9:1]) to the base (Src[8:0]) to locate the register holding the target word.  The word ID (Dest[0]) identifies the word's position within that long register.

- At the end of ALTSW execution, the optional auto-indexer value (usually 0, 1, or -1) is added to the 10-bit index (Dest) for a future ALTSW+SETWORD iteration.

In syntax 2, Dest serves as the full word address— it's the same format as in syntax 1, but represents the target long's absolute address and its word index instead of the long's relative index (to add to a base) and word index.

Notes:

- The instruction following ALTSW is shielded from interrupt

- ALTSW alters the next instruction regardless of its kind— the intention is for it to be a SETWORD

- Field value modification occurs in the instruction pipeline only; code is not altered, values do not persist

- SETQ / SETQ2 does not affect ALTx instructions— the Q value passes through to the next instruction

## AND / ANDN

And not

Bit Operation Instruction - Bitwise AND a value with another, or with the NOT of another.

AND	Dest, {#}Src	{WC|WZ|WCZ}

ANDN	Dest, {#}Src	{WC|WZ|WCZ}

Result: Dest AND Src (or Dest AND !Src) is stored in Dest and flags are optionally updated with parity and zero status.

- Dest is the register containing the value to bitwise AND with Src (or with !Src) and is the destination in which to write the result.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value (or inverse value) will be bitwise ANDed into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0101000 | CZI | DDDDDDDDD | SSSSSSSSS | D | Parity of Result | Result = 0 | 2 |
| EEEE | 0101001 | CZI | DDDDDDDDD | SSSSSSSSS | D | Parity of Result | Result = 0 | 2 |

Explanation:

AND or ANDN performs a bitwise AND of the value in Src (or !Src) into that of Dest.

If the WC or WCZ effect is specified, the C flag is set (1) if the result contains an odd number of high (1) bits, or is cleared (0) if it contains an even number of high bits.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest AND Src (or Dest AND !Src) equals zero, or is cleared (0) if it is non-zero.

## AUGD

Augment destination

Augmentation Instruction - Augment next literal Dest to 32-bits.

AUGD #Dest

Result: The 23-bit value formed from Dest is queued to prefix the next literal Dest occurrence (#Dest) to form a 32-bit literal for that instruction; interrupts are also temporarily disabled..

- Dest is a 32-bit literal whose upper 23 bits are prepended to the next literal Dest occurrence.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 11111DD | DDD | DDDDDDDDD | DDDDDDDDD | Hidden D Queue | — | — | 2 |

Related: AUGS

Explanation:

AUGD is an assistant instruction to aid with literal values that exceed 9 bits.  Most PASM2 instructions have 9 bits available for literal Dest values; enough for many uses, but not all.  AUGD augments the next occurrence of a literal Dest value to be a full 32-bits.  When the instruction with the soon-to-be-augmented literal is later executed, the cog uses the lower 9 bits encoded in the instruction's Dest field and prepends AUGD's 23 bits to it.

Notes:

- All instructions following AUGD are shielded from interrupt until after the instruction with the newly-augmented literal Dest value is executed

- Dest value augmentation occurs in the instruction pipeline only; code is not altered, value does not persist

- SETQ / SETQ2 does not affect AUGD— the Q value passes through to the next instruction

Tip: Though AUGD may be manually entered wherever needed, the Parallax P2 compiler supports a convenient way to use this feature.  In the target instruction's Dest field, use "##" followed by the desired 32-bit literal (instead of "#" followed by a 9-bit literal); the compiler will automatically invoke AUGD immediately before.  When counting clock cycles, make sure to account for 2 extra clock cycles for instructions containing ## augmented literals.

Example

Both samples below use greater-than 9-bit literals (via AUGD functionality).  The first is more convenient and readable than using the AUGD instruction directly.

PUSHA   ##$ABCDEF	'2+3..10 cycles (in Cog RAM)

QMUL    ##8192, #4	'2+2..9 cycles

is the same as:

AUGD    #$ABCDEF	'2 cycles

PUSHA   #$ABCDEF & $1FF	'3..10 cycles (in Cog RAM)

AUGD    #8192	'2 cycles

QMUL    #8192 & $1FF, #4 	'2..9 cycles

## AUGS

Augment source

Augmentation Instruction - Augment next literal Src to 32-bits.

AUGS #Src

Result: The 23-bit value formed from Src is queued to prefix the next literal Src occurrence (#Src) to form a 32-bit literal for that instruction; interrupts are also temporarily disabled.

- Src is a 32-bit literal whose upper 23 bits are prepended to the next literal Src occurrence.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 11110SS | SSS | SSSSSSSSS | SSSSSSSSS | Hidden S Queue | — | — | 2 |

Related: AUGD

Explanation:

AUGS is an assistant instruction to aid with literal values that exceed 9 bits.  Most PASM2 instructions have 9 bits available for literal Src values; enough for many uses, but not all.  AUGS augments the next occurrence of a literal Src value to be a full 32-bits.  When the instruction with the soon-to-be-augmented literal is later executed, the cog uses the lower 9 bits encoded in the instruction's Src field and prepends AUGS's 23 bits to it.

Notes:

- All instructions following AUGS are shielded from interrupt until after the instruction with the newly-augmented literal Src value is executed

- Src value augmentation occurs in the instruction pipeline only; code is not altered, value does not persist

- SETQ / SETQ2 does not affect AUGS— the Q value passes through to the next instruction

Tip: Though AUGS may be manually entered wherever needed, the Parallax P2 compiler supports a convenient way to use this feature.  In the target instruction's Src field, use "##" followed by the desired 32-bit literal (instead of "#" followed by a 9-bit literal); the compiler will automatically invoke AUGS immediately before.  When counting clock cycles, make sure to account for 2 extra clock cycles for instructions containing ## augmented literals.

Example

Both samples below use greater-than 9-bit literals (via AUGS functionality).  The first is more convenient and readable than using the AUGS instruction directly.

AND   address, ##$FFFFF	'2+2 cycles

DJNZ  idx, ##far_away	'2+(2 or 4) cycles (in Cog RAM)

is the same as:

AUGS  #$FFFFF	'2 cycles

AND   address, #$FFFFF & $1FF	'2 cycles

AUGS  #far_away	'2 cycles

DJNZ  idx, #far_away & $1FF	'2 or 4 cycles (in Cog RAM)

## BITC / BITNC

Bit C or not C

Bit Operation Instruction - Set bit(s) low/high according to C or !C.

BITC	Dest, {#}Src	{WCZ}

BITNC	Dest, {#}Src	{WCZ}

Result: Dest bit(s) described by Src are set to C or !C; the rest are left as-is.

- Dest is the register whose value will have one or more bits set to C or !C.

- Src is a register, 9-bit literal, or 10-bit augmented literal whose value identifies the bit(s) to modify.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0100010 | CZI | DDDDDDDDD | SSSSSSSSS | D | Original D base bit | Original D base bit | 2 |
| EEEE | 0100011 | CZI | DDDDDDDDD | SSSSSSSSS | D | Original D base bit | Original D base bit | 2 |

Explanation:

BITC or BITNC alters the Dest bit(s) designated by Src to equal the state, or inverse state, of the C flag.  All other bits are left unchanged.

Src[4:0] indicates the bit number (0—31).  For a range of bits, Src[4:0] indicates the base bit number (0—31) and Src[9:5] indicates how many contiguous bits beyond the base should be affected (1–31).

A 9-bit literal Src is enough to express the base bit (Src[4:0]) and a range of up to 16 contiguous bits (Src[8:5]).  If needed, use the augmented literal feature (##Src) to augment Src to a 10-bit literal value— this inserts an AUGS instruction prior.

When Src is a register, the register's value bits [9:0] are used as-is to form the 10-bit ID range, unless a SETQ instruction immediately precedes the BITC / BITNC instruction; substituting SETQ's Dest[4:0] in place of value bits[9:5], for BITC / BITNC's use.

If the WCZ effect is specified, the C and Z flags are updated to the original state of Dest's base bit, identified by Src.

## BITH / BITL

Bit high or low

Bit Operation Instruction - Set bit(s) high (1) or low (0).

BITH Dest, {#}Src {WCZ}

BITL Dest, {#}Src {WCZ}

Result: Dest bit(s) described by Src are set high (1) or low (0); the rest are left as-is.

- Dest is the register whose value will have one or more bits set high or low.

- Src is a register, 9-bit literal, or 10-bit augmented literal whose value identifies the bit(s) to modify.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0100001 | CZI | DDDDDDDDD | SSSSSSSSS | D | Original D base bit | Original D base bit | 2 |
| EEEE | 0100000 | CZI | DDDDDDDDD | SSSSSSSSS | D | Original D base bit | Original D base bit | 2 |

Explanation:

BITH or BITL alters the Dest bit(s) designated by Src to be high (1) or low (0).  All other bits are left unchanged.

Src[4:0] indicates the bit number (0—31).  For a range of bits, Src[4:0] indicates the base bit number (0—31) and Src[9:5] indicates how many contiguous bits beyond the base should be affected (1–31).

A 9-bit literal Src is enough to express the base bit (Src[4:0]) and a range of up to 16 contiguous bits (Src[8:5]).  If needed, use the augmented literal feature (##Src) to augment Src to a 10-bit literal value— this inserts an AUGS instruction prior.

When Src is a register, the register's value bits [9:0] are used as-is to form the 10-bit ID range, unless a SETQ instruction immediately precedes the BITH / BITL instruction; substituting SETQ's Dest[4:0] in place of value bits[9:5], for BITH / BITL's use.

If the WCZ effect is specified, the C and Z flags are updated to the original state of Dest's base bit, identified by Src.

## BITNOT

Bit not

Bit Operation Instruction - Toggle bit(s) to the opposite state.

BITNOT Dest, {#}Src {WCZ}

Result: Dest bit(s) described by Src are toggled to their opposite state(s); the rest are left as-is.

- Dest is the register whose value will have one or more bits toggled.

- Src is a register, 9-bit literal, or 10-bit augmented literal whose value identifies the bit(s) to modify.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0100111 | CZI | DDDDDDDDD | SSSSSSSSS | D | Original D base bit | Original D base bit | 2 |

Explanation:

BITNOT alters the Dest bit(s) designated by Src to their inverse state.  All other bits are left unchanged.

Src[4:0] indicates the bit number (0—31).  For a range of bits, Src[4:0] indicates the base bit number (0—31) and Src[9:5] indicates how many contiguous bits beyond the base should be affected (1–31).

A 9-bit literal Src is enough to express the base bit (Src[4:0]) and a range of up to 16 contiguous bits (Src[8:5]).  If needed, use the augmented literal feature (##Src) to augment Src to a 10-bit literal value— this inserts an AUGS instruction prior.

When Src is a register, the register's value bits [9:0] are used as-is to form the 10-bit ID range, unless a SETQ instruction immediately precedes the BITNOT instruction; substituting SETQ's Dest[4:0] in place of value bits[9:5], for BITNOT's use.

If the WCZ effect is specified, the C and Z flags are updated to the original state of Dest's base bit, identified by Src.

## BITRND

Bit random

Bit Operation Instruction - Set bit(s) random low/high.

BITRND Dest, {#}Src {WCZ}

Result: Dest bit(s) described by Src are each set randomly low or high; the rest are left as-is.

- Dest is the register whose value will have one or more bits set randomly low or high.

- Src is a register, 9-bit literal, or 10-bit augmented literal whose value identifies the bit(s) to modify.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0100110 | CZI | DDDDDDDDD | SSSSSSSSS | D | Original D base bit | Original D base bit | 2 |

Explanation:

BITRND alters the Dest bit(s) designated by Src to each be an independent random low and high value, based on bit(s) from the Xoroshiro128** PRNG.  All other bits are left unchanged.

Src[4:0] indicates the bit number (0—31).  For a range of bits, Src[4:0] indicates the base bit number (0—31) and Src[9:5] indicates how many contiguous bits beyond the base should be affected (1–31).

A 9-bit literal Src is enough to express the base bit (Src[4:0]) and a range of up to 16 contiguous bits (Src[8:5]).  If needed, use the augmented literal feature (##Src) to augment Src to a 10-bit literal value— this inserts an AUGS instruction prior.

When Src is a register, the register's value bits [9:0] are used as-is to form the 10-bit ID range, unless a SETQ instruction immediately precedes the BITRND instruction; substituting SETQ's Dest[4:0] in place of value bits[9:5], for BITRND's use.

If the WCZ effect is specified, the C and Z flags are updated to the original state of Dest's base bit, identified by Src.

## BITZ / BITNZ

Bit Z or not Z

Bit Operation Instruction - Set bit(s) low/high according to Z or !Z.

BITZ	Dest, {#}Src	{WCZ}

BITNZ	Dest, {#}Src	{WCZ}

Result: Dest bit(s) described by Src are set to Z or !Z; the rest are left as-is.

- Dest is the register whose value will have one or more bits set to Z or !Z.

- Src is a register, 9-bit literal, or 10-bit augmented literal whose value identifies the bit(s) to modify.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0100100 | CZI | DDDDDDDDD | SSSSSSSSS | D | Original D base bit | Original D base bit | 2 |
| EEEE | 0100101 | CZI | DDDDDDDDD | SSSSSSSSS | D | Original D base bit | Original D base bit | 2 |

Explanation:

BITZ or BITNZ alters the Dest bit(s) designated by Src to equal the state, or inverse state, of the Z flag.  All other bits are left unchanged.

Src[4:0] indicates the bit number (0—31).  For a range of bits, Src[4:0] indicates the base bit number (0—31) and Src[9:5] indicates how many contiguous bits beyond the base should be affected (1–31).

A 9-bit literal Src is enough to express the base bit (Src[4:0]) and a range of up to 16 contiguous bits (Src[8:5]).  If needed, use the augmented literal feature (##Src) to augment Src to a 10-bit literal value— this inserts an AUGS instruction prior.

When Src is a register, the register's value bits [9:0] are natrally used as-is to form the 10-bit ID range, unless a SETQ instruction immediately precedes the BITZ / BITNZ instruction; substituting SETQ's Dest[4:0] in place of value bits[9:5], for BITZ / BITNZ's use.

If the WCZ effect is specified, the C and Z flags are updated to the original state of Dest's base bit, identified by Src.

## BMASK

Bit mask

Bit Operation Instruction - Get 1..32-bit mask into Dest.

BMASK Dest, {#}Src

BMASK Dest

Result: Bit mask of size Src+1, or Dest+1 (1—32 bits) is stored into Dest.

- Dest is the register in which to store the generated bit mask and optionally begins by containing the 5-bit mask size it is requesting (syntax 2).

- Src is an optional register or 5-bit literal whose value is the size of the bit mask to generate.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001110 | 01I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1001110 | 010 | DDDDDDDDD | DDDDDDDDD | D | — | — | 2 |

Explanation:

BMASK generates an LSB-justified bit mask (all ones) of Src+1 or Dest+1 length and stores it in Dest.  The size value, whether specified by Src or Dest, is in the range 0—31 to generate 1 to 32 bits of bit mask.

In effect, Dest becomes (%10 << size) - 1 via the BMASK instruction.

- A size value of 0 generates a bit mask of %00000000_00000000_00000000_00000001.

- A size value of 5 generates a bit mask of %00000000_00000000_00000000_00111111.

- A size value of 15 generates a bit mask of %00000000_00000000_11111111_11111111.

A bit mask is often useful in bitwise operations (AND, OR, XOR) to filter out or affect special groups of bits.

## BRK

Break

Interrupt Instruction - Trigger breakpoint in current cog.

BRK {#}Dest

Result: If debug interrupts are enabled, a debug interrupt is triggered in the current cog and Dest's value becomes the debug code or the next debug condition.

- Dest is the register, 9-bit literal, or 32-bit augmented literal whose value becomes the debug code or condition depending on the state of execution (outside or inside of a Debug ISR).

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | 00L | DDDDDDDDD | 000110110 | Code/Cond.1 | — | — | 2 |

1 Cog's internal Debug Code or Condition register is written depending on the state of execution.

Explanation:

BRK triggers a breakpoint in the current cog and either defines a breakpoint code or the next breakpoint condition(s).  The cog must have debug interrupts enabled, and if BRK is to be executed within the normal program (outside the Debug ISR), the "BRK instruction" interrupt must first be enabled from within a prior Debug ISR (interrupt service routine).

During normal program execution, the BRK instruction is used to generate a debug interrupt with an 8-bit code (from D[7:0]) which can be read within the Debug ISR using a GETBRK instruction.

During a Debug ISR, the BRK instruction is used instead to establish the next debug interrupt condition(s) and to select INA/INB, instead of the IJMP0/IRET0 registers exposed during the ISR, so that the pins' inputs states may be read.

The format of Dest for Debug ISR use is %AAAAAAAAAAAAAAAAAAAA_BCDEFGHIJKLM

A:	20-bit breakpoint address or 4-bit event code

B:	1 = map INA/INB normally, 0 = map IJMP0/IRET0 at INA/INB (default during ISR).  If Debug ISR sets B to 1, it must reset it to 0 before exiting the Debug ISR so the RETI0 instruction sees IJMP0 and IRET0.

C:	1 = enable interrupt on breakpoint address match

D:	1 = enable interrupt on event %eeee

E:	1 = enable interrupt on asynchronous breakpoint (via COGBRK from another cog)

F:	1 = enable interrupt on INT3 ISR entry

G:	1 = enable interrupt on INT2 ISR entry

H:	1 = enable interrupt on INT1 ISR entry

I:	1 = enable interrupt on BRK instruction

J:	1 = enable interrupt on INT3 ISR code (single step)

K:	1 = enable interrupt on INT2 ISR code (single step)

L:	1 = enable interrupt on INT1 ISR code (single step)

M:	1 = enable interrupt on non-ISR code (single step)

Upon Debug ISR entry, bits B through M are cleared (0).  If a subsequent debug interrupt is desired, a BRK instruction must be executed before exiting the Debug ISR in order to establish the next breakpoint condition(s).

## CALL

Call

Flow Control Instruction - Call a subroutine; store return context on the stack.

CALL #{\}Addr

CALL Dest {WC|WZ|WCZ}

Result: Push current C and Z flags and address of the next instruction onto the hardware stack, set PC to new relative or absolute address, and optionally update C and/or Z to new state.

- Addr is a symbolic reference to the target subroutine; the location to set PC to.  Relative addressing is the default; use '\' to force absolute addressing.

- Dest is a register containing the 20-bit absolute address to set PC to and optional new C and Z states.

- WC, WZ, or WCZ are optional effects to update the flags from Dest's upper bit states.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101101 | RAA | AAAAAAAAA | AAAAAAAAA | K1 and PC | — | — | 4 / 13–20 |
| EEEE | 1101011 | CZ0 | DDDDDDDDD | 000101101 | K1 and PC | D[31] | D[30] | 4 / 13–20 |

1 The current C, Z, and effective next PC values are pushed onto the stack (K) prior to replacing them with that of Addr or Dest.

Explanation:

CALL records the current state of the C and Z flags and the address of the next instruction (PC + 1 if Cog/LUT execution; PC + 4 if Hub execution) by pushing to the stack (K), potentially updates the C and Z flags with new given states, and jumps to the given address or offset.  The routine at the new address should eventually execute a RET instruction, or an instruction with a _RET_ condition, to return to the recorded address (the instruction following the CALL) and optionally restore the C and Z flag state as it was prior.

In syntax 1, #Addr and #\Addr encodes the instruction with relative and absolute addressing, respectively.  The relative form (the default) is vital for creating relocatable code.  In either case, use symbolic references for Addr and the assembler will encode it properly.  Examples: CALL #SendBit or CALL #\DebugStatus

In syntax 2, the format of the value at Dest is CZxxxxxx_xxxxAAAA_AAAAAAAA_AAAAAAAA.  C is the new C flag state, Z is the new Z flag state, A is the new 20-bit address to jump to, and x are don't-care bits.  Syntax 2 effectively swaps the flags and PC with the value in the Dest register (and RET swaps them back), making it convenient for switching between two threads.

If the WC or WCZ effect is specified, the C flag is updated to match D[31], after its original state is recorded.

If the WZ or WCZ effect is specified, the Z flag is updated to match D[30], after its original state is recorded.

## CALLA / CALLB

Call A or B

Flow Control Instruction - Call a subroutine; store return context in the Hub long at PTRA++ or PTRB++.

CALLA #{\}Addr

CALLA Dest {WC|WZ|WCZ}

CALLB #{\}Addr

CALLB Dest {WC|WZ|WCZ}

Result: Write current C and Z flags and address of the next instruction into the 4-byte Hub RAM location at PTRA or PTRB, increment pointer, set PC to new relative or absolute address, and optionally update C and/or Z to new state.

- Addr is a symbolic reference to the target subroutine; the location to set PC to.  Relative addressing is the default; use '\' to force absolute addressing.

- Dest is a register containing the 20-bit absolute address to set PC to and optional new C and Z states.

- WC, WZ, or WCZ are optional effects to update the flags from Dest's upper bit states.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101110 | RAA | AAAAAAAAA | AAAAAAAAA | Hub1 and PC | — | — | 5–122 / 14–322 |
| EEEE | 1101011 | CZ0 | DDDDDDDDD | 000101110 | Hub1 and PC | D[31] | D[30] | 5–122 / 14–322 |
| EEEE | 1101111 | RAA | AAAAAAAAA | AAAAAAAAA | Hub1 and PC | — | — | 5–122 / 14–322 |
| EEEE | 1101011 | CZ0 | DDDDDDDDD | 000101111 | Hub1 and PC | D[31] | D[30] | 5–122 / 14–322 |

1 The current C, Z, and effective next PC values are written to the Hub RAM long (H) referenced by PTRA or PTRB prior to replacing them with that of Addr or Dest.

2 +1 clock cycle if target address isn't long-aligned in Hub RAM.

Explanation:

CALLA and CALLB records the current state of the C and Z flags and the address of the next instruction (PC + 1 if Cog/LUT execution; PC + 4 if Hub execution) by writing them to the Hub RAM long (H) referenced by PTRA or PTRB, increments PTRA or PTRB, potentially updates the C and Z flags with new given states, and jumps to the given address or offset.  The routine at the new address should eventually execute a RETA or RETB instruction to return to the recorded address (the instruction following the CALLA or CALLB) and optionally restore the C and Z flag state as it was prior.

In syntax 1 and 3, #Addr and #\Addr encodes the instruction with relative and absolute addressing, respectively.  The relative form (the default) is vital for creating relocatable code.  In either case, use symbolic references for Addr and the assembler will encode it properly.  Examples: CALLA #SendBit or CALLB #\DebugStatus

In syntax 2 and 4, the format of the value at Dest is CZxxxxxx_xxxxAAAA_AAAAAAAA_AAAAAAAA.  C is the new C flag state, Z is the new Z flag state, A is the new 20-bit address to jump to, and x are don't-care bits.  Syntax 2 effectively swaps the flags and PC with the value in the Dest register (and RETA or RETB swaps them back), making it convenient for switching between two threads.

If the WC or WCZ effect is specified, the C flag is updated to match D[31], after its original state is recorded.

If the WZ or WCZ effect is specified, the Z flag is updated to match D[30], after its original state is recorded.

## CALLD

Call D

Flow Control Instruction - Call a subroutine; store return context in PA/PB/PTRA/PTRB/D.

CALLD PA|PB|PTRA|PTRB, #{\}Addr

CALLD Dest, {#}Src {WC|WZ|WCZ}

Result: Write current C and Z flags and address of the next instruction into either PA, PB, PTRA, PTRB, or Dest, set PC to new relative or absolute address (in Addr or Src), and optionally update C and/or Z to new state.

- PA|PB|PTRA|PTRB is the special register to store the current C and Z flags and next address into.

- Addr is a symbolic reference to the target subroutine; the location to set PC to.  Relative addressing is the default; use '\' to force absolute addressing.

- Dest is a register to write the current C and Z flags and the address of the next instruction into.

- Src is a register, 9-bit literal, or 32-bit augmented literal that contains the relative or absolute address to set PC to and optional new C and Z states.  Use # for relative addressing; omit # for absolute addressing.

- WC, WZ, or WCZ are optional effects to update the flags from Src's upper bit states.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 11100WW | RAA | AAAAAAAAA | AAAAAAAAA | Pxxx1 and PC | — | — | 4 / 13–20 |
| EEEE | 1011001 | CZI | DDDDDDDDD | SSSSSSSSS | D1 and PC | S[31] | S[30] | 4 / 13–20 |

1 The current C, Z, and effective next PC values are written to PA, PB, PTRA, PTRB, or Dest prior to replacing them with that of Addr or Src.

Explanation:

CALLD records the current state of the C and Z flags and the address of the next instruction (PC + 1 if Cog/LUT execution; PC + 4 if Hub execution) by writing them to the PA, PB, PTRA, PTRB, or Dest register, potentially updates the C and Z flags with new given states, and jumps to the given address or offset.  The routine at the new address should eventually execute another CALLD instruction to return to the recorded address (the instruction following the original CALLD) optionally restore the C and Z flag state as it was prior, and optionally prep for another CALLD.  This instruction is typically used for the P2 DEBUG function.

In syntax 1, #Addr and #\Addr encodes the instruction with relative and absolute addressing, respectively.  The relative form (the default) is vital for creating relocatable code.  In either case, use symbolic references for Addr and the assembler will encode it properly.  Examples: CALLD PA, #SendBit or CALLD PB, #\DebugStatus

In syntax 2, the format of the value at Src is CZxxxxxx_xxxxAAAA_AAAAAAAA_AAAAAAAA.  C is the new C flag state, Z is the new Z flag state, A is the new 20-bit address to jump to, and x are don't-care bits.  If Src is a 9-bit literal (immediate), it will be sign-extended to 20 bits and used as a relative offset; giving a range of -256 to +255 instructions, relative to the instruction following the CALLD.  When relative, PC is adjusted by signed(Src) if Cog/LUT execution, or by signed(Src*4) if Hub execution.

If the WC or WCZ effect is specified, the C flag is updated to match S[31], after its original state is recorded.

If the WZ or WCZ effect is specified, the Z flag is updated to match S[30], after its original state is recorded.

## CALLPA / CALLPB

Call parameter A or B

Flow Control Instruction - Call a subroutine; store return context on the stack and copy D into PA or PB.

CALLPA {#}Dest, {#}Src

CALLPB {#}Dest, {#}Src

Result: Push current C and Z flags and address of the next instruction onto the hardware stack, copy D to PA or PB, and set PC to new relative or absolute address.

- Dest is a register, 9-bit literal, or 32-bit augmented literal whose value is copied to PA or PB.

- Src is a register, 9-bit literal, or 32-bit augmented literal that contains the relative or absolute address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011010 | 0LI | DDDDDDDDD | SSSSSSSSS | K1, PA and PC | — | — | 4 / 13–20 |
| EEEE | 1011010 | 1LI | DDDDDDDDD | SSSSSSSSS | K1, PB and PC | — | — | 4 / 13–20 |

1 The current C, Z, and effective next PC values are pushed onto the stack (K) prior to updating PC with Src.

Explanation:

CALLPA and CALLPB records the current state of the C and Z flags and the address of the next instruction (PC + 1 if Cog/LUT execution; PC + 4 if Hub execution) by pushing to the stack (K), copies Dest to PA or PB, and jumps to the given address or offset in Src.  The routine at the new address should eventually execute a RET instruction, or an instruction with a _RET_ condition, to return to the recorded address (the instruction following the CALLPA or CALLPB) and optionally restore the C and Z flag state as it was prior.

The Src can be absolute or relative; use #Addr for calling a range of -256 to +255 instructions, relative to the instruction following the CALLPA or CALLPA.  When relative, PC is adjusted by signed(Src) if Cog/LUT execution, or by signed(Src*4) if Hub execution.

## CMP

Compare

Math Instruction - Compare two unsigned values.

CMP Dest, {#}Src {WC|WZ|WCZ}

Result: Greater/lesser and equality status is optionally written to the C and Z flags.

- Dest is the register containing the value to compare with that of Src.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is compared to Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0010000 | CZI | DDDDDDDDD | SSSSSSSSS | — | Unsigned (D < S) | D = S | 2 |

Related: CMPR, CMPX, CMPS, and CMPSX

Explanation:

CMP compares the unsigned values of Dest and Src (by subtracting Src from Dest) and optionally setting the C and Z flags accordingly.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest is less than Src.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Dest equals Src.

To compare unsigned, multi-long values, use CMP followed by CMPX as described in Comparing Two Multi-Long Values.

## CMPM

Compare most significant bit

Math Instruction - Compare two unsigned values, get MSB of difference.

CMPM Dest, {#}Src {WC|WZ|WCZ}

Result: Greater/lesser and equality status is optionally written to the C and Z flags.

- Dest is the register containing the value to compare with that of Src.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is compared to Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0010101 | CZI | DDDDDDDDD | SSSSSSSSS | — | Result[31] | D = S | 2 |

Explanation:

CMPM compares the unsigned values of Dest and Src (by subtracting Src from Dest) and optionally setting the C and Z flags accordingly.

If the WC or WCZ effect is specified, the C flag is updated to the MSB of (Dest - Src).

If the WZ or WCZ effect is specified, the Z flag is set (1) if Dest equals Src.

## CMPR

Compare reverse

Math Instruction - Compare two unsigned values (in reverse order to CMP).

CMPR Dest, {#}Src {WC|WZ|WCZ}

Result: Greater/lesser and equality status is optionally written to the C and Z flags.

- Dest is the register containing the value to compare with that of Src.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is compared to Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0010100 | CZI | DDDDDDDDD | SSSSSSSSS | — | Unsigned (S < D) | D = S | 2 |

Related: CMP

Explanation:

CMP compares the unsigned values of Dest and Src (by subtracting Dest from Src) and optionally setting the C and Z flags accordingly.

If the WC or WCZ effect is specified, the C flag is set (1) if Src is less than Dest.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Dest equals Src.

To compare unsigned, multi-long values, use CMP (instead) followed by CMPX as described in Comparing Two Multi-Long Values.

## CMPS

Compare signed

Math Instruction - Compare two signed values.

CMPS Dest, {#}Src {WC|WZ|WCZ}

Result: Greater/lesser and equality status is optionally written to the C and Z flags.

- Dest is the register containing the value to compare with that of Src.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is compared to Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0010010 | CZI | DDDDDDDDD | SSSSSSSSS | — | Signed (D < S) | D = S | 2 |

Related: CMP, CMPX, and CMPSX

Explanation:

CMPS compares the signed values of Dest and Src (by subtracting Src from Dest) and optionally setting the C and Z flags to indicate the comparison and operation results.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest is less than Src.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Dest equals Src.

To compare signed, multi-long values, use CMP (not CMPS) followed possibly by CMPX, and finally CMPSX as described in Comparing Two Multi-Long Values.

## CMPSUB

Compare and subtract

Math Instruction - Compare two unsigned values and subtract the second if it is lesser or equal.

CMPSUB Dest, {#}Src {WC|WZ|WCZ}

Result: Dest is decremented by Src unless it is less than Src, and the comparison results are optionally written to the C and Z flags.

- Dest is the register containing the value to compare with Src and is the destination written to if a subtraction is performed.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is compared with and possibly subtracted from Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0010111 | CZI | DDDDDDDDD | SSSSSSSSS | D1 | Unsigned(D => S) | Result = 0 | 2 |

1 Dest is only written if a subtraction was performed on it.

Explanation:

CMPSUB compares the unsigned values of Dest and Src, and if Src is less than or equal to Dest then it is subtracted from Dest.  Optionally, the C and Z flags are set to indicate the comparison and operation results.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest was greater than or equal to Src.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result equals 0.

## CMPSX

Compare signed, extended

Math Instruction - Compare two signed values plus carry flag.

CMPSX Dest, {#}Src {WC|WZ|WCZ}

Result: Greater/lesser and equality status is optionally written to the C and Z flags.

- Dest is the register containing the value to compare with that of Src.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is compared to Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0010011 | CZI | DDDDDDDDD | SSSSSSSSS | — | Signed (D < S+C) | Z AND (D = (S+C)) | 2 |

Related: CMP, CMPX, and CMPSX

Explanation:

CMPSX compares the signed values of Dest and Src plus C (by subtracting Src + C from Dest) and optionally setting the C and Z flags accordingly.  The CMPSX instruction is used to perform signed multi-long comparisons, such as 64-bit comparisons.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest is less than Src + C (as multi-long values).  Use WC or WCZ on preceding CMP and CMPX instructions for proper final C flag.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Z was previously set and the result of Dest - (Src + C) is zero, or it is cleared (0) if non-zero.  Use WZ or WCZ on preceding SUB and SUBX instructions for proper final Z flag.

To compare signed, multi-long values, use CMP (not CMPS) followed possibly by CMPX, and finally CMPSX as described in Comparing Two Multi-Long Values.

## CMPX

Compare extended

Math Instruction - Compare two unsigned values plus carry flag.

CMPX Dest, {#}Src {WC|WZ|WCZ}

Result: Greater/lesser and equality status is optionally written to the C and Z flags.

- Dest is a register containing the value to compare with that of Src plus C.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value plus C is compared to Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0010001 | CZI | DDDDDDDDD | SSSSSSSSS | — | Unsigned (D < S+C) | Z AND (D = S+C) | 2 |

Related: CMP, CMPX, and CMPSX

Explanation:

CMPX compares the unsigned values of Dest and Src plus C (by subtracting Src + C from Dest) and optionally setting the C and Z flags accordingly.  The CMPX instruction is used to perform unsigned multi-long comparisons, such as 64-bit comparisons.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest is less than Src plus C, or is cleared (0) otherwise.  Use WC or WCZ on preceding CMP and CMPX instructions for proper final C flag.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Z was previously set and Dest equals Src + C, or it is cleared (0) otherwise.  Use WZ or WCZ on preceding CMP and CMPX instructions for proper final Z flag.

To compare multi-long values, use CMP followed by one or more CMPX instructions as described in Comparing Two Multi-Long Values.

## COGATN

Cog attention

Event Handling Instruction - Get the attention of one or more other cogs.

COGATN {#}Dest

Result: The attention signal of one or more cogs is strobed.

- Dest is the register or 9-bit literal whose value (lower 8-bit pattern) indicates which cogs to signal.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | 00L | DDDDDDDDD | 000111111 | — | — | — | 2 |

Related: POLLATN, WAITATN, JATN, and JNATN

Explanation:

COGATN strobes the attention signal for one or more cogs.  Dest bit positions 7:0 represent cogs 7 through 0; high (1) bits indicate the cog(s) to signal.  The receiving cog(s) then latch the signal, setting an internal flag, and can use any of the attention monitor instructions (JATN, JNATN, POLLATN, WAITATN) or interrupts to respond and clear the flag.

COGATN   #%00100010			'Get attention of cogs 1 and 5

In the intended use case, the cog receiving an attention request knows which other cog is strobing it and how to respond.  In cases where multiple cogs may request the attention of a single cog, some messaging structure may need to be implemented in Hub RAM to differentiate requests.

## COGBRK

Cog break

Interrupt Instruction - Trigger breakpoint in specified cog.

COGBRK {#}Dest

Result: If in the Debug ISR, trigger an asynchronous breakpoint in cog identified by Dest.

- Dest is the register or 9-bit literal whose value (lower 3-bits) indicates which cog to trigger.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | 00L | DDDDDDDDD | 000110101 | — | — | — | 2 |

Explanation:

COGBRK triggers an asynchronous breakpoint in a designated cog.  The COGBRK instruction must be executed from within a Debug ISR (interrupt service routine) and the designated cog must already have its asynchronous breakpoint interrupt enabled.  Dest[2:0] indicates the ID of the desired cog.

## COGID

Cog identification

Cog Control Instruction - Get current cog’s ID or any cog's status by ID.

COGID {#}Dest {WC}

Result: Current cog's ID is written to Dest or C is set (1) or cleared (0) if the Dest cog is running or stopped.

- Dest is the register where the current cog's ID will be written, or is the register or 9-bit literal whose value (lower 3-bits) indicates which cog to get the status for.

- WC is an optional effect to update the C flag with the Dest cog's running status.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | C0L | DDDDDDDDD | 000000001 | D if reg and !WC | Cog Running | — | 2–9, +2 if result |

Related: COGINIT and COGSTOP

Explanation:

COGID writes the current cog's ID into Dest (if Dest is a register and WC is omitted) or sets/clears the C flag according to the running/stopped state of the cog indicated by Dest[2:0] (if WC is given).

If the WC effect is specified, the C flag is set (1) if the Dest[2:0] cog is running, or is cleared (0) if stopped.

## COGINIT

Cog initialize

Cog Control Instruction - Start an available cog, or restart a cog by ID.

COGINIT {#}Dest, {#}Src {WC}

Result: Target cog is started, according to Dest, to execute code from Src.  The code pointer (Src) is written to the target Cog's PTRB, and optionally a data pointer or user value is written to its PTRA (if previous SETQ is provided).

- Dest is the register or 9-bit literal describing the type of launch and possibly the ID of the desired cog to launch.  If Dest is a register and WC is given, Dest is also where the ID of the launched cog will be written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value (lower 20 bits) is the target RAM address (for code) and the new cog's PTRB value.

- WC is an optional effect to update the C flag with the success (0) or fail (1) status and triggers Dest to be overwritten with new cog's ID.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1100111 | CLI | DDDDDDDDD | SSSSSSSSS | D if reg and WC | No cog available | — | 2–9, +2 if result |

Related: COGID and COGSTOP

Explanation:

COGINIT starts a new (unused) cog, a new pair of cogs (that may share LUT memory), or a specific cog by ID, to load code from Hub RAM to be executed within Reg/LUT RAM or to be executed right from Hub RAM.

If the WC effect is specified, the C flag is set (1) if the instruction failed to launch a cog, or is cleared (0) if it was  successful.  Also with the WC effect, if Dest is a register, it is overwritten with $F (on failure) or with the ID of the newly launched cog —or the lowest of the pair of cogs— (on success).

The format of Dest is %E_N_xVVV.

E:	0 = load from Hub RAM (Src) into target cog's Reg RAM ($000–$1F7) and begin execution at its Reg 	RAM ($000); 1 = no load, just execute from target cog's existing Reg/LUT RAM, or from Hub RAM.

N:	0 = target cog ID is V; 1 = start a free cog or cog pair according to V.

V:	when N = 0, V is a specific cog ID (0–7; %000–%111); when N = 1, V is 0 (%000) to start a free cog or 1 (%001) to start a free odd/even pair of cogs for LUT RAM sharing.  Use the WC effect to get the results in Dest and C.

The lower 20 bits of Src is the address of the start of code; the address in Hub RAM (or Reg/LUT RAM) where the executable code image begins.  The entire 32-bit Src value will be written into the target cog's PTRB register.

If COGINIT is preceded by SETQ, the SETQ value will be written into the target cog's PTRA register. This is intended as a convenient means of pointing the target cog's program to some runtime data structure or passing it a 32-bit parameter. If no SETQ is used, the target cog's PTRA register will be cleared to zero.

Related instructions are COGID and COGSTOP.

Execution Modes

Code can be loaded into Cog RAM (Reg RAM and optionally LUT RAM) and executed, or code can be executed from Hub RAM without pre-loading into Cog RAM.  For PASM2 programs, the former achieves the fastest execution speed and natural isolation from other cogs while the latter achieves a larger effective code space beyond the ≈1,000 instruction limit inside Cog RAM.  It is common to run PASM2 code from Reg RAM.

To execute code from Reg RAM, it must first be loaded from Hub RAM (i.e. a Dest value where E = 0 achieves both load and execute in a single COGINIT instruction).

To execute code from LUT RAM, it must first be manually copied or streamed into LUT RAM by code executing in Reg RAM (or by the neighboring cog in an even/odd pair that are sharing LUT RAM).

To execute code from Hub RAM, the Dest's E bit must be 1 and the Src must be an address beyond $3FF.  Note that with a single COGINIT instruction, code could start out loaded into Reg RAM (E = 0), execute from there, and can branch to $400 or beyond (Hub execution), or it can even copy or stream code into LUT RAM, execute in Reg+LUT RAM, and then branch to $400 or beyond (Hub execution).

Additionally, it's possible to load and execute code in another cog's Reg RAM (one COGINIT instruction) and restart that cog thereafter (another COGINIT instruction) to execute starting at any place in its existing Reg RAM or LUT RAM (from the previous load) by using a Dest value with E = 1 and a Src value of less than $400.  This works whether or not the cog is terminated first, since its Cog RAM is retained during its terminated (dormant) state.  For example, one possibility is a Reg/LUT RAM-resident program consisting of a number of small independent routines which are each exclusively started at-will by another cog— apparent cog launch and parallel processing with no "loading" overhead latency.

COGINIT Constants

A number of constants are predefined for use in forming desired Dest values for COGINIT instructions.

| ###### COGINIT Constants |  |  |
| --- | --- | --- |
| Symbol (for Dest) | Dest Value | Notes |
| COGEXEC | %0_0_0000 | The target cog loads its own Reg RAM registers $000–$1F7 from the Hub RAM starting at Src address, then begins execution at register address $000. Target cog ID must be specified; i.e. COGEXEC+5 will launch Cog 5 in this fashion. |
| COGEXEC_NEW | %0_1_0000 | Same as above except an available (non-running) cog is chosen automatically; do not specify an ID. Use WC effect and a register for Dest (register's value set to COGEXEC_NEW) to get pass/fail (0/1) result in C flag and ID of employed cog in Dest register. |
| COGEXEC_NEW_PAIR | %0_1_0001 | Same as above except an available (non-running) even/odd pair of cogs is chosen automatically— useful for LUT RAM sharing. Dest receives the lowest ID of the pair if successful (when the WC effect is used). |
| HUBEXEC | %1_0_0000 | No loading; the target cog begins execution at Reg/LUT/Hub RAM address Src. Target cog ID must be specified; i.e. HUBEXEC+2 will launch Cog 2 in this fashion. NOTE: Though named HUBEXEC, this constant can also be used to execute pre-existing code in Reg RAM or LUT RAM as well as Hub RAM. |
| HUBEXEC_NEW | %1_1_0000 | Same as above except an available (non-running) cog is chosen automatically; do not specify an ID. Use WC effect and a register for Dest (register's value set to HUBEXEC_NEW) to get pass/fail (0/1) result in C flag and ID of employed cog in Dest register. |
| HUBEXEC_NEW_PAIR | %1_1_0001 | Same as above except an available (non-running) even/odd pair of cogs is chosen automatically— useful for LUT RAM sharing. Dest receives the lowest ID of the pair if successful (when the WC effect is used). |

Examples

The following are individual examples.

COGINIT #1,#$100	'load and start Cog 1 from Hub RAM $100

—or—

COGINIT #COGEXEC+1,#$100

COGINIT #%1_0_0101, PTRA	'skip load and start Cog 5 at PTRA

—or—

COGINIT #HUBEXEC+5, PTRA

SETQ    ptra_val	'ptra_val will go into target cog's PTRA register

COGINIT #%0_1_0000, addr	'load and start a free cog at addr

—or—

SETQ    ptra_val

COGINIT #COGEXEC_NEW, addr

COGINIT #%1_1_0001, addr	'skip load, start a free pair of cogs at addr (for LUT RAM sharing)

—or—

COGINIT #HUBEXEC_NEW_PAIR, addr

COGINIT id, addr  WC	'(id=HUBEXEC_NEW) skip load, start a free cog at addr

'C=0 and id=cog if okay

COGID   myID	'reload and restart me at Hub RAM PTRB

COGINIT myID, PTRB

## COGSTOP

Cog stop

Cog Control Instruction - Stop a cog by ID.

COGSTOP {#}Dest

Result: Cog indicated by Dest is terminated (stopped).

- Dest is the register or 9-bit literal indicating (in lowest 3 bits) which cog to stop.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | 00L | DDDDDDDDD | 000000011 | — | — | — | 2–9 |

Related: COGINIT and COGSTOP

Explanation:

COGSTOP terminates the cog identified by Dest[2:0].  In this dormant state, the cog ceases to execute code and power consumption is greatly reduced.

## Conditions ( IF_x )

Every PASM2 instruction has an optional “condition” that dynamically executes or excludes the instruction based on flag settings at runtime.  A condition, if provided, is placed in front of the instruction it controls.

{Label}  {Condition}  Instruction  Operands  {Effect}

Conditions are optional; omitting the condition means "always execute" the instruction (the default behavior).  The Condition field on an instruction line can contain one of fifty condition symbols (including blank; the default).  The full set of condition symbols resolves down to 16 unique condition patterns (4-bits) that are stored in the instruction's opcode (the COND field) during assembly.

This Condition feature, along with the instructions’ optional Effect feature, makes Propeller 2 Assembly very powerful.  For example, the C and Z flags can be affected at will and later instructions can be conditionally executed based on those results, making for easy behavioral changes through a single non-branching code path.

When an instruction’s condition evaluates to FALSE, the instruction effectively does nothing— still elapsing 2 clock cycles, but not affecting any flags or registers. This makes the timing of multi-decision code deterministic.

| ###### Conditions |  |  |
| --- | --- | --- |
| Condition1 | Encoding2 | Instruction Executes... |
| IF_E | %1010 | if comparison/subtraction was equal (Z = 1) |
| IF_NE | %0101 | if comparison/subtraction was not equal (Z = 0) |
| IF_A | %0001 | if comparison/subtraction was above (C = 0 and Z = 0) |
| IF_AE | %0011 | if comparison/subtraction was above or equal (C = 0) |
| IF_B | %1100 | if comparison/subtraction was below (C = 1) |
| IF_BE | %1110 | if comparison/subtraction was below or equal (C = 1 or Z = 1) |
| IF_GT | %0001 | if comparison/subtraction was greater than (C = 0 and Z = 0) |
| IF_GE | %0011 | if comparison/subtraction was greater than or equal (C = 0) |
| IF_LT | %1100 | if comparison/subtraction was less than (C = 1) |
| IF_LE | %1110 | if comparison/subtraction was less than or equal (C = 1 or Z = 1) |
| IF_C | %1100 | if C set (C = 1) |
| IF_NC | %0011 | if C clear (C = 0) |
| IF_Z | %1010 | if Z set (Z = 1) |
| IF_NZ | %0101 | if Z clear (Z = 0) |
| IF_C_EQ_Z | %1001 | if C equal to Z (C = 0 and Z = 0 --or-- C = 1 and Z = 1) |
| IF_C_NE_Z | %0110 | if C not equal to Z (C = 0 and Z = 1 --or-- C = 1 and Z = 0) |
| IF_C_AND_Z | %1000 | if C set and Z set (C = 1 and Z = 1) |
| IF_C_AND_NZ | %0100 | if C set and Z clear (C = 1 and Z = 0) |
| IF_NC_AND_Z | %0010 | if C clear and Z set (C = 0 and Z = 1) |
| IF_NC_AND_NZ | %0001 | if C clear and Z clear (C = 0 and Z = 0) |
| IF_C_OR_Z | %1110 | if C set or Z set (C = 1 or Z = 1) |
| IF_C_OR_NZ | %1101 | if C set or Z clear (C = 1 or Z = 0) |
| IF_NC_OR_Z | %1011 | if C clear or Z set (C = 0 or Z = 1) |
| IF_NC_OR_NZ | %0111 | if C clear or Z clear (C = 1 or Z = 0) |
| IF_Z_EQ_C | %1001 | if Z equal to C (Z = 0 and C = 0 --or-- Z = 1 and C = 1) |
| IF_Z_NE_C | %0110 | if Z not equal to C (Z = 0 and C = 1 --or-- Z = 1 and C = 0) |
| IF_Z_AND_C | %1000 | if Z set and C set (Z = 1 and C = 1) |
| IF_Z_AND_NC | %0010 | if Z set and C clear (Z = 1 and C = 0) |
| IF_NZ_AND_C | %0100 | if Z clear and C set (Z = 0 and C = 1) |
| IF_NZ_AND_NC | %0001 | if Z clear and C clear (Z = 0 and C = 0) |
| IF_Z_OR_C | %1110 | if Z set or C set (Z = 1 or C = 1) |
| IF_Z_OR_NC | %1011 | if Z set or C clear (Z = 1 or C = 0) |
| IF_NZ_OR_C | %1101 | if Z clear or C set (Z = 0 or C = 1) |
| IF_NZ_OR_NC | %0111 | if Z clear or C clear (Z = 0 or C = 0) |
| IF_00 | %0001 | if C clear and Z clear (C = 0 and Z = 0) |
| IF_01 | %0010 | if C clear and Z set (C = 0 and Z = 1) |
| IF_10 | %0100 | if C set and Z clear (C = 1 and Z = 0) |
| IF_11 | %1000 | if C set and Z set (C = 1 and Z = 1) |
| IF_X0 | %0101 | if Z clear (Z = 0) |
| IF_X1 | %1010 | if Z set (Z = 1) |
| IF_0X | %0011 | if C clear (C = 0) |
| IF_1X | %1100 | if C set (C = 1) |
| IF_NOT_00 | %1110 | if C clear and Z clear (C = 0 and Z = 0) |
| IF_NOT_01 | %1101 | if C set or Z clear (C = 1 or Z = 0) |
| IF_NOT_10 | %1011 | if C clear or Z set (C = 0 or Z = 1) |
| IF_NOT_11 | %0111 | if C clear or Z clear (C = 0 or Z = 0) |
| IF_DIFF | %0110 | if C not equal to Z (C = 0 and Z = 1 --or-- C = 1 and Z = 0) |
| IF_SAME | %1001 | if C equal to Z (C = 0 and Z = 0 --or-- C = 1 and Z = 1) |
| _RET_ | %0000 | always; execute instruction then return if no branch; no context restore |
|  | %1111 | always; this is the default, no condition expressed |

1 The Condition symbol must be entered in place of the {Condition} item (ahead of the Instruction) on an instruction line.

2 The Encoding value is emitted into the COND field (the EEEE bits) in the compiled instruction opcode.

Note that for every Condition that acts upon the state of C and/or Z flags, there's also a Modifier (for use with MODxx instructions) capable of applying flag state(s) based on similar logic.   Additionally, the Condition symbols and descriptions are similar to, and the encoding exactly matches, that of the related Modifiers.

## DECMOD

Decrement modulus

Math Instruction - Decrement with modulus.

DECMOD Dest, {#}Src {WC|WZ|WCZ}

Result: If Dest was not equal to 0, it is decremented by 1; otherwise Dest is reset to Src.  Optionally, C and Z are updated to indicate reset and zero result status.

- Dest is a register containing the value to decrement down to 0 with modulus, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is the modulus limit to apply to Dest's decrement operation.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0111001 | CZI | DDDDDDDDD | SSSSSSSSS | D | Modulus triggered | Result = 0 | 2 |

Related: INCMOD

Explanation:

DECMOD compares Dest with 0— if not equal, it decrements Dest; otherwise it sets Dest equal to Src.  If Dest begins in the range 0 to Src, iterations of DECMOD will decrement Dest repetitively from Src to 0.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest was equal to 0 and subsequently reset to Src; or is cleared (0) if not reset.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

DECMOD does not limit Dest within the specified range— if Dest begins as greater than Src, iterations of DECMOD will continue to decrement it down through Src before it will effectively cycle from Src to 0.

## DECOD

Decode

Bit Operation Instruction - Decode value (0—31) into single-high-bit long.

DECOD Dest, {#}Src

DECOD Dest

Result: A 32-bit value, with the bit position corresponding to Src, or Dest, value (0—31) set high, is stored in Dest.

- Dest is the register in which to store the decoded value and optionally begins by containing the 5-bit bit position value it is requesting (syntax 2).

- Src is an optional register or 5-bit literal whose value is the bit position to set high in the decoded value.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001110 | 00I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1001110 | 000 | DDDDDDDDD | DDDDDDDDD | D | — | — | 2 |

Related: ENCOD

Explanation:

DECOD generates a 32-bit value with just one bit high, corresponding to the Src, or Dest, value (0—31) and stores that result in Dest.

In effect, Dest becomes %1 << value via the DECOD instruction; where value is Src[4:0] or Dest[4:0].

- A value of 0 generates %00000000_00000000_00000000_00000001.

- A value of 5 generates %00000000_00000000_00000000_00100000.

- A value of 15 generates %00000000_00000000_10000000_00000000.

DECOD is the complement of ENCOD.

## DIRC / DIRNC

Direction C or not C

I/O Pin Instruction - Set pin(s) direction to input/output according to C or !C.

DIRC	{#}Dest	{WCZ}

DIRNC	{#}Dest	{WCZ}

Result: The I/O pin direction bit(s), described by Dest, are set to output/input according to C or !C; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to output or input.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000010 | DIRx | Orig DIRx base bit | Orig DIRx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000011 | DIRx | Orig DIRx base bit | Orig DIRx base bit | 2 |

Related: DIRZ, DIRNZ, DIRL, DIRH, DIRNOT, and DIRRND

Explanation:

DIRC or DIRNC alters the direction register's bit(s) designated by Dest to equal the state, or inverse state, of the C flag; i.e. set pin to the output (1) or input (0) direction.  All other bits (pins) are left unchanged.  Each of these instructions, DIRC and DIRNC, can affect one or more of the bits within the DIRA or DIRB registers.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DIRC / DIRNC instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DIRC / DIRNC's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of DIRA / DIRB's base bit, identified by Dest.

## DIRH / DIRL

Direction high or low

I/O Pin Instruction - Set pin(s) direction to output (high; 1) or input (low; 0).

DIRH {#}Dest {WCZ}

DIRL {#}Dest {WCZ}

Result: The I/O pin direction bit(s), described by Dest, are set high (1; output) or low (0; input); the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to output or input.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000001 | DIRx | Orig DIRx base bit | Orig DIRx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000000 | DIRx | Orig DIRx base bit | Orig DIRx base bit | 2 |

Related: DIRC, DIRNC, DIRZ, DIRNZ, DIRNOT, and DIRRND

Explanation:

DIRH or DIRL alters the direction register's bit(s) designated by Dest to be high (1) or low (0); i.e. set to the output or input direction.  All other bits (pins) are left unchanged.  Each of these instructions, DIRH and DIRL, can affect one or more of the bits within the DIRA or DIRB registers.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DIRH / DIRL instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DIRH / DIRL's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of DIRA / DIRB's base bit, identified by Dest.

## DIRNOT

Direction not

I/O Pin Instruction - Toggle pin(s) to the opposite direction.

DIRNOT {#}Dest {WCZ}

Result: The I/O pin direction bit(s), described by Dest, are toggled to their opposite state(s); the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to toggle to the opposite direction.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000111 | DIRx | Orig DIRx base bit | Orig DIRx base bit | 2 |

Related: DIRRND, DIRL, DIRH, DIRC, DIRNC, DIRZ, and DIRNZ

Explanation:

DIRNOT alters the direction register's bit(s) designated by Dest to their inverse state.  All other bits are left unchanged.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DIRNOT instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DIRNOT's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of DIRA / DIRB's base bit, identified by Dest.

## DIRRND

Direction random

I/O Pin Instruction - Set pin(s) direction to random input/output.

DIRRND {#}Dest {WCZ}

Result: The I/O pin direction bit(s), described by Dest,  are each set randomly low or high (input or output); the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the pins set randomly to inputs or outputs.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000110 | DIRx | Orig DIRx base bit | Orig DIRx base bit | 2 |

Related: DIRNOT, DIRL, DIRH, DIRC, DIRNC, DIRZ, and DIRNZ

Explanation:

DIRRND alters the direction register's bit(s) designated by Dest to be random low and high (input and output), based on bit(s) from the Xoroshiro128** PRNG.  All other bits are left unchanged.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DIRRND instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DIRRND's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of DIRA / DIRB's base bit, identified by Dest.

## DIRZ / DIRNZ

Direction Z or not Z

I/O Pin Instruction - Set pin(s) direction to input/output according to Z or !Z.

DIRZ	{#}Dest	{WCZ}

DIRNZ	{#}Dest	{WCZ}

Result: The I/O pin direction bit(s), described by Dest, are set to output/input according to Z or !Z; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to output or input.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZI | DDDDDDDDD | 001000100 | DIRx | Orig DIRx base bit | Orig DIRx base bit | 2 |
| EEEE | 1101011 | CZI | DDDDDDDDD | 001000101 | DIRx | Orig DIRx base bit | Orig DIRx base bit | 2 |

Related: DIRC, DIRNC, DIRNOT, DIRRND, DIRL, and DIRH

Explanation:

DIRZ or DIRNZ alters the direction register's bit(s) designated by Dest to equal the state, or inverse state, of the Z flag; i.e. set pin to the output (1) or input (0) direction.  All other bits (pins) are left unchanged.  Each of these instructions, DIRZ and DIRNZ, can affect one or more of the bits within the DIRA or DIRB registers.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DIRZ / DIRNZ instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DIRZ / DIRNZ's use.

If the WCZ effect is specified, the C and Z flags are updated to the original state of DIRA / DIRB's base bit, identified by Dest.

## DJF / DJNF

Decrement, jump if full or not full

Flow Control Instruction - Decrement value and jump if full (-1; $FFFF_FFFF) or not full (<> -1; <> $FFFF_FFFF).

DJF	Dest, {#}Src

DJNF	Dest, {#}Src

Result: Dest is decremented, and if the result is full (or not full in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Dest is a register whose value is decremented and tested for full or not full.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011011 | 10I | DDDDDDDDD | SSSSSSSSS | D and PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011011 | 11I | DDDDDDDDD | SSSSSSSSS | D and PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 Dest is always written; PC is written only when the result in Dest is full (or not full in syntax 2).

Explanation:

DJF or DJNF decrements the value in Dest, writes the result, and jumps to the address described by Src if the result is full (-1; $FFFF_FFFF; in syntax 1) or not full (<> -1; <> $FFFF_FFFF; in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the DJF / DJNF.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## DJZ / DJNZ

Decrement, jump if zero or not zero

Flow Control Instruction - Decrement, jump if zero or not zero.

DJZ	Dest, {#}Src

DJNZ	Dest, {#}Src

Result: Dest is decremented, and if the result is zero (or not zero in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Dest is a register whose value is decremented and tested for zero or not zero.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011011 | 00I | DDDDDDDDD | SSSSSSSSS | D and PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011011 | 01I | DDDDDDDDD | SSSSSSSSS | D and PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 Dest is always written; PC is written only when the result in Dest is zero (or not zero in syntax 2).

Explanation:

DJZ or DJNZ decrements the value in Dest, writes the result, and jumps to the address described by Src if the result is zero (in syntax 1) or not zero (in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the DJZ / DJNZ.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## DRVC / DRVNC

Drive C or not C

I/O Pin Instruction - Set pin(s) direction to output and output level to low/high according to C or !C.

DRVC	{#}Dest	{WCZ}

DRVNC	{#}Dest	{WCZ}

Result: The I/O pins described by Dest are set to the output direction and to an output level of low/high according to C or !C; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to output direction and output levels of low or high.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001011010 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001011011 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a DRVC or DRVNC.

Explanation:

DRVC or DRVNC sets the I/O pin(s) designated by Dest to the output direction and to a low/high output level according to the state, or inverse state, of the C flag; i.e. alters the pin's direction and output registers.  All other pins are left unchanged.  Each of these instructions, DRVC and DRVNC, can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

DRVC or DRVNC achieves the same effect as two instructions— OUTC, or OUTNC, followed by DIRH.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DRVC / DRVNC instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DRVC / DRVNC's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## DRVH / DRVL

Drive high or low

I/O Pin Instruction - Set pin(s) direction to output and output level high (1) or low (0).

DRVH {#}Dest {WCZ}

DRVL {#}Dest {WCZ}

Result: The I/O pins described by Dest are set to the output direction and to an output level of high or low; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to output direction and output levels of high or low.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001011001 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001011000 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a DRVH or DRVL.

Explanation:

DRVH or DRVL sets the I/O pin(s) designated by Dest to the output direction and to a high or low output level; i.e. alters the pin's direction and output registers.  All other pins are left unchanged.  Each of these instructions, DRVH and DRVL, can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

DRVH or DRVL achieves the same effect as two instructions— OUTH, or OUTL, followed by DIRH.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DRVH / DRVL instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DRVH / DRVL's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## DRVNOT

Drive not

I/O Pin Instruction - Set pin(s) direction to output and toggle to the opposite output level.

DRVNOT {#}Dest {WCZ}

Result: The I/O pins described by Dest are set to the output direction and to their opposite output level(s); the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to the output direction and toggle to opposite output levels.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001011111 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a DRVNOT.

Explanation:

DRVNOT sets the I/O pin(s) designated by Dest to the output direction and to their opposite output level(s); i.e. alters the pin's direction and output registers.  All other pins are left unchanged.  This instruction can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

DRVNOT achieves the same effect as two instructions— OUTNOT followed by DIRH.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DRVNOT instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DRVNOT's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## DRVRND

Drive random

I/O Pin Instruction - Set pin(s) direction to output and output level to random low/high.

DRVRND {#}Dest {WCZ}

Result: The I/O pins described by Dest are set to the output direction and each output level is set randomly low or high; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to the output direction and with output level(s) set randomly to low or high.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001011110 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a DRVRND.

Explanation:

DRVRND sets the I/O pin(s) designated by Dest to the output direction and with output level(s) set randomly low and high, based on bit(s) from the Xoroshiro128** PRNG.  All other pins are left unchanged.  This instruction can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

DRVRND achieves the same effect as two instructions— OUTRND followed by DIRH.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DRVRND instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DRVRND's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## DRVZ / DRVNZ

Drive Z or not Z

I/O Pin Instruction - Set pin(s) direction to output and output level to low/high according to Z or !Z.

DRVZ	{#}Dest	{WCZ}

DRVNZ	{#}Dest	{WCZ}

Result: The I/O pins described by Dest are set to the output direction and to an output level of low/high according to Z or !Z; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to output direction and output levels of low or high.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001011100 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001011101 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a DRVZ or DRVNZ.

Explanation:

DRVZ or DRVNZ sets the I/O pin(s) designated by Dest to the output direction and to a low/high output level according to the state, or inverse state, of the Z flag; i.e. alters the pin's direction and output registers.  All other pins are left unchanged.  Each of these instructions, DRVZ and DRVNZ, can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

DRVZ or DRVNZ achieves the same effect as two instructions— OUTZ, or OUTNZ, followed by DIRH.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the DRVZ / DRVNZ instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for DRVZ / DRVNZ's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## ENCOD

Encode

Bit Operation Instruction - Get bit position of top-most 1 of Src or Dest into Dest.

ENCOD Dest, {#}Src	{WC|WZ|WCZ}

ENCOD Dest	{WC|WZ|WCZ}

Result: The bit position value of the top-most high bit (1) in Src, or Dest, is stored in Dest.

- Dest is the register in which to store the encoded bit position value and optionally begins by containing the 32-bit value it is encoding (syntax 2).

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose value is to be encoded into a bit position.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0111100 | CZI | DDDDDDDDD | SSSSSSSSS | D | S != 0 | Result = 0 | 2 |
| EEEE | 0111100 | CZ0 | DDDDDDDDD | DDDDDDDDD | D | Original D != 0 | Result = 0 | 2 |

Related: DECOD

Explanation:

ENCOD stores the bit position value (0—31) of the top-most high bit (1) of Src, or Dest, into Dest.  If the value to encode (Src, or original Dest (in syntax 2)) may be %0, the resulting Dest will be 0— use the WC or WCZ effect and check the resulting C flag to distinguish between the cases of input Src/Dest = %1 verses input Src/Dest = %0.

If the WC or WCZ effect is specified, the C flag is set (1) if Src (or original Dest in syntax 2) was not zero, or is cleared (0) if it was zero.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result equals zero, or is cleared (0) if not zero.

- A long of %00000000_00000000_00000000_00000001 encodes to 0.

- A long of %00000000_00000000_00000000_00100000 encodes to 5.

- A long of %00000000_00000000_10000001_01000000 encodes to 15.

- A long of %00000000_00000000_00000000_00000000 encodes to 0 with optional C cleared to 0.

ENCOD is the complement of DECOD.

## Effects ( WC, WZ, WCZ, ANDC, etc. )

Nearly half of PASM2 instructions feature optional effects to modify the C and/or Z flags.  An Effect is placed at the end of such instructions.

{Label}  {Condition}  Instruction  Operands  {Effect}

When included (where allowed) the flag or flags are updated by the instruction execution; when omitted, the flags remain as-is.  Only zero or one Effect is allowed per instruction.

| ###### Effects |  |
| --- | --- |
| Effect | Description |
| ANDC | AND tested bit/pin into current C; used on TESTxx instructions |
| ANDZ | AND tested bit/pin into current Z; used on TESTxx instructions |
| ORC | OR tested bit/pin into current C; used on TESTxx instructions |
| ORZ | OR tested bit/pin into current Z; used on TESTxx instructions |
| XORC | XOR tested bit/pin into current C; used on TESTxx instructions |
| XORZ | XOR tested bit/pin into current Z; used on TESTxx instructions |
| WC | Write C flag; used on many instructions |
| WCZ | Write both C and Z flags; used on many instructions |
| WZ | Write Z flag; used on many instructions |

For example:

AND   config, #%1000  WZ	'config[3] low? z=1

TESTP #4              ANDZ	'pin 4 high? z=1

if_z        JMP   #MoreCode	'jump if config[3] low and pin 4 high

This code jumps to the label MoreCode only if config bit 3 is low (0) and I/O pin 4 is high.  This first instruction bitwise AND's %1000 into config and sets the Z flag if that result is zero.  The second instruction tests I/O pin 4 and bitwise AND's its high/low state into the Z flag.  The third instruction, which specifies an if_z condition, executes only if the z flag is high by then, jumping to MoreCode; otherwise it behaves like a nop instruction.  Using an Effect on instructions, along with a Condition on later instructions, enables code to be much more powerful than what is possible with typical assembly languages. See IF_x (Conditions) for more information.  NOTE: In this example, the first instruction modifies config— to do non-destructive bit tests, use a testb instead of and instruction.

## FGE

Force greater or equal

Math Instruction - Force unsigned value to be greater than or equal to another.

FGE Dest, {#}Src {WC|WZ|WCZ}

Result: Unsigned Dest is set to unsigned Src if Dest was less than Src.  Optionally the C and Z flag indicates if the replacement happened and the zero status of the result.

- Dest is a register containing the unsigned value to limit to a minimum of unsigned Src, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose unsigned value is the lower limit to force upon Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0011000 | CZI | DDDDDDDDD | SSSSSSSSS | D | limit enforced | Result = 0 | 2 |

Related: FLE, FGES, and FLES

Explanation:

FGE sets unsigned Dest to unsigned Src if Dest is less than Src.  This is also known as a limit minimum function; preventing Dest from sinking below Src.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest was limited (Dest was less than Src and now Dest is equal to Src), or is cleared (0) if not limited.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## FGES

Force greater or equal, signed

Math Instruction - Force signed value to be greater than or equal to another.

FGES Dest, {#}Src {WC|WZ|WCZ}

Result: Signed Dest is set to signed Src if Dest was less than Src.  Optionally the C and Z flag indicates if the replacement happened and the zero status of the result.

- Dest is a register containing the signed value to limit to a minimum of signed Src, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose signed value is the lower limit to force upon Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0011010 | CZI | DDDDDDDDD | SSSSSSSSS | D | limit enforced | Result = 0 | 2 |

Related: FLES, FGE, and FLE

Explanation:

FGES sets signed Dest to signed Src if Dest is less than Src.  This is also known as a limit minimum function; preventing Dest from sinking below Src.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest was limited (Dest was less than Src and now Dest is equal to Src), or is cleared (0) if not limited.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## FLE

Force lesser or equal

Math Instruction - Force unsigned value to be less than or equal to another.

FLE Dest, {#}Src {WC|WZ|WCZ}

Result: Unsigned Dest is set to unsigned Src if Dest was greater than Src.  Optionally the C and Z flag indicates if the replacement happened and the zero status of the result.

- Dest is a register containing the unsigned value to limit to a maximum of unsigned Src, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose unsigned value is the upper limit to force upon Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0011001 | CZI | DDDDDDDDD | SSSSSSSSS | D | limit enforced | Result = 0 | 2 |

Related: FGE, FLES, and FGES

Explanation:

FLE sets unsigned Dest to unsigned Src if Dest is greater than Src.  This is also known as a limit maximum function; preventing Dest from rising above Src.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest was limited (Dest was greater than Src and now Dest is equal to Src), or is cleared (0) if not limited.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## FLES

Force lesser or equal, signed

Math Instruction - Force signed value to be less than or equal to another.

FLES Dest, {#}Src {WC|WZ|WCZ}

Result: Signed Dest is set to signed Src if Dest was greater than Src.  Optionally the C and Z flag indicates if the replacement happened and the zero status of the result.

- Dest is a register containing the signed value to limit to a maximum of signed Src, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose signed value is the upper limit to force upon Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0011011 | CZI | DDDDDDDDD | SSSSSSSSS | D | limit enforced | Result = 0 | 2 |

Related: FGES, FLE, and FGE

Explanation:

FLES sets signed Dest to signed Src if Dest is greater than Src.  This is also known as a limit maximum function; preventing Dest from rising above Src.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest was limited (Dest was greater than Src and now Dest is equal to Src), or is cleared (0) if not limited.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## FLTC / FLTNC

Float C or not C

I/O Pin Instruction - Set pin(s) direction to input and an output level of low/high according to C.

FLTC	{#}Dest	{WCZ}

FLTNC	{#}Dest	{WCZ}

Result: The I/O pins described by Dest are set to the input direction and to an output level of low/high according to C or !C; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to input direction and output levels of low or high.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001010010 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001010011 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a FLTC or FLTNC.

Explanation:

FLTC or FLTNC sets the I/O pin(s) designated by Dest to the input direction and to a low/high output level according to the state, or inverse state, of the C flag; i.e. alters the pin's direction and output registers.  All other pins are left unchanged.  Each of these instructions, FLTC and FLTNC, can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

FLTC or FLTNC achieves the same effect as two instructions— DIRL followed by OUTC or OUTNC.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the FLTC / FLTNC instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for FLTC / FLTNC's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## FLTH / FLTL

Float high or low

I/O Pin Instruction - Set pin(s) direction to input and to an output level of high (1) or low (0).

FLTH {#}Dest {WCZ}

FLTL {#}Dest {WCZ}

Result: The I/O pins described by Dest are set to the input direction and to an output level of high or low; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to input direction and output levels of high or low.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001010001 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001010000 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a FLTH or FLTL.

Explanation:

FLTH or FLTL sets the I/O pin(s) designated by Dest to the input direction and to a high or low output level; i.e. alters the pin's direction and output registers.  All other pins are left unchanged.  Each of these instructions, FLTH and FLTL, can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

FLTH or FLTL achieves the same effect as two instructions— DIRL followed by OUTH or OUTL.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the FLTH / FLTL instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for FLTH / FLTL's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## FLTNOT

Float not

I/O Pin Instruction - Set pin(s) direction to input and toggle to the opposite output level.

FLTNOT {#}Dest {WCZ}

Result: The I/O pins described by Dest are set to the input direction and to their opposite output level(s); the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to the input direction and toggle to opposite output levels.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001010111 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a FLTNOT.

Explanation:

FLTNOT sets the I/O pin(s) designated by Dest to the input direction and to their opposite output level(s); i.e. alters the pin's direction and output registers.  All other pins are left unchanged.  This instruction can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

FLTNOT achieves the same effect as two instructions— DIRL followed by OUTNOT.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the FLTNOT instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for FLTNOT's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## FLTRND

Float random

I/O Pin Instruction - Set pin(s) direction to input and to an output level of random low/high.

FLTRND {#}Dest {WCZ}

Result: The I/O pins described by Dest are set to the input direction and each output level is set randomly low or high; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to the input direction and with output level(s) set randomly to low or high.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001010110 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a FLTRND.

Explanation:

FLTRND sets the I/O pin(s) designated by Dest to the input direction and with output level(s) set randomly low and high, based on bit(s) from the Xoroshiro128** PRNG.  All other pins are left unchanged.  This instruction can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

FLTRND achieves the same effect as two instructions— DIRL followed by OUTRND.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the FLTRND instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for FLTRND's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## FLTZ / FLTNZ

Float Z or not Z

I/O Pin Instruction - Set pin(s) direction to input and an output level of low/high according to Z.

FLTZ	{#}Dest	{WCZ}

FLTNZ	{#}Dest	{WCZ}

Result: The I/O pins described by Dest are set to the input direction and to an output level of low/high according to Z or !Z; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set to input direction and output levels of low or high.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001010100 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001010101 | DIRx1 + OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

1 New DIRx state is not data-forwarded; the next pipelined instruction sees the old state.  Make sure any instruction that reads or modifies DIRx is at least two instructions after a FLTZ or FLTNZ.

Explanation:

FLTZ or FLTNZ sets the I/O pin(s) designated by Dest to the input direction and to a low/high output level according to the state, or inverse state, of the Z flag; i.e. alters the pin's direction and output registers.  All other pins are left unchanged.  Each of these instructions, FLTZ and FLTNZ, can affect one or more of the bits within the DIRA or DIRB and OUTA or OUTB registers.

FLTZ or FLTNZ achieves the same effect as two instructions— DIRL followed by OUTZ or OUTNZ.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the FLTZ / FLTNZ instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for FLTZ / FLTNZ's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (DIRA or DIRB and OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## GETBYTE

Get byte

Bit Operation Instruction - Get a byte from a value.

GETBYTE Dest, {#}Src, #Num

GETBYTE Dest

Result: Byte Num (0–3) of Src, or a byte from a source described by prior ALTGB instruction, is written to Dest.

- Dest is the register in which to store the byte.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value contains the target byte to read.

- Num is a 2-bit literal identifying the byte ID (0–3) of Src to read.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1000111 | NNI | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1000111 | 000 | DDDDDDDDD | 000000000 | D | — | — | 2 |

Related: ALTGB, GETNIB, GETWORD, SETNIB, SETBYTE, SETWORD, ROLNIB, ROLBYTE, and ROLWORD

Explanation:

GETBYTE reads the byte identified by Num (0–3) from Src, or a byte from the source described by a prior ALTGB instruction.

Num (0–3) identifies a value's individual bytes, by position, in least-significant byte order.

Syntax 2 is intended for use after an ALTGB instruction; i.e. in a loop to iteratively read a series of byte values within contiguous long registers.

## GETNIB

Get nibble

Bit Operation Instruction - Get a nibble from a value.

GETNIB Dest, {#}Src, #Num

GETNIB Dest

Result: Nibble Num (0–7) of Src, or a nibble from a source described by prior ALTGN instruction, is written to Dest.

- Dest is the register in which to store the nibble.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value contains the target nibble to read.

- Num is a 3-bit literal identifying the nibble ID (0–7) of Src to read.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 100001N | NNI | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1000010 | 000 | DDDDDDDDD | 000000000 | D | — | — | 2 |

Related: ALTGN, GETBYTE, GETWORD, SETNIB, SETBYTE, SETWORD, ROLNIB, ROLBYTE, and ROLWORD

Explanation:

GETNIB reads the nibble identified by Num (0–7) from Src, or a nibble from the source described by a prior ALTGN instruction.

Num (0–7) identifies a value's individual nibbles, by position, in least-significant nibble order.

Syntax 2 is intended for use after an ALTGN instruction; i.e. in a loop to iteratively read a series of nibble values within contiguous long registers.

## GETWORD

Get word

Bit Operation Instruction - Get a word from a value.

GETWORD Dest, {#}Src, #Num

GETWORD Dest

Result: Word Num (0–1) of Src, or a word from a source described by prior ALTGW instruction, is written to Dest.

- Dest is the register in which to store the word.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value contains the target word to read.

- Num is a 1-bit literal identifying the word ID (0–1) of Src to read.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001001 | 1NI | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1001001 | 100 | DDDDDDDDD | 000000000 | D | — | — | 2 |

Related: ALTGW, GETNIB, GETBYTE, SETNIB, SETBYTE, SETWORD, ROLNIB, ROLBYTE, and ROLWORD

Explanation:

GETWORD reads the word identified by Num (0–1) from Src, or a word from the source described by a prior ALTGW instruction.

Num (0–1) identifies a value's words, by position, in least-significant word order.

Syntax 2 is intended for use after an ALTGW instruction; i.e. in a loop to iteratively read a series of word values within contiguous long registers.

## IF_x (Conditions)

See Conditions ( IF_x ).

## IJZ / IJNZ

Increment, jump if zero or not zero

Flow Control Instruction - Increment value and jump if zero or not zero.

IJZ	Dest, {#}Src

IJNZ	Dest, {#}Src

Result: Dest is incremented, and if the result is zero (or not zero in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Dest is a register whose value is incremented and tested for zero or not zero.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011100 | 00I | DDDDDDDDD | SSSSSSSSS | D and PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011100 | 01I | DDDDDDDDD | SSSSSSSSS | D and PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 Dest is always written; PC is written only when the result in Dest is zero (or not zero in syntax 2).

Explanation:

IJZ or IJNZ increments the value in Dest, writes the result, and jumps to the address described by Src if the result is zero (syntax 1) or not zero (in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the IJZ / IJNZ.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## INCMOD

Increment modulus

Math Instruction - Increment with modulus.

INCMOD Dest, {#}Src {WC|WZ|WCZ}

Result: If Dest was not equal to Src, it is incremented by 1; otherwise Dest is reset to 0.  Optionally, C and Z are updated to indicate reset and zero result status.

- Dest is a register containing the value to increment up to Src with modulus, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is the modulus limit to apply to Dest's increment operation.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0111000 | CZI | DDDDDDDDD | SSSSSSSSS | D | Modulus triggered | Result = 0 | 2 |

Related: DECMOD

Explanation:

INCMOD compares Dest with Src— if not equal, it increments Dest; otherwise it sets Dest equal to 0.  If Dest begins in the range 0 to Src, iterations of INCMOD will increment Dest repetitively from 0 to Src.

If the WC or WCZ effect is specified, the C flag is set (1) if Dest was equal to Src and subsequently reset to 0; or is cleared (0) if not reset.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

INCMOD does not limit Dest within the specified range— if Dest begins as greater than Src, iterations of INCMOD will continue to increment it through the 32-bit rollover point (back to 0) before it will effectively cycle from 0 to Src.

## JATN / JNATN

Jump if attention or not attention

Event Branch Instruction - Jump if attention flag is set or clear.

JATN	{#}Src

JNATN	{#}Src

Result: If ATN event flag is set (or is clear in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000001110 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000011110 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the ATN event flag is set (or is clear in syntax 2).

Explanation:

JATN or JNATN checks the cog's attention signal and jumps to the address described by Src if attention is set (in syntax 1) or is clear (in syntax 2).  The cog's attention signal, when set, indicates that one or more other cogs are requesting this cog's attention.  See the Hardware Manual's Cog Attention section for more information.

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JATN / JNATN.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## JCT1/2/3 / JNCT1/2/3

Jump if counter 1/2/3 or not counter 1/2/3

Event Branch Instruction - Jump if counter 1, 2, or 3 event flag is set or clear.

JCT1	{#}Src

JCT2	{#}Src

JCT3	{#}Src

JNCT1	{#}Src

JNCT2	{#}Src

JNCT3	{#}Src

Result: If counter 1, 2, or 3 event flag is set (or is clear in syntax 4–6), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000000001 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000000010 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000000011 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000010001 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000010010 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000010001 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the counter event flag is set (or is clear in syntax 4–6).

Explanation:

JCT1, JCT2, JCT3, or JNCT1, JNCT2, JNCT3 checks the cog's counter 1, 2, or 3 event flag and jumps to the address described by Src if the flag is set (in syntax 1–3) or is clear (in syntax 4–6).  The cog's hidden registers, CT1, CT2, and CT3 are dedicated to System Counter timing and events— when a counter event flag is set, it means a specified time period has elapsed.  See the Hardware Manual's System Counter section for more information.

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JCTx / JNCTx.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

Related instructions are ADDCTx, POLLCTx, and WAITCTx.

## JFBW / JNFBW

Jump if FIFO block wrap or not FIFO block wrap

Event Branch Instruction - Jump if FIFO interface block wrap event flag is set or clear.

JFBW	{#}Src

JNFBW	{#}Src

Result: If FIFO interface block wrap event flag is set (or is clear in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000001001 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000011001 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the FIFO interface block wrap event flag is set (or is clear in syntax 2).

Explanation:

JFBW or JNFBW checks the cog's FIFO interface block wrap flag and jumps to the address described by Src if the flag is set (in syntax 1) or is clear (in syntax 2).  The cog's Fast Sequential FIFO Interface is used to swiftly transfer data between Hub and Cog.  When the FIFO runs out of data (block count), it sets this flag before wrapping around to the start of the block again.

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JFBW / JNFBW.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## JINT / JNINT

Jump if interrupt or not interrupt

Event Branch Instruction - Jump if interrupt-occurred event flag is set or clear.

JINT	{#}Src

JNINT	{#}Src

Result: If interrupt-occurred event flag is set (or is clear in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000000000 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000010000 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the interrupt event flag is set (or is clear in syntax 2).

Explanation:

JINT or JNINT checks the cog's interrupt-occurred flag and jumps to the address described by Src if the flag is set (in syntax 1) or is clear (in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JINT / JNINT.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## JPAT / JNPAT

Jump if pattern or not pattern

Event Branch Instruction - Jump if pin pattern event flag set or clear.

JPAT	{#}Src

JNPAT	{#}Src

Result: If pin pattern event flag is set (or is clear in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000001000 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000011000 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the pin pattern event flag is set (or is clear in syntax 2).

Explanation:

JPAT or JNPAT checks the cog's pin-pattern-detected event flag and jumps to the address described by Src if the flag is set (in syntax 1) or is clear (in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JPAT / JNPAT.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## JQMT / JNQMT

Jump if CORDIC empty or not CORDIC empty

Event Branch Instruction - Jump if CORDIC-read-but-empty event flag set or clear.

JQMT	{#}Src

JNQMT	{#}Src

Result: If CORDIC-read-but-empty event flag is set (or is clear in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000001111 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000011111 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the CORDIC-read-but-empty event flag is set (or is clear in syntax 2).

Explanation:

JQMT or JNQMT checks the cog's CORDIC results event flag and jumps to the address described by Src if the flag is set (in syntax 1) or is clear (in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JQMT / JNQMT.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## JSE1/2/3/4 / JNSE1/2/3/4

Jump if selectable event 1/2/3/4 or not selectable event 1/2/3/4

Event Branch Instruction - Jump if selectable event 1, 2, 3, or 4 flag is set or clear.

JSE1	{#}Src

JSE2	{#}Src

JSE3	{#}Src

JSE4	{#}Src

JNSE1	{#}Src

JNSE2	{#}Src

JNSE3	{#}Src

JNSE4	{#}Src

Result: If selectable event 1, 2, 3, or 4 flag is set (or is clear in syntax 5–8), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000000100 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000000101 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000000110 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000000111 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000010100 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000010101 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000010110 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000010111 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the selectable event 1, 2, 3, or 4 flag is set (or is clear in syntax 5–8).

Explanation:

JSE1, JSE2, JSE3, JSE4, or JNSE1, JNSE2, JNSE3, JNSE4 checks the cog's selectable event 1, 2, 3, or 4 event flag and jumps to the address described by Src if the flag is set (in syntax 1–4) or is clear (in syntax 5–8).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JSEx / JNSEx.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## JXFI / JNXFI

Jump if streamer finished or not streamer finished

Event Branch Instruction - Jump if streamer finished event flag set or clear.

JXFI	{#}Src

JNXFI	{#}Src

Result: If streamer finished event flag is set (or is clear in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000001011 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000011011 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the streamer finished event flag is set (or is clear in syntax 2).

Explanation:

JXFI or JNXFI checks the cog's streamer finished event flag and jumps to the address described by Src if the flag is set (in syntax 1) or is clear (in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JXFI / JNXFI.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## JXMT / JNXMT

Jump if streamer empty or not streamer empty

Event Branch Instruction - Jump if streamer empty event flag set or clear.

JXMT	{#}Src

JNXMT	{#}Src

Result: If streamer empty event flag is set (or is clear in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000001010 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000011010 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the streamer empty event flag is set (or is clear in syntax 2).

Explanation:

JXMT or JNXMT checks the cog's streamer empty event flag and jumps to the address described by Src if the flag is set (in syntax 1) or is clear (in syntax 2).  The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JXMT / JNXMT.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## JXRL / JNXRL

Jump if streamer rollover LUT or not streamer rollover LUT

Event Branch Instruction - Jump if streamer LUT RAM rollover event flag set or clear.

JXRL	{#}Src

JNXRL	{#}Src

Result: If streamer LUT RAM rollover event flag is set (or is clear in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000001101 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000011101 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the streamer LUT RAM rollover event flag is set (or is clear in syntax 2).

Explanation:

JXRL or JNXRL checks the cog's streamer LUT RAM rollover event flag and jumps to the address described by Src if the flag is set (in syntax 1) or is clear (in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JXRL / JNXRL.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## JXRO / JNXRO

Jump if streamer rollover NCO or not streamer rollover NCO

Event Branch Instruction - Jump if streamer NCO rollover event flag set or clear.

JXRO	{#}Src

JNXRO	{#}Src

Result: If streamer NCO rollover event flag is set (or is clear in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 01I | 000001100 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011110 | 01I | 000011100 | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when the streamer NCO rollover event flag is set (or is clear in syntax 2).

Explanation:

JXRO or JNXRO checks the cog's streamer NCO rollover event flag and jumps to the address described by Src if the flag is set (in syntax 1) or is clear (in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the JXRO / JNXRO.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## MODC / MODZ / MODCZ

Modify C, Z, or C and Z

Flag Modification Instruction - Modify C and/or Z flag(s) according to modifier mode and current state(s).

MODC	CModifier {WC}

MODZ	ZModifier {WZ}

MODCZ	CModifier, ZModifier {WC|WZ|WCZ}

Result: The C and/or Z flag is set or cleared according to the given Modifier and the current state of the C and/or Z flags.

- CModifer is a Modifier symbol for the designated mode to apply to the C flag.

- ZModifer is a Modifier symbol for the designated mode to apply to the Z flag.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | C01 | 0cccc0000 | 001101111 | — | C=cccc[{C,Z}] | — | 2 |
| EEEE | 1101011 | 0Z1 | 00000zzzz | 001101111 | — | — | Z=zzzz[{C,Z}] | 2 |
| EEEE | 1101011 | CZ1 | 0cccczzzz | 001101111 | — | C=cccc[{C,Z}] | Z=zzzz[{C,Z}] | 2 |

Explanation:

MODC, MODZ, or MODCZ sets or clears the C and/or Z flag based on the mode described by the given Modifier symbol(s) and the current state of the C and/or Z flag.  The WC, WZ, and WCZ effects are required to affect the designated flag.

These flag modifier instructions allow code to preset flags to a desired state which may be required for entry into certain code routines, or to set a special state based on multiple events that are otherwise not possible to realize with a single instruction.

If the WC, WZ, or WCZ effect is specified, the C, Z, or both C and Z flags are updated according to the given CModifier or ZModifier.  See the Modifier Symbols table for details.

| ###### Modifier Symbols (for MODC / MODZ / MODCZ) |  |  |
| --- | --- | --- |
| Modifier1 | Encoding2 | Description |
| _CLR | %0000 | Clear C/Z (C == 0 and/or Z == 0) |
| _E | %1010 | Set C/Z if comparison/subtraction was equal (C == Z and/or Z == Z) |
| _NE | %0101 | Set C/Z if comparison/subtraction was not equal (C == !Z and/or Z == !Z) |
| _GT | %0001 | Set C/Z if comparison/subtraction was greater than (C == !C AND !Z and/or Z == !C AND !Z) |
| _GE | %0011 | Set C/Z if comparison/subtraction was greater than or equal (C == !C and/or Z == !C) |
| _LT | %1100 | Set C/Z if comparison/subtraction was less than (C == C and/or Z == C) |
| _LE | %1110 | Set C/Z if comparison/subtraction was less than or equal (C == C OR Z and/or Z == C OR Z) |
| _C | %1100 | Set C/Z to C (C == C and/or Z == C) |
| _NC | %0011 | Set C/Z to inverse of C (C == !C and/or Z == !C) |
| _Z | %1010 | Set C/Z to Z (C == Z and/or Z == Z) |
| _NZ | %0101 | Set C/Z to inverse of Z (C == !Z and/or Z == !Z) |
| _C_EQ_Z | %1001 | Set C/Z if C equal to Z (C == C = Z and/or Z == C = Z) |
| _C_NE_Z | %0110 | Set C/Z if C not equal to Z (C == C <> Z and/or Z == C <> Z) |
| _C_AND_Z | %1000 | Set C/Z to C AND Z (C == C AND Z and/or Z == C AND Z) |
| _C_AND_NZ | %0100 | Set C/Z to C AND NOT Z (C == C AND !Z and/or Z == C AND !Z) |
| _NC_AND_Z | %0010 | Set C/Z to NOT C AND Z (C == !C AND Z and/or Z == !C AND Z) |
| _NC_AND_NZ | %0001 | Set C/Z to NOT C AND NOT Z (C == !C AND !Z and/or Z == !C AND !Z) |
| _C_OR_Z | %1110 | Set C/Z to C OR Z (C == C OR Z and/or Z == C OR Z) |
| _C_OR_NZ | %1101 | Set C/Z to C OR NOT Z (C == C OR !Z and/or Z == C OR !Z) |
| _NC_OR_Z | %1011 | Set C/Z to NOT C OR Z (C == !C OR Z and/or Z == !C OR Z) |
| _NC_OR_NZ | %0111 | Set C/Z to NOT C OR NOT Z (C == !C OR !Z and/or Z == !C OR !Z) |
| _Z_EQ_C | %1001 | Set C/Z if Z equal to C (C == Z = C and/or Z == Z = C) |
| _Z_NE_C | %0110 | Set C/Z if Z not equal to C (C == Z <> C and/or Z == Z <> C) |
| _Z_AND_C | %1000 | Set C/Z to Z AND C (C == Z AND C and/or Z == Z AND C) |
| _Z_AND_NC | %0010 | Set C/Z to Z AND NOT C (C == Z AND !C and/or Z == Z AND !C) |
| _NZ_AND_C | %0100 | Set C/Z to NOT Z AND C (C == !Z AND C and/or Z == !Z AND C) |
| _NZ_AND_NC | %0001 | Set C/Z to NOT Z AND NOT C (C == !Z AND !C and/or Z == !Z AND !C) |
| _Z_OR_C | %1110 | Set C/Z to Z OR C (C == Z OR C and/or Z == Z OR C) |
| _Z_OR_NC | %1011 | Set C/Z to Z OR NOT C (C == Z OR !C and/or Z == Z OR !C) |
| _NZ_OR_C | %1101 | Set C/Z to NOT Z OR C (C == !Z OR C and/or Z == !Z OR C) |
| _NZ_OR_NC | %0111 | Set C/Z to NOT Z OR NOT C (C == !Z OR !C and/or Z == !Z OR !C) |
| _SET | %1111 | Set C/Z (C == 1 and/or Z == 1) |

1	Use Modifier symbol(s) in MODC, MODZ, and MODCZ instructions.  Note that the symbol and description is similar to, and the encoding exactly matches, that of the related Conditions used to include/exclude instructions at run time.

2	The encoding is the 4-bit value placed into the MODC, MODZ, and MODCZ instruction's cccc or zzzz opcode field by the compiler.

Examples:

MODCZ   _NZ, 0          WC	'C = !Z,  Z = Z (unaltered)

MODCZ   0, _SET         WZ	'C = C (unaltered),  Z = 1

MODCZ   _CLR, _Z_OR_C   WCZ	'C = 0,  Z |= C

MODC    _NZ_AND_C       WC	'C = !Z & C

MODZ    _Z_NE_C         WZ	'Z = Z ^ C;  (Z <> C)

## MOV

Move

Bit Operation Instruction - Set a value into a register.

MOV Dest, {#}Src {WC|WZ|WCZ}

Result: The Src value is stored in Dest and optionally flags are updated with the sign bit and zero status.

- Dest is the register to receive the Src value.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value will be stored into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0110000 | CZI | DDDDDDDDD | SSSSSSSSS | D | S[31] | Result = 0 | 2 |

Explanation:

Mov stores the Src value into Dest.

If the WC or WCZ effect is specified, the C flag is updated to be Src[31].

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest result equals zero, or is cleared (0) if it is non-zero.

## MUL

Multiply

Math Instruction - Multiply unsigned 16-bit x 16-bit values.

MUL Dest, {#}Src {WZ}

Result: The 32-bit unsigned product of the 16-bit Dest and Src multiplication is stored into Dest and optionally the Z flag is updated to the Dest or Src zero status.

- Dest is a register containing the 16-bit value to multiply with Src, and is where the result is written.

- Src is a register, 9-bit literal, or 16-bit augmented literal whose value is multiplied into Dest.

- WZ is an optional effect to update the Z flag.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1010000 | 0ZI | DDDDDDDDD | SSSSSSSSS | D | — | (D = 0) ¦ (S = 0) | 2 |

Related: MULS, SCA, and QMUL

Explanation:

MUL multiplies the lower 16-bits of each of Dest and Src together and stores the 32-bit product result into the Dest register.  This is a fast (2-clock) 16 x 16 bit multiplication operation— to multiply larger factors, use the CORDIC Solver QMUL instruction.

If the WZ effect is specified, the Z flag is set (1) if either the Dest or Src values are zero, or is cleared (0) if both are non-zero.

## MULS

Multiply, signed

Math Instruction - Multiply signed 16-bit x 16-bit values.

MULS Dest, {#}Src {WZ}

Result: The 32-bit signed product of the signed 16-bit Dest and Src multiplication is stored into Dest and optionally the Z flag is updated to the Dest or Src zero status.

- Dest is a register containing the signed 16-bit value to multiply with Src, and is where the result is written.

- Src is a register, 9-bit literal, or signed 16-bit augmented literal whose value is multiplied into Dest.

- WZ is an optional effect to update the Z flag.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1010000 | 1ZI | DDDDDDDDD | SSSSSSSSS | D | — | (D = 0) ¦ (S = 0) | 2 |

Related: MUL, SCA, and QMUL

Explanation:

MULS multiplies the signed lower 16-bits of each of Dest and Src together and stores the 32-bit signed product result into the Dest register.  This is a fast (2-clock) signed 16 x 16 bit multiplication operation— to multiply larger factors, use the CORDIC Solver QMUL instruction.

If the WZ effect is specified, the Z flag is set (1) if either the Dest or Src values are zero, or is cleared (0) if both are non-zero.

## MUXC / MUXNC

Mux C or mux not C

Bit Operation Instructions - Set discrete bits to C or !C

MUXC	Dest, {#}Src	{WC|WZ|WCZ}

MUXNC	Dest, {#}Src	{WC|WZ|WCZ}

Result: Dest bit(s) described by Src are set to C or !C; the rest are left as-is.  Flags are optionally updated with parity and zero status of the result.

- Dest is the register whose value will have one or more bits set to C or !C.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value identifies the bit(s) to modify.

- WC, WZ, or WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0101100 | CZI | DDDDDDDDD | SSSSSSSSS | D | Parity of Result | Result = 0 | 2 |
| EEEE | 0101101 | CZI | DDDDDDDDD | SSSSSSSSS | D | Parity of Result | Result = 0 | 2 |

Explanation:

MUXC or MUXNC alters the Dest bit(s) designated by Src (high bits) to equal the state, or inverse state, of the C flag.  All Dest bits corresponding to high (1) bits in Src are modified; all other Dest bits are left unchanged.

If the WC or WCZ effect is specified, the C flag is set (1) if the number of high (1) bits in the result is odd, or is cleared (0) if it is even.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is not zero.

## MUXNIBS

Mux nibbles

Bit Operation Instruction - Set discrete nibbles of a value to non-zero nibble states of another.

MUXNIBS Dest, {#}Src

Result: Dest nibbles corresponding to non-zero Src nibbles are set to those nibble values; the rest are left as-is.

- Dest is a register whose value will be updated according to Src.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose non-zero nibbles will replace the corresponding nibbles in Dest.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001111 | 01I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |

Explanation:

MUXNIBS copies any non-zero nibbles from Src into the corresponding nibbles of Dest and leaves the rest of Dest's nibbles as-is.

## MUXNITS

Mux nits

Bit Operation Instruction - Set discrete bit pairs of a value to non-zero bit pair states of another.

MUXNITS Dest, {#}Src

Result: Dest bit pairs corresponding to non-zero Src bit pairs are set to those bit values; the rest are left as-is.

- Dest is a register whose value will be updated according to Src.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose non-zero bit pairs will replace the corresponding bit pairs in Dest.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001111 | 00I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |

Explanation:

MUXNITS copies any non-zero bit pairs from Src into the corresponding bit pairs of Dest and leaves the rest of Dest's bit pairs as-is.

## MUXQ

Mux Q

Bit Operation Instruction - Set discrete bits of a value to that of another.

MUXQ Dest, {#}Src

Result: Dest bits described by Q are set to corresponding Src bits; the rest are left as-is.

- Dest is a register whose bits will be updated according to Q and Src.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose select bits will replace the corresponding bits in Dest.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001111 | 10I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |

Explanation:

MUXQ copies all bits from Src corresponding to high (1) bits of Q into the corresponding bits of Dest.  All other Dest bits are left as-is.

MUXQ must be preceded by SETQ to function properly since the Q value's high bits identify the bits to target in Src and Dest.

## MUXZ / MUXNZ

Mux Z or mux not Z

Bit Operation Instructions - Set discrete bits to Z or !Z

MUXZ	Dest, {#}Src	{WC|WZ|WCZ}

MUXNZ	Dest, {#}Src	{WC|WZ|WCZ}

Result: Dest bit(s) described by Src are set to Z or !Z; the rest are left as-is.  Flags are optionally updated with parity and zero status of the result.

- Dest is the register whose value will have one or more bits set to Z or !Z.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value identifies the bit(s) to modify.

- WC, WZ, or WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0101110 | CZI | DDDDDDDDD | SSSSSSSSS | D | Parity of Result | Result = 0 | 2 |
| EEEE | 0101111 | CZI | DDDDDDDDD | SSSSSSSSS | D | Parity of Result | Result = 0 | 2 |

Explanation:

MUXZ or MUXNZ alters the Dest bit(s) designated by Src (high bits) to equal the state, or inverse state, of the Z flag.  All Dest bits corresponding to high (1) bits in Src are modified; all other Dest bits are left unchanged.

If the WC or WCZ effect is specified, the C flag is set (1) if the number of high (1) bits in the result is odd, or is cleared (0) if it is even.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is not zero.

## NEG

Negate

Math Instruction - Negate a value.

NEG Dest, {#}Src	{WC|WZ|WCZ}

NEG Dest	{WC|WZ|WCZ}

Result: The Src or Dest value is negated and stored into Dest and optionally the C and Z flags are updated to the resulting sign and zero status.

- Dest is a register to receive the -Src value (syntax 1), or contains the value to negate (syntax 2).

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose negated value is stored into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0110011 | CZI | DDDDDDDDD | SSSSSSSSS | D | Sign of result | Result = 0 | 2 |
| EEEE | 0110011 | CZ0 | DDDDDDDDD | DDDDDDDDD | D | Sign of result | Result = 0 | 2 |

Related: ABS, NEGC, NEGNC, NEGZ and NEGNZ

Explanation:

NEG negates Src (syntax 1) or Dest (syntax 2) and stores the result in the Dest register.  The negation flips the value's sign; ex: 78 becomes -78, or -306 becomes 306.

If the WC or WCZ effect is specified, the C flag is set (1) if the result is negative, or is cleared (0) if positive.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## NEGC

Negate C

Math Instruction - Negate value according to C.

NEGC Dest, {#}Src	{WC|WZ|WCZ}

NEGC Dest	{WC|WZ|WCZ}

Result: The Src or Dest value, possibly negated according to C, is stored into Dest and optionally the C and Z flags are updated to the resulting sign and zero status.

- Dest is a register to receive the Src or -Src value (syntax 1), or contains the value to negate (syntax 2) according to C.

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose value (if C=0) or negated value (if C=1) is stored into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0110100 | CZI | DDDDDDDDD | SSSSSSSSS | D | Sign of result | Result = 0 | 2 |
| EEEE | 0110100 | CZ0 | DDDDDDDDD | DDDDDDDDD | D | Sign of result | Result = 0 | 2 |

Related: NEGNC, NEGZ and NEGNZ

Explanation:

NEGC negates Src (syntax 1) or Dest (syntax 2) if C = 1 and stores the result in the Dest register.  If C = 0, the Src or Dest value is left as-is (not negated) and is stored into Dest.  If the negation is performed, it flips the value's sign; ex: 5 becomes -5, or -200 becomes 200.

If the WC or WCZ effect is specified, the C flag is set (1) if the result is negative, or is cleared (0) if positive.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## NEGNC

Negate not C

Math Instruction - Negate value according to !C.

NEGNC Dest, {#}Src	{WC|WZ|WCZ}

NEGNC Dest	{WC|WZ|WCZ}

Result: The Src or Dest value, possibly negated according to !C, is stored into Dest and optionally the C and Z flags are updated to the resulting sign and zero status.

- Dest is a register to receive the Src or -Src value (syntax 1), or contains the value to negate (syntax 2) according to !C.

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose value (if !C=0) or negated value (if !C=1) is stored into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0110101 | CZI | DDDDDDDDD | SSSSSSSSS | D | Sign of result | Result = 0 | 2 |
| EEEE | 0110101 | CZ0 | DDDDDDDDD | DDDDDDDDD | D | Sign of result | Result = 0 | 2 |

Related: NEGC, NEGZ and NEGNZ

Explanation:

NEGNC negates Src (syntax 1) or Dest (syntax 2) if !C = 1 and stores the result in the Dest register.  If !C = 0, the Src or Dest value is left as-is (not negated) and is stored into Dest.  If the negation is performed, it flips the value's sign; ex: 21 becomes -21, or -1,374 becomes 1,374.

If the WC or WCZ effect is specified, the C flag is set (1) if the result is negative, or is cleared (0) if positive.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## NEGNZ

Negate not Z

Math Instruction - Negate value according to !Z.

NEGNZ Dest, {#}Src	{WC|WZ|WCZ}

NEGNZ Dest	{WC|WZ|WCZ}

Result: The Src or Dest value, possibly negated according to !Z, is stored into Dest and optionally the C and Z flags are updated to the resulting sign and zero status.

- Dest is a register to receive the Src or -Src value (syntax 1), or contains the value to negate (syntax 2) according to !C.

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose value (if !C=0) or negated value (if !C=1) is stored into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0110111 | CZI | DDDDDDDDD | SSSSSSSSS | D | Sign of result | Result = 0 | 2 |
| EEEE | 0110111 | CZ0 | DDDDDDDDD | DDDDDDDDD | D | Sign of result | Result = 0 | 2 |

Related: NEGZ, NEGC and NEGNC

Explanation:

NEGNZ negates Src (syntax 1) or Dest (syntax 2) if !Z = 1 and stores the result in the Dest register.  If !Z = 0, the Src or Dest value is left as-is (not negated) and is stored into Dest.  If the negation is performed, it flips the value's sign; ex: 193 becomes -193, or -3,062 becomes 3,062.

If the WC or WCZ effect is specified, the C flag is set (1) if the result is negative, or is cleared (0) if positive.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## NEGZ

Negate Z

Math Instruction - Negate value according to Z.

NEGZ Dest, {#}Src	{WC|WZ|WCZ}

NEGZ Dest	{WC|WZ|WCZ}

Result: The Src or Dest value, possibly negated according to Z, is stored into Dest and optionally the C and Z flags are updated to the resulting sign and zero status.

- Dest is a register to receive the Src or -Src value (syntax 1), or contains the value to negate (syntax 2) according to Z.

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose value (if Z=0) or negated value (if Z=1) is stored into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0110110 | CZI | DDDDDDDDD | SSSSSSSSS | D | Sign of result | Result = 0 | 2 |
| EEEE | 0110110 | CZ0 | DDDDDDDDD | DDDDDDDDD | D | Sign of result | Result = 0 | 2 |

Related: NEGNZ, NEGC and NEGNC

Explanation:

NEGZ negates Src (syntax 1) or Dest (syntax 2) if Z = 1 and stores the result in the Dest register.  If Z = 0, the Src or Dest value is left as-is (not negated) and is stored into Dest.  If the negation is performed, it flips the value's sign; ex: 526 becomes -526, or -41 becomes 41.

If the WC or WCZ effect is specified, the C flag is set (1) if the result is negative, or is cleared (0) if positive.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## NOP

No operation

Miscellaneous Instruction - No operation, just elapse two cycles.

NOP

Result: Two clock cycles are consumed.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 | 0000000 | 000 | 000000000 | 000000000 | — | — | — | 2 |

Explanation:

NOP simply consumes two clock cycles; no other operation is performed.

## NOT

Not

Bit Operation Instruction - Bitwise NOT a value.

NOT Dest, {#}Src	{WC|WZ|WCZ}

NOT Dest	{WC|WZ|WCZ}

Result: !Src (or !Dest) is stored in Dest and flags are optionally updated with the NOTed high bit and zero status.

- Dest is the register containing the value to bitwise NOT (syntax 2) or to be replaced by the bitwise NOT of Src (syntax 1).

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose value will be bitwise NOTed and stored into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0110001 | CZI | DDDDDDDDD | SSSSSSSSS | D | !S[31] | Result = 0 | 2 |
| EEEE | 0110001 | CZ0 | DDDDDDDDD | DDDDDDDDD | D | !D[31] | Result = 0 | 2 |

Explanation:

NOT performs a bitwise NOT (inverting all bits) of the value in Src (or in Dest) and stores the result into Dest.

If the WC or WCZ effect is specified, the C flag value is replaced by the inverse of either S[31] (syntax 1) or D[31] (syntax 2).

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result of !Src (syntax 1) or !Dest (syntax 2) equals zero, or is cleared (0) if it is non-zero.

## OR

Or

Bit Operation Instruction - Bitwise OR a value with another.

OR Dest, {#}Src {WC|WZ|WCZ}

Result: Dest OR Src is stored in Dest and flags are optionally updated with parity and zero status.

- Dest is the register containing the value to bitwise OR with Src and is the destination in which to write the result.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value will be bitwise ORed into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0101010 | CZI | DDDDDDDDD | SSSSSSSSS | D | Parity of Result | Result = 0 | 2 |

Explanation:

OR performs a bitwise OR of the value in Src into that of Dest.

If the WC or WCZ effect is specified, the C flag is set (1) if the result contains an odd number of high (1) bits, or is cleared (0) if it contains an even number of high bits.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest OR Src result equals zero, or is cleared (0) if it is non-zero.

## ONES

Ones

Bit Operation Instruction - Get number of 1s from Dest or Src into Dest.

ONES Dest, {#}Src	{WC|WZ|WCZ}

ONES Dest	{WC|WZ|WCZ}

Result: The number of high bits in Src, or Dest, is stored in Dest.

- Dest is the register in which to store the number of high bits found and optionally it begins by containing the value to check (syntax 2).

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose value is to be checked for ones.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0111101 | CZI | DDDDDDDDD | SSSSSSSSS | D | Result is odd | Result = 0 | 2 |
| EEEE | 0111101 | CZ0 | DDDDDDDDD | DDDDDDDDD | D | Result is odd | Result = 0 | 2 |

Explanation:

ONES tallies the number of high bits of Src, or Dest, and stores the count into Dest.

If the WC or WCZ effect is specified, the C flag is set (1) if the count is odd, or is cleared (0) if it is even.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result equals zero, or is cleared (0) if not zero.

## OUTC / OUTNC

Output C or not C

I/O Pin Instruction - Set pin(s) output level to low/high according to C or !C.

OUTC	{#}Dest	{WCZ}

OUTNC	{#}Dest	{WCZ}

Result: The I/O pin output level bit(s), described by Dest, are set to low/high according to C or !C; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) for which output levels are to be set low or high.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001001010 | OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001001011 | OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

Explanation:

OUTC or OUTNC alters the output level register's bit(s) designated by Dest to equal the state, or inverse state, of the C flag; i.e. set pin's output level low or high.  All other bits (pins) are left unchanged.  Each of these instructions, OUTC and OUTNC, can affect one or more of the bits within the OUTA or OUTB registers.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the OUTC / OUTNC instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for OUTC / OUTNC's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## OUTH / OUTL

Output high or low

I/O Pin Instruction - Set pin(s) output level to high (1) or low (0).

OUTH {#}Dest {WCZ}

OUTL {#}Dest {WCZ}

Result: The I/O pin output level bit(s), described by Dest, are set high (1) or low (0); the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to set high or low.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001001001 | OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001001000 | OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

Explanation:

OUTH or OUTL alters the output level register's bit(s) designated by Dest to be high (1) or low (0).  All other bits (pins) are left unchanged.  Each of these instructions, OUTH and OUTL, can affect one or more of the bits within the OUTA or OUTB registers.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the OUTH / OUTL instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for OUTH / OUTL's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## OUTNOT

Output not

I/O Pin Instruction - Toggle pin(s) to the opposite output level.

OUTNOT {#}Dest {WCZ}

Result: The I/O pin output level bit(s), described by Dest, are toggled to their opposite state(s); the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) to toggle to the opposite output level.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001001111 | OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

Explanation:

OUTNOT alters the output level register's bit(s) designated by Dest to their inverse state.  All other bits are left unchanged.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the OUTNOT instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for OUTNOT's use.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## OUTRND

Output random

I/O Pin Instruction - Set pin(s) output level to random low/high.

OUTRND {#}Dest {WCZ}

Result: The I/O pin output level bit(s), described by Dest,  are each set randomly low or high; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the pins set randomly to low or high.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001001110 | OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

Explanation:

OUTRND alters the output level register's bit(s) designated by Dest to be random low and high, based on bit(s) from the Xoroshiro128** PRNG.  All other bits are left unchanged.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the OUTRND instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for OUTRND's use.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## OUTZ / OUTNZ

Output Z or not Z

I/O Pin Instruction - Set pin(s) output level to low/high according to Z or !Z.

OUTZ	{#}Dest	{WCZ}

OUTNZ	{#}Dest	{WCZ}

Result: The I/O pin output level bit(s), described by Dest, are set to low/high according to Z or !Z; the rest are left as-is.

- Dest is the register, 9-bit literal, or 11-bit augmented literal whose value identifies the I/O pin(s) for which output levels are to be set low or high.

- WCZ is an optional effect to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001001100 | OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001001101 | OUTx | Orig OUTx base bit | Orig OUTx base bit | 2 |

Explanation:

OUTZ or OUTNZ alters the output level register's bit(s) designated by Dest to equal the state, or inverse state, of the Z flag; i.e. set pin's output level low or high.  All other bits (pins) are left unchanged.  Each of these instructions, OUTZ and OUTNZ, can affect one or more of the bits within the OUTA or OUTB registers.

Dest[5:0] indicates the pin number (0–63).  For a range of pins, Dest[5:0] indicates the base pin number (0–63) and Dest[10:6] indicates how many contiguous pins beyond the base should be affected (1–31).

A 9-bit literal Dest is enough to express the base pin (Dest[5:0]) and a range of up to 8 contiguous pins (Dest[8:6]).  If needed, use the augmented literal feature (##Dest) to augment Dest to an 11-bit literal value— this inserts an AUGD instruction prior.

When Dest is a register, the register's value bits [10:0] are used as-is to form the 11-bit ID range, unless a SETQ instruction immediately precedes the OUTZ / OUTNZ instruction; substituting SETQ's Dest[4:0] in place of value bits[10:6], for OUTZ / OUTNZ's use.

The range calculation (from Dest[5:0] up to Dest[5:0]+Dest[10:6]) will wrap within the same 32-pin group (OUTA or OUTB); it will not cross the port boundary.

If the WCZ effect is specified, the C and Z flags are updated to the original state of OUTA / OUTB's base bit, identified by Dest.

## POLLATN

Poll attention

Event Monitor Instruction - Retrieve and clear attention flag.

POLLATN {WC|WZ|WCZ}

Result: Attention event flag is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000001110 | 000100100 | — | ATN Event | ATN Event | 2 |

Related: COGATN, WAITATN, JATN, and JNATN

Explanation:

POLLATN copies the state of the attention event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the attention event flag prior to clearing it.

The attention event flag is set whenever another cog issues an attention request for this cog.  The attention event flag is cleared upon cog start, or execution of POLLATN, WAITATN, JATN, or JNATN instructions.

## POLLCT1/2/3

Poll counter 1/2/3

Event Monitor Instruction - Retrieve and clear counter event 1/2/3 flag.

POLLCT1 {WC|WZ|WCZ}

POLLCT2 {WC|WZ|WCZ}

POLLCT3 {WC|WZ|WCZ}

Result: Counter event flag 1, 2, or 3 state is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000000001 | 000100100 | — | CT1 Event | CT1 Event | 2 |
| EEEE | 1101011 | CZ0 | 000000010 | 000100100 | — | CT2 Event | CT2 Event | 2 |
| EEEE | 1101011 | CZ0 | 000000011 | 000100100 | — | CT3 Event | CT3 Event | 2 |

Related: ADDCTx, WAITCTx, JCTx, and JNCTx

Explanation:

POLLCT1, POLLCT2, or POLLCT3 copies the state of the count 1, 2, or 3 event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the corresponding counter event flag prior to clearing it.

The counter 1, 2, or 3 event flag is set whenever the System Counter (CT) passes the value in the CT1, CT2, or CT3 event trigger register, respectively; i.e. MSB of (CT - CTx is 0).  The counter event flags are cleared upon execution of the corresponding ADDCTx, POLLCTx, WAITCTx, JCTx, or JNCTx instructions.

## POLLFBW

Poll FIFO block wrap

Event Monitor Instruction - Retrieve and clear FIFO-interface-block-wrap event flag.

POLLFBW {WC|WZ|WCZ}

Result: FIFO-interface-block-wrap event flag is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000001001 | 000100100 | — | FBW Event | FBW Event | 2 |

Related: RDFAST, WRFAST, FBLOCK, WAITFBW, JFBW, and JNFBW

Explanation:

POLLFBW copies the state of the FIFO-interface-block-wrap event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the FIFO-interface‑block‑ wrap event flag prior to clearing it.

The FIFO-interface-block-wrap event flag is set whenever the Hub RAM FIFO interface exhausts its block count and reloads its block count and start address.  The FIFO-interface-block-wrap event flag is cleared upon execution of RDFAST, WRFAST, FBLOCK, POLLFBW, WAITFBW, JFBW, or JNFBW instructions.

## POLLINT

Poll interrupt

Event Monitor Instruction - Retrieve and clear interrupt-occurred event flag.

POLLINT {WC|WZ|WCZ}

Result: Interrupt-occurred event flag is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000000000 | 000100100 | — | INT Event | INT Event | 2 |

Related: WAITINT, JINT, and JNINT

Explanation:

POLLINT copies the state of the interrupt-occurred event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the interrupt-occurred event flag prior to clearing it.

The interrupt-occurred event flag is set whenever interrupt 1, 2, or 3 occurs— debug interrupts are ignored.  The interrupt-occurred event flag is cleared upon cog start, or execution of POLLINT, WAITINT, JINT, or JNINT instructions.

## POLLPAT

Poll pattern

Event Monitor Instruction - Retrieve and clear pin-pattern-detected event flag.

POLLPAT {WC|WZ|WCZ}

Result: Pin-pattern-detected event flag is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000001000 | 000100100 | — | PAT Event | PAT Event | 2 |

Related: SETPAT, WAITPAT, JPAT, and JNPAT

Explanation:

POLLPAT copies the state of the pin-pattern-detected event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the pin-pattern-detected event flag prior to clearing it.

The pin-pattern-detected event flag is set whenever the masked input pins match or don't match the pattern described by a previous SETPAT instruction.  The pin-pattern-detected event flag is cleared upon execution of SETPAT, POLLPAT, WAITPAT, JPAT, or JNPAT instructions.

## POLLQMT

Poll CORDIC empty

Event Monitor Instruction - Retrieve and clear CORDIC-read-but-empty event flag.

POLLQMT {WC|WZ|WCZ}

Result: CORDIC-read-but-empty event flag is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000001111 | 000100100 | — | QMT Event | QMT Event | 2 |

Related: JQMT and JNQMT

Explanation:

POLLQMT copies the state of the CORDIC-read-but-empty event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the CORDIC-read-but- empty event flag prior to clearing it.

The CORDIC-read-but-empty event flag is set whenever GETQX / GETQY executes without any CORDIC results available or in progress.  The pin-pattern-detected event flag is cleared upon cog start or execution of POLLQMT, WAITQMT, JQMT, or JNQMT instructions.

## POLLSE1/2/3/4

Poll selectable event 1/2/3/4

Event Monitor Instruction - Retrieve and clear selectable event 1, 2, 3, or 4 flag.

POLLSE1 {WC|WZ|WCZ}

POLLSE2 {WC|WZ|WCZ}

POLLSE3 {WC|WZ|WCZ}

POLLSE4 {WC|WZ|WCZ}

Result: Selectable event flag 1, 2, 3, or 4 state is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000000100 | 000100100 | — | SE1 Event | SE1 Event | 2 |
| EEEE | 1101011 | CZ0 | 000000101 | 000100100 | — | SE2 Event | SE2 Event | 2 |
| EEEE | 1101011 | CZ0 | 000000110 | 000100100 | — | SE3 Event | SE3 Event | 2 |
| EEEE | 1101011 | CZ0 | 000000111 | 000100100 | — | SE4 Event | SE4 Event | 2 |

Related: SETSEx, WAITSEx, JSEx, and JNSEx

Explanation:

POLLSE1, POLLSE2, POLLSE3, or POLLSE4 copies the state of the selectable event 1, 2, 3, or 4 flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the corresponding selectable event flag prior to clearing it.

The selectable event 1, 2, 3, or 4 flag is set whenever the corresponding configured event occurs.  The selectable event flag is cleared upon execution of the corresponding SETSEx, POLLSEx, WAITSEx, JSEx, or JNSEx instructions.

## POLLXFI

Poll streamer finished

Event Monitor Instruction - Retrieve and clear streamer-finished event flag.

POLLXFI {WC|WZ|WCZ}

Result: Streamer-finished event flag is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000001011 | 000100100 | — | XFI Event | XFI Event | 2 |

Related: XINIT, XZERO, XCONT, WAITXFI, JXFI, and JNXFI

Explanation:

POLLXFI copies the state of the streamer-finished event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the streamer-finished event flag prior to clearing it.

The streamer-finished event flag is set whenever the streamer runs out of commands to process.  The streamer-finished event flag is cleared upon execution of XINIT, XZERO, XCONT, POLLXFI, WAITXFI, JXFI, or JNXFI instructions.

## POLLXMT

Poll streamer empty

Event Monitor Instruction - Retrieve and clear streamer-empty event flag.

POLLXMT {WC|WZ|WCZ}

Result: Streamer-empty event flag is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000001010 | 000100100 | — | XMT Event | XMT Event | 2 |

Related: XINIT, XZERO, XCONT, WAITXMT, JXMT, and JNXMT

Explanation:

POLLXMT copies the state of the streamer-empty event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the streamer-empty event flag prior to clearing it.

The streamer-empty event flag is set whenever the streamer is ready for a new command.  The streamer-empty event flag is cleared upon execution of XINIT, XZERO, XCONT, POLLXMT, WAITXMT, JXMT, or JNXMT instructions.

## POLLXRL

Poll streamer rollover LUT

Event Monitor Instruction - Retrieve and clear streamer-LUT-RAM-rollover event flag.

POLLXRL {WC|WZ|WCZ}

Result: Streamer-LUT-RAM-rollover event flag is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000001101 | 000100100 | — | XRL Event | XRLEvent | 2 |

Related: XINIT, XZERO, XCONT, WAITXRL, JXRL, and JNXRL

Explanation:

POLLXRL copies the state of the streamer-LUT-RAM-rollover event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the streamer-LUT-RAM- rollover event flag prior to clearing it.

The streamer-LUT-RAM-rollover event flag is set whenever location $1FF of the Lookup RAM is read by the streamer.  The streamer-LUT-RAM-rollover event flag is cleared upon cog start or upon execution of POLLXRL, WAITXRL, JXRL, or JNXRL instructions.

## POLLXRO

Poll streamer rollover NCO

Event Monitor Instruction - Retrieve and clear streamer-NCO-rollover event flag.

POLLXRO {WC|WZ|WCZ}

Result: Streamer-NCO-rollover event flag is optionally copied into C and/or Z, then it is cleared.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000001100 | 000100100 | — | XRO Event | XRO Event | 2 |

Related: XINIT, XZERO, XCONT, WAITXRO, JXRO, and JNXRO

Explanation:

POLLXRO copies the state of the streamer NCO rollover event flag into C and/or Z and then clears the flag (unless it's being set again by the event sensor).

If the WC, WZ, or WCZ effect is specified, the C flag and/or Z flag is updated to the state of the streamer-NCO- rollover event flag prior to clearing it.

The streamer-NCO-rollover event flag is set whenever the streamer's numerically-controlled oscillator (NCO) rolls over.  The streamer-NCO-rollover event flag is cleared upon execution of XINIT, XZERO, XCONT, POLLXRO, WAITXRO, JXRO, or JNXRO instructions.

## RCL

Rotate carry left

Bit Operation Instruction - Rotate carry flag left into value.

RCL Dest, {#}Src {WC|WZ|WCZ}

Result: The bits of Dest are shifted left by Src bits, inserting C as new LSBs.

- Dest is the register containing the value to rotate carry left by Src bits.

- Src is a register or 5-bit literal whose value indicates the number of bit positions to rotate.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0000101 | CZI | DDDDDDDDD | SSSSSSSSS | D | Last bit out1 | Result = 0 | 2 |

1 C = last bit shifted out if Src[4:0] > 0, else C = Dest[31].

Explanation:

RCL shifts Dest's binary value left by Src places (0–31 bits) and sets the new LSBs to C.

If the WC or WCZ effect is specified, the C flag is updated to the value of the last bit shifted out if Src is 1–31, or to Dest[31] if Src is 0.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest result equals zero, or is cleared (0) if it is non-zero.

## RCR

Rotate carry right

Bit Operation Instruction - Rotate carry flag right into value.

RCR Dest, {#}Src {WC|WZ|WCZ}

Result: The bits of Dest are shifted right by Src bits, inserting C as new MSBs.

- Dest is the register containing the value to rotate carry right by Src bits.

- Src is a register or 5-bit literal whose value indicates the number of bit positions to rotate.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0000100 | CZI | DDDDDDDDD | SSSSSSSSS | D | Last bit out1 | Result = 0 | 2 |

1 C = last bit shifted out if Src[4:0] > 0, else C = Dest[0].

Explanation:

RCR shifts Dest's binary value right by Src places (0–31 bits) and sets the new MSBs to C.

If the WC or WCZ effect is specified, the C flag is updated to the value of the last bit shifted out if Src is 1–31, or to Dest[0] if Src is 0.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest result equals zero, or is cleared (0) if it is non-zero.

## RCZL

Rotate carry and zero left

Bit Operation Instruction - Rotate carry and zero flags left into value (2-bit rotate left).

RCZL Dest {WC|WZ|WCZ}

Result: The bits of Dest are shifted left by two places and C and Z are inserted as new LSBs.

- Dest is the register containing the value to rotate the carry and zero flags left into.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | DDDDDDDDD | 001101011 | D | D[31] | D[30] | 2 |

Explanation:

RCZL shifts Dest's binary value left by two places and sets Dest[1] to C and Dest[0] to Z.

If the WC or WCZ effect is specified, the C flag is updated to the original Dest[31] state.

If the WZ or WCZ effect is specified, the Z is flag is updated to the original Dest[30] state.

## RCZR

Rotate carry and zero right

Bit Operation Instruction - Rotate carry and zero flags right into value (2-bit rotate right).

RCZR Dest {WC|WZ|WCZ}

Result: The bits of Dest are shifted right by two places and C and Z are inserted as new MSBs.

- Dest is the register containing the value to rotate the carry and zero flags right into.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | DDDDDDDDD | 001101010 | D | D[1] | D[0] | 2 |

Explanation:

RCZR shifts Dest's binary value right by two places and sets Dest[31] to C and Dest[30] to Z.

If the WC or WCZ effect is specified, the C flag is updated to the original Dest[1] state.

If the WZ or WCZ effect is specified, the Z is flag is updated to the original Dest[0] state.

## Registers

Special Purpose Registers - Rotate a byte left into a value.

Most Reg RAM registers are general-purpose (used for code and data) while a small set has a special purpose.

| Name | Address | Access2 | Special Purpose |
| --- | --- | --- | --- |
| IJMP3 | $1F01 | Read/Write | Interrupt call address for INT3; set by code |
| IRET3 | $1F11 | Read/Write | Interrupt return address and C/Z flags for INT3; set by cog |
| IJMP2 | $1F21 | Read/Write | Interrupt call address for INT2; set by code |
| IRET2 | $1F31 | Read/Write | Interrupt return address and C/Z flags for INT2; set by cog |
| IJMP1 | $1F41 | Read/Write | Interrupt call address for INT1; set by code |
| IRET1 | $1F51 | Read/Write | Interrupt return address and C/Z flags for INT1; set by cog |
| PA | $1F61 | Read/Write | CALLD-imm return, CALLPA parameter, or LOC address |
| PB | $1F71 | Read/Write | CALLD-imm return, CALLPB parameter, or LOC address |
| PTRA | $1F8 | Read Special / Write Special | Pointer A to Hub RAM |
| PTRB | $1F9 | Read Special / Write Special | Pointer B to Hub RAM |
| DIRA | $1FA | Read / Write Special | Output enables (direction bits) for P31..P0 |
| DIRB | $1FB | Read / Write Special | Output enables (direction bits) for P63..P32 |
| OUTA | $1FC | Read / Write Special | Output states for P31..P0 |
| OUTB | $1FD | Read / Write Special | Output states for P63..P32 |
| INA | $1FE3 | Read Special | Input states for P31..P0 |
| INB | $1FF4 | Read Special | Input states for P63..P32 |

1	Locations $1F0–$1F7 are general-purpose code/data registers by default but become these named special-purpose registers if their associated functions are enabled.

2	Most reads and writes involve the underlying Reg RAM at these locations; however, those marked "special" may read from a special internal register (instead of Reg RAM) or write to both the Reg RAM location as well as to a special internal register.

3	Also debug interrupt call address.

4	Also debug interrupt return address.

Additionally, for PASM2 code that is either in-line (within a Spin2 method) or called (by a Spin2 method), the registers $1D8–$1DF are readable/writable by both languages using the symbols PR0–PR7.  PASM2 code that is launched into another cog does not share this Reg RAM space with Spin2.

| Name | Address | Access | Special Purpose |
| --- | --- | --- | --- |
| PR0–PR7 | $1D8–$1DF | Read/Write | PASM2 ⇄ Spin2 communication mechanism |

## REV

Reverse

Bit Operation Instruction - Reverse bits of value.

REV Dest

Result: The 32-bit pattern in Dest is reversed.

- Dest is the register containing the bit value to reverse.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | 000 | DDDDDDDDD | 001101001 | D | — | — | 2 |

Explanation:

REV performs a bitwise reverse (bits 31:0 ⇒ bits 0:31) of the value in Dest and stores the result back into Dest.  This is useful for processing binary data in a different MSB/LSB order than it is transmitted with.

## ROL

Rotate left

Bit Operation Instruction - Rotate bits left.

ROL Dest, {#}Src {WC|WZ|WCZ}

Result: The bits of Dest are rotated left by Src bits; any departing MSBs are moved into LSBs.

- Dest is the register containing the value to rotate left by Src bits.

- Src is a register or 5-bit literal whose value indicates the number of bit positions to rotate.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0000001 | CZI | DDDDDDDDD | SSSSSSSSS | D | Last bit out1 | Result = 0 | 2 |

1 C = last bit rotated out if Src[4:0] > 0, else C = Dest[31].

Explanation:

ROL rotates Dest's binary value left by Src places (0–31 bits).  All MSBs rotated out are moved into the new LSBs.

If the WC or WCZ effect is specified, the C flag is updated to the value of the last bit rotated out (effectively C = result bit "0") if Src is 1–31, or to Dest[31] if Src is 0.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest result equals zero, or is cleared (0) if it is non-zero.  Since no bits are lost by this operation, the result will only be zero if Dest started at zero.

## ROLBYTE

Rotate left byte

Bit Operation Instruction - Rotate a byte left into a value.

ROLBYTE Dest, {#}Src, #Num

ROLBYTE Dest

Result: Byte Num (0–3) of Src, or a byte from a source described by prior ALTGB instruction, is rotated left into Dest.

- Dest is the register in which to rotate the byte into.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value contains the target byte to read.

- Num is a 2-bit literal identifying the nibble ID (0–3) of Src to read.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001000 | NNI | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1001000 | 000 | DDDDDDDDD | 000000000 | D | — | — | 2 |

Related: ALTGB, ROLNIB, ROLWORD, GETNIB, GETBYTE, GETWORD, SETNIB, SETBYTE, and  SETWORD

Explanation:

ROLBYTE reads the byte identified by Num (0–3) from Src, or a byte from the source described by a prior ALTGB instruction, and rotates it left into Dest.

ROLBYTE achieves the same effect as two instructions— an 8-bit SHL followed by SETBYTE into byte 0.

Num (0–3) identifies a value's individual bytes, by position, in least-significant byte order.

Syntax 2 is intended for use after an ALTGB instruction; i.e. in a loop to iteratively read a series of byte values within contiguous long registers.

## ROLNIB

Rotate left nibble

Bit Operation Instruction - Rotate a nibble left into a value.

ROLNIB Dest, {#}Src, #Num

ROLNIB Dest

Result: Nibble Num (0–7) of Src, or a nibble from a source described by prior ALTGN instruction, is rotated left into Dest.

- Dest is the register in which to rotate the nibble into.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value contains the target nibble to read.

- Num is a 3-bit literal identifying the nibble ID (0–7) of Src to read.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 100010N | NNI | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1000100 | 000 | DDDDDDDDD | 000000000 | D | — | — | 2 |

Related: ALTGN, ROLBYTE, ROLWORD, GETNIB, GETBYTE, GETWORD, SETNIB, SETBYTE, and  SETWORD

Explanation:

ROLNIB reads the nibble identified by Num (0–7) from Src, or a nibble from the source described by a prior ALTGN instruction, and rotates it left into Dest.

ROLNIB achieves the same effect as two instructions— a 4-bit SHL followed by SETNIB into nibble 0.

Num (0–7) identifies a value's individual nibbles, by position, in least-significant nibble order.

Syntax 2 is intended for use after an ALTGN instruction; i.e. in a loop to iteratively read a series of nibble values within contiguous long registers.

## ROLWORD

Rotate left word

Bit Operation Instruction - Rotate a word left into a value.

ROLWORD Dest, {#}Src, #Num

ROLWORD Dest

Result: Word Num (0–1) of Src, or a word from a source described by prior ALTGW instruction, is rotated left into Dest.

- Dest is the register in which to rotate the word into.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value contains the target word to read.

- Num is a 1-bit literal identifying the nibble ID (0–1) of Src to read.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001010 | 0NI | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1001010 | 000 | DDDDDDDDD | 000000000 | D | — | — | 2 |

Related: ALTGW, ROLNIB, ROLBYTE, GETNIB, GETBYTE, GETWORD, SETNIB, SETBYTE, and SETWORD

Explanation:

ROLWORD reads the word identified by Num (0–1) from Src, or a word from the source described by a prior ALTGW instruction, and rotates it left into Dest.

ROLWORD achieves the same effect as two instructions— a 16-bit SHL followed by SETWORD into word 0.

Num (0–1) identifies a value's individual words, by position, in least-significant word order.

Syntax 2 is intended for use after an ALTGW instruction; i.e. in a loop to iteratively read a series of word values within contiguous long registers.

## ROR

Rotate right

Bit Operation Instruction - Rotate bits right.

ROR Dest, {#}Src {WC|WZ|WCZ}

Result: The bits of Dest are rotated right by Src bits; any departing LSBs are moved into MSBs.

- Dest is the register containing the value to rotate right by Src bits.

- Src is a register or 5-bit literal whose value indicates the number of bit positions to rotate.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0000000 | CZI | DDDDDDDDD | SSSSSSSSS | D | Last bit out1 | Result = 0 | 2 |

1 C = last bit rotated out if Src[4:0] > 0, else C = Dest[0].

Explanation:

ROR rotates Dest's binary value right by Src places (0–31 bits).  All LSBs rotated out are moved into the new MSBs.

If the WC or WCZ effect is specified, the C flag is updated to the value of the last bit rotated out (effectively C = result bit "31") if Src is 1–31, or to Dest[0] if Src is 0.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest result equals zero, or is cleared (0) if it is non-zero.  Since no bits are lost by this operation, the result will only be zero if Dest started at zero.

## SAL

Shift arithmetic left

Bit Operation Instruction - Shift bits left, extending the LSB.

SAL Dest, {#}Src {WC|WZ|WCZ}

Result: The bits of Dest are shifted left by Src bits, extending Dest[0] into new rightmost bits.

- Dest is the register containing the value to arithmetically left shift by Src bits.

- Src is a register or 5-bit literal whose value indicates the number of bits to arithmetically shift left.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0000111 | CZI | DDDDDDDDD | SSSSSSSSS | D | Last bit out1 | Result = 0 | 2 |

1 C = last bit shifted out if Src[4:0] > 0, else C = Dest[31].

Explanation:

SAL shifts Dest's binary value left by Src places (0–31 bits) and sets the new LSBs to that of the original Dest[0].  SAL is the complement of SAR for bit streams but not for math operations— use SHL instead for swift 32-bit integer multiplication by a power-of-two.

If the WC or WCZ effect is specified, the C flag is updated to the value of the last bit shifted out (effectively C = result bit "32") if Src is 1–31, or to Dest[31] if Src is 0.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest result equals zero, or is cleared (0) if it is non-zero.

## SAR

Shift arithmetic right

Math Instruction - Divide signed 32-bit integer by power-of-two; a.k.a shift bits right, extending the MSB.

SAR Dest, {#}Src {WC|WZ|WCZ}

Result: The bits of Dest are shifted right by Src bits, extending Dest[31] (the sign bit) into new leftmost bits.

- Dest is the register containing the value to arithmetically right shift by Src bits.

- Src is a register or 5-bit literal whose value indicates the number of bits to arithmetically shift right.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0000110 | CZI | DDDDDDDDD | SSSSSSSSS | D | Last bit out1 | Result = 0 | 2 |

1 C = last bit shifted out if Src[4:0] > 0, else C = Dest[0].

Related: SHR, SH

Explanation:

SAR shifts Dest's binary value right by Src places (0–31 bits) and sets the new MSBs to that of the original Dest[31]; preserving the sign of a signed integer.  This is useful for bit stream manipulation and for swift division— it is similar to SHR for swift division by a power-of-two, but is safe for both signed and unsigned integers.

If the WC or WCZ effect is specified, the C flag is updated to the value of the last bit shifted out (effectively C = result bit "-1") if Src is 1–31, or to Dest[0] if Src is 0.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest result equals zero, or is cleared (0) if it is non-zero.

## SCA

Scale

Math Instruction - Create unsigned 16-bit scale value for next instruction's S value.

SCA Dest, {#}Src {WZ}

Result: The upper 16 bits of the unsigned product from the 16-bit Dest and Src multiplication is substituted as the next instruction's Src value and optionally the Z flag is updated to the zero status.

- Dest is a register containing the 16-bit value to multiply with Src.

- Src is a register, 9-bit literal, or 16-bit augmented literal whose value is multiplied with Dest.

- WZ is an optional effect to update the Z flag.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1010001 | 0ZI | DDDDDDDDD | SSSSSSSSS | — | — | Product = 0 | 2 |

Related: SCAS

Explanation:

SCA multiplies the lower 16-bits of each of Dest and Src together, right shifts the 32-bit product by 16 (to scale down the result), and substitutes this value as the next instruction's Src value.

If the WZ effect is specified, the Z flag is set (1) if the product (before scaling down) is zero, or is cleared (0) if non-zero.

Note:  The instruction following SCA is shielded from interrupt.

## SCAS

Scale, signed

Math Instruction - Create signed 18-bit scale value for next instruction's S value.

SCAS Dest, {#}Src {WZ}

Result: The upper 18 bits of the signed product from the 16-bit Dest and Src multiplication is substituted as the next instruction's Src value and optionally the Z flag is updated to the zero status.

- Dest is a register containing the signed 16-bit value to multiply with Src.

- Src is a register, 9-bit literal, or signed 16-bit augmented literal whose value is multiplied with Dest.

- WZ is an optional effect to update the Z flag.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1010001 | 1ZI | DDDDDDDDD | SSSSSSSSS | — | — | Product = 0 | 2 |

Related: SCA

Explanation:

SCAS multiplies the lower, signed 16-bits of each of Dest and Src together, right shifts the 32-bit product by 14 (to scale down the result), and substitutes this value as the next instruction's Src value.

If the WZ effect is specified, the Z flag is set (1) if the product (before scaling down) is zero, or is cleared (0) if non-zero.

Note:  The instruction following SCAS is shielded from interrupt.

## SETBYTE

Set byte

Bit Operation Instruction - Set a byte to new value.

SETBYTE Dest, {#}Src, #Num

SETBYTE {#}Src

Result: Src[7:0] is written to byte Num (0–3) of Dest, or to another register byte described by prior ALTSB instruction.

- Dest is the register in which to modify a byte.

- Src is a register or 8-bit literal whose bits [7:0] will be stored in the designated location.

- Num is a 2-bit literal identifying the nibble ID (0–3) of Dest to modify.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1000110 | NNI | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1000110 | 00I | 000000000 | SSSSSSSSS | D1 | — | — | 2 |

1 Dest, and the target byte ID, is specified by a prior ALTSB instruction.

Related: ALTSB, SETNIB, SETWORD, GETNIB, GETBYTE, GETWORD, ROLNIB, ROLBYTE, and ROLWORD

Explanation:

SETBYTE stores Src[7:0] into the byte identified by Num within Dest, or the byte and register described by a prior ALTSB instruction.  No other bits are modified.

Num (0–3) identifies a value's individual bytes, by position, in least-significant byte order.

Syntax 2 is intended for use after an ALTSB instruction; i.e. in a loop to iteratively affect a series of byte values within contiguous long registers.

## SETD

Set destination

Indirection Instruction - Set template D field for ALTI.

SETD Dest, {#}Src

Result: The D field [17:9] of template Dest is set to Src[8:0].

- Dest is the register whose 32-bit value is a template for use with an ALTI instruction.

- Src is a register or 9-bit literal whose value (Src[8:0]) is copied to the D field of Dest.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001101 | 10I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |

Related: SETS, SETR, and ALTI

Explanation:

SETD copies Src[8:0] to the D field of the template, Dest, to be used with an ALTI instruction.  Bits outside the D field remain unaffected.  The D field (or Dest field) holds the address of a register (or sometimes a literal value) for the instruction to use as it's destination value, and usually as its result destination, during its execution.

The ALTI template is a 32-bit value with the following format:

| Bits | 31:28 (4 bits) | 27:19 (9 bits) | 18 | 17:9 (9 bits) | 8:0 (9 bits) |
| --- | --- | --- | --- | --- | --- |
| Field Description | Condition Field | Result Field | Indirect "I" Field | Dest "D" Field | Source "S" Field |

SETD can also be used in self-modifying Reg RAM code.  Unlike with ALTx instructions, when used this way, field value modification occurs in the program code itself (not the instruction pipeline); code is altered, values persist.  Due to the instruction pipeline nature, after modifying a code register, it is necessary to elapse at least two instructions before executing the modified register.

SETD    inst, op	'set "inst" register[17:9] to op[8:0]

NOP	'first spacer instruction, could be anything

NOP	'second spacer instruction, could be anything

inst  MOV     x, y	'operate on x using y; x may become any register per SETD

## SETNIB

Set nibble

Bit Operation Instruction - Set a nibble to new value.

SETNIB Dest, {#}Src, #Num

SETNIB {#}Src

Result: Src[3:0] is written to nibble Num (0–7) of Dest, or to another register nibble described by prior ALTSN instruction.

- Dest is the register in which to modify a nibble.

- Src is a register or 4-bit literal whose bits [3:0] will be stored in the designated location.

- Num is a 3-bit literal identifying the nibble ID (0–7) of Dest to modify.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 100000N | NNI | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1000000 | 00I | 000000000 | SSSSSSSSS | D1 | — | — | 2 |

1 Dest, and the target nibble ID, is specified by a prior ALTSN instruction.

Related: ALTSN, SETBYTE, SETWORD, GETNIB, GETBYTE, GETWORD, ROLNIB, ROLBYTE, and ROLWORD

Explanation:

SETNIB stores Src[3:0] into the nibble identified by Num within Dest, or the nibble and register described by a prior ALTSN instruction.  No other bits are modified.

Num (0–7) identifies a value's individual nibbles, by position, in least-significant nibble order.

Syntax 2 is intended for use after an ALTSN instruction; i.e. in a loop to iteratively affect a series of nibble values within contiguous long registers.

## SETR

Set result

Indirection Instruction - Set template Result field for ALTI.

SETR Dest, {#}Src

Result: The Result field [27:19] of template Dest is set to Src[8:0].

- Dest is the register whose 32-bit value is a template for use with an ALTI instruction.

- Src is a register or 9-bit literal whose value (Src[8:0]) is copied to the D field of Dest.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001101 | 01I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |

Related: SETD, SETS, and ALTI

Explanation:

SETR copies Src[8:0] to the Result field of the template, Dest, to be used with an ALTI instruction.  Bits outside the Result field remain unaffected.  The Result field does not exist in instruction opcodes, but takes its value from the Dest field, holding the address of a register for the instruction to use as its result destination upon execution.

The ALTI template is a 32-bit value with the following format:

| Bits | 31:28 (4 bits) | 27:19 (9 bits) | 18 | 17:9 (9 bits) | 8:0 (9 bits) |
| --- | --- | --- | --- | --- | --- |
| Field Description | Condition Field | Result Field | Indirect "I" Field | Dest "D" Field | Source "S" Field |

SETR can also be used in self-modifying Reg RAM code, though it won't affect the Register field (which doesn't exist in instruction opcodes) but rather will affect the Instr field and the upper two bits of the FX field.  Unlike with ALTx instructions, when used this way, field value modification occurs in the program code itself (not the instruction pipeline); code is altered, values persist.  Due to the instruction pipeline nature, after modifying a code register, it is necessary to elapse at least two instructions before executing the modified register.

SETR    inst, op	'set "inst" register[27:19] to op[8:0]

NOP	'first spacer instruction, could be anything

NOP	'second spacer instruction, could be anything

inst  MOV     x, y	'operate on x using y; MOV can become AND/OR/etc. per SETR

## SETS

Set source

Indirection Instruction - Set template S field for ALTI.

SETS Dest, {#}Src

Result: The S field [8:0] of template Dest is set to Src[8:0].

- Dest is the register whose 32-bit value is a template for use with an ALTI instruction.

- Src is a register or 9-bit literal whose value (Src[8:0]) is copied to the S field of Dest.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001101 | 11I | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |

Related: SETD, SETR, and ALTI

Explanation:

SETS copies Src[8:0] to the S field of the template, Dest, to be used with an ALTI instruction.  Bits outside the S field remain unaffected.  The S field (or Src field) holds the address of a register or literal value for an instruction to use as it's source value during its execution.

The ALTI template is a 32-bit value with the following format:

| Bits | 31:28 (4 bits) | 27:19 (9 bits) | 18 | 17:9 (9 bits) | 8:0 (9 bits) |
| --- | --- | --- | --- | --- | --- |
| Field Description | Condition Field | Result Field | Indirect "I" Field | Dest "D" Field | Source "S" Field |

SETS can also be used in self-modifying Reg RAM code.  Unlike with ALTx instructions, when used this way, field value modification occurs in the program code itself (not the instruction pipeline); code is altered, values persist.  Due to the instruction pipeline nature, after modifying a code register, it is necessary to elapse at least two instructions before executing the modified register.

SETS    inst, op	'set "inst" register[8:0] to op[8:0]

NOP	'first spacer instruction, could be anything

NOP	'second spacer instruction, could be anything

inst  MOV     x, y	'operate on x using y; y may become any register/value per SETS

## SETWORD

Set word

Bit Operation Instruction - Set a word to new value.

SETWORD Dest, {#}Src, #Num

SETWORD {#}Src

Result: Src[15:0] is written to word Num (0–1) of Dest, or to another register word described by prior ALTSW instruction.

- Dest is the register in which to modify a word.

- Src is a register, 9-bit literal, or 16-bit augmented literal whose bits [15:0] will be stored in the designated location.

- Num is a 1-bit literal identifying the word ID (0–1) of Dest to modify.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1001001 | 0NI | DDDDDDDDD | SSSSSSSSS | D | — | — | 2 |
| EEEE | 1001001 | 00I | 000000000 | SSSSSSSSS | D1 | — | — | 2 |

1 Dest, and the target word ID, is specified by a prior ALTSW instruction.

Related: ALTSW, SETNIB, SETBYTE, GETNIB, GETBYTE, GETWORD, ROLNIB, ROLBYTE, and ROLWORD

Explanation:

SETWORD stores Src[15:0] into the word identified by Num within Dest, or the word and register described by a prior ALTSW instruction.  No other bits are modified.

Num (0–1) identifies a value's individual words, by position, in least-significant word order.

Syntax 2 is intended for use after an ALTSW instruction; i.e. in a loop to iteratively affect a series of word values within contiguous long registers.

## SHL

Shift left

Bit Operation / Math Instruction - Shift bits left; a.k.a. multiply 32-bit integer by power-of-two.

SHL Dest, {#}Src {WC|WZ|WCZ}

Result: The bits of Dest are shifted left by Src bits, inserting zeros (0) as new rightmost bits.

- Dest is the register containing the value to left shift by Src bits.

- Src is a register or 5-bit literal whose value indicates the number of bits to shift left.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0000011 | CZI | DDDDDDDDD | SSSSSSSSS | D | Last bit out1 | Result = 0 | 2 |

1 C = last bit shifted out if Src[4:0] > 0, else C = Dest[31].

Explanation:

SHL shifts Dest's binary value left by Src places (0–31 bits) and sets the new LSBs to 0.  This is useful for bit-stream manipulation as well as for swift multiplication; signed or unsigned 32-bit integer multiplication by a power-of-two.  Care must be taken for power-of-two multiplications since upper bits will shift through the MSB (sign bit); mangling large signed values.

If the WC or WCZ effect is specified, the C flag is updated to the value of the last bit shifted out (effectively C = result bit "32") if Src is 1–31, or to Dest[31] if Src is 0.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest result equals zero, or is cleared (0) if it is non-zero.

## SHR

Shift right

Bit Operation / Math Instruction - Shift bits right; a.k.a. divide unsigned 32-bit integer by power-of-two.

SHR Dest, {#}Src {WC|WZ|WCZ}

Result: The bits of Dest are shifted right by Src bits, inserting zeros (0) as new leftmost bits.

- Dest is the register containing the value to right shift by Src bits.

- Src is a register or 5-bit literal whose value indicates the number of bits to shift right.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0000010 | CZI | DDDDDDDDD | SSSSSSSSS | D | Last bit out1 | Result = 0 | 2 |

1 C = last bit shifted out if Src[4:0] > 0, else C = Dest[0].

Explanation:

SHR shifts Dest's binary value right by Src places (0–31 bits) and sets the new MSBs to 0.  This is useful for bit-stream manipulation as well as for swift division; unsigned 32-bit integer division by a power-of-two.  For similar division of a signed value, use SAR instead.

If the WC or WCZ effect is specified, the C flag is updated to the value of the last bit shifted out (effectively C = result bit "-1") if Src is 1–31, or to Dest[0] if Src is 0.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest result equals zero, or is cleared (0) if it is non-zero.

## SIGNX

Sign extend

Math Instruction - Sign-extend value beyond designated bit.

SIGNX Dest, {#}Src {WC|WZ|WCZ}

Result: The Dest value is sign-extended above the bit indicated by Src and is stored in Dest.  Optionally the C and Z flags are updated to the resulting MSB and zero status.

- Dest is a register containing the value to sign-extend above bit Src[4:0] and is where the result is written.

- Src is a register or 9-bit literal whose value (lower 5 bits) identifies the bit of Dest to zero-extend beyond.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0111011 | CZI | DDDDDDDDD | SSSSSSSSS | D | MSB of result | Result = 0 | 2 |

Related: ZEROX

Explanation:

SIGNX fills the bits of Dest, above the bit indicated by Src[4:0], with the value of that identified bit; i.e. sign-extending the value.  This is handy when converting encoded or received signed values from a small bit width to a large bit with; i.e. 32 bits.

If the WC or WCZ effect is specified, the C flag is set to the result's MSB value.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

## STALLI

Stall interrupts

Interrupt Instruction - Prevent further interrupts.

STALLI

Result: All future interrupts are disallowed.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | 000 | 000100001 | 000100100 | — | — | — | 2 |

Related: ALLOWI

Explanation:

STALLI disables interrupt branching.  STALLI is the complement of the ALLOWI instruction— both are used to protect short, vital sections of main code from timing jitter or state loss caused by asynchronous interrupt handling.

## SUB

Subtract

Math Instruction - Subtract one unsigned value from another.

SUB Dest, {#}Src {WC|WZ|WCZ}

Result: Difference of unsigned Dest and unsigned Src is stored in Dest and optionally the C and Z flags are updated to the borrow and zero status.

- Dest is a register containing the value to subtract Src from, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is subtracted from Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0001100 | CZI | DDDDDDDDD | SSSSSSSSS | D | borrow of (D - S) | Result = 0 | 2 |

Related: SUBX, SUBS, SUBSX, ADD, and SUBR

Explanation:

SUB subtracts the unsigned Src from the unsigned Dest and stores the result into the Dest register.

If the WC or WCZ effect is specified, the C flag is set (1) if the subtraction results in a 32-bit underflow (unsigned borrow), or is cleared (0) if no borrow.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result of Dest - Src is zero, or is cleared (0) if it is non-zero.

To subtract unsigned, multi-long values, use SUB followed by SUBX as described in Subtracting Two Multi-Long Values.  SUB and SUBX are also used in subtracting signed, multi-long values with SUBSX ending the sequence.

## SUBR

Subtract reverse

Math Instruction - Subtract one unsigned value from another (in reverse order to SUB).

SUBR Dest, {#}Src {WC|WZ|WCZ}

Result: Difference of unsigned Src and unsigned Dest is stored in Dest and optionally the C and Z flags are updated to the borrow and zero status.

- Dest is a register containing the value to subtract from Src, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is subtracted by Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0010110 | CZI | DDDDDDDDD | SSSSSSSSS | D | borrow of (S - D) | Result = 0 | 2 |

Related: SUB

Explanation:

SUBR subtracts the unsigned Dest from the unsigned Src and stores the result into the Dest register.  This is the reverse of the subtraction order of SUB.

If the WC or WCZ effect is specified, the C flag is set (1) if the subtraction results in a 32-bit underflow (unsigned borrow), or is cleared (0) if no borrow.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result of Dest - Src is zero, or is cleared (0) if it is non-zero.

## SUBS

Subtract signed

Math Instruction - Subtract one signed value from another.

SUBS Dest, {#}Src {WC|WZ|WCZ}

Result: Difference of signed Dest and signed Src is stored in Dest and optionally the C and Z flags are updated to the sign and zero status.

- Dest is a register containing the value to subtract Src from, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value is subtracted from Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0001110 | CZI | DDDDDDDDD | SSSSSSSSS | D | sign of (D - S) | Result = 0 | 2 |

Related: SUB, SUBX, SUBSX, and ADDS

Explanation:

SUBS subtracts the signed Src from the signed Dest and stores the result into the Dest register.  If Src is a 9-bit literal, its value is interpreted as positive (0-511; it is not sign-extended) — use ##Value (or insert a prior AUGS instruction) for a 32-bit signed value; negative or positive.

If the WC or WCZ effect is specified, the C flag is set (1) if the subtraction results in a signed underflow (signed borrow), or is cleared (0) if no underflow.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result of Dest - Src is zero, or is cleared (0) if it is non-zero.

To subtract signed, multi-long values, use SUB (not SUBS) followed possibly by SUBX, and finally SUBSX as described in Subtracting Two Multi-Long Values.

## SUBSX

Subtract signed, extended

Math Instruction - Subtract one signed extended value from another.

SUBSX Dest, {#}Src {WC|WZ|WCZ}

Result: Difference of signed Dest and signed Src (plus C) is stored in Dest and optionally the C and Z flags are updated to the extended sign and zero status.

- Dest is a register containing the value to subtract Src plus C from, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value plus C is subtracted from Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0001111 | CZI | DDDDDDDDD | SSSSSSSSS | D | sign of D-(S+C) | Z AND (Result = 0) | 2 |

Related: SUB, SUBX, SUBSX, and ADDSX

Explanation:

SUBSX subtracts the signed value of Src plus C from the signed Dest and stores the result into the Dest register.  The SUBSX instruction is used to perform signed multi-long (extended) subtraction, such as 64-bit subtraction.

If the WC or WCZ effect is specified, the C flag is set (1) if the result is negative (Result[31] = 1), or is cleared (0) if positive.  Use WC or WCZ on preceding SUB and SUBX instructions for proper final C flag.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Z was previously set and the result of Dest - (Src + C) is zero, or it is cleared (0) if non-zero.  Use WZ or WCZ on preceding SUB and SUBX instructions for proper final Z flag.

To subtract signed multi-long values, use SUB (not SUBS) followed possibly by SUBX, and finally SUBSX as described in Subtracting Two Multi-Long Values.

## SUBX

Subtract extended

Math Instruction - Subtract one unsigned extended value from another.

SUBX Dest, {#}Src {WC|WZ|WCZ}

Result: Difference of unsigned Dest and unsigned Src (plus C) is stored in Dest and optionally the C and Z flags are updated to the extended borrow and zero status.

- Dest is a register containing the value to subtract Src plus C from, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value plus C is subtracted from Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0001101 | CZI | DDDDDDDDD | SSSSSSSSS | D | borrow of D-(S+C) | Z AND (Result = 0) | 2 |

Related: SUB, SUBSX, and ADDX

Explanation:

SUBX subtracts the unsigned value of Src plus C from the unsigned Dest and stores the result into the Dest register.  The SUBX instruction is used to perform unsigned multi-long (extended) subtraction, such as 64-bit subtraction.

If the WC or WCZ effect is specified, the C flag is set (1) if the subtraction results in an unsigned borrow, or is cleared (0) if no borrow.  Use WC or WCZ on preceding SUB and SUBX instructions for proper final C flag.  If C is set after the last SUBX in a multi-long subtraction, it indicates unsigned underflow.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Z was previously set and the result of Dest - (Src + C) is zero, or it is cleared (0) if non-zero.  Use WZ or WCZ on preceding SUB and SUBX instructions for proper final Z flag.

To subtract unsigned multi-long values, use SUB followed by one or more SUBX instructions as described in Subtracting Two Multi-Long Values.

## SUMC / SUMNC

Sum C or Sum not C

Math Instruction - Adjust signed value by other C-negated or !C-negated value.

SUMC Dest, {#}Src	{WC|WZ|WCZ}

SUMNC Dest, {#}Src	{WC|WZ|WCZ}

Result: The sum of signed Dest and either Src or -Src (according to C) is stored in Dest and optionally the C and Z flags are updated to the sign and zero status.

- Dest is a register containing the signed value to adjust by Src or -Src, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value (if C=0 or !C=0) or negated value (if C=1 or !C=1) is added into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0011100 | CZI | DDDDDDDDD | SSSSSSSSS | D | sign of (D +/- S) | Result = 0 | 2 |
| EEEE | 0011101 | CZI | DDDDDDDDD | SSSSSSSSS | D | sign of (D +/- S) | Result = 0 | 2 |

Related: SUMZ and SUMNZ

Explanation:

SUMC or SUMNC adjusts the signed Dest value by Src or -Src (depending on C or !C) and stores the result into the Dest register.  Prior to adding to Dest, the Src value is negated if C (SUMC) or !C (SUMNC) is 1.

If the WC or WCZ effect is specified, the C flag is set (1) if the result is negative, or is cleared (0) if positive.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result of Dest +/- Src is zero, or is cleared (0) if it is non-zero.

## SUMZ / SUMNZ

Sum Z or Sum not Z

Math Instruction - Adjust signed value by other Z-negated or !Z-negated value.

SUMZ Dest, {#}Src	{WC|WZ|WCZ}

SUMNZ Dest, {#}Src	{WC|WZ|WCZ}

Result: The sum of signed Dest and either Src or -Src (according to Z) is stored in Dest and optionally the C and Z flags are updated to the sign and zero status.

- Dest is a register containing the signed value to adjust by Src or -Src, and is where the result is written.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value (if Z=0 or !Z=0) or negated value (if Z=1 or !Z=1) is added into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0011110 | CZI | DDDDDDDDD | SSSSSSSSS | D | sign of (D +/- S) | Result = 0 | 2 |
| EEEE | 0011111 | CZI | DDDDDDDDD | SSSSSSSSS | D | sign of (D +/- S) | Result = 0 | 2 |

Related: SUMC and SUMNC

Explanation:

SUMZ or SUMNZ adjusts the signed Dest value by Src or -Src (depending on Z or !Z) and stores the result into the Dest register.  Prior to adding to Dest, the Src value is negated if Z (SUMZ) or !Z (SUMNZ) is 1.

If the WC or WCZ effect is specified, the C flag is set (1) if the result is negative, or is cleared (0) if positive.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result of Dest +/- Src is zero, or is cleared (0) if it is non-zero.

## TEST

Test

Bit Operation Instruction - Test D, or bitwise AND D with S, to affect flags.

TEST Dest	{WC|WZ|WCZ}

TEST Dest, {#}Src	{WC|WZ|WCZ}

Result: The parity and zero-state of Dest, or of Dest bitwise ANDed with Src, is stored in the C and Z flags.

- Dest is the register whose value will be tested.

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose value is ANDed with Dest.

- WC, WZ, and WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0111110 | CZ0 | DDDDDDDDD | DDDDDDDDD | — | Parity of D | D = 0 | 2 |
| EEEE | 0111110 | CZI | DDDDDDDDD | SSSSSSSSS | — | Parity of (D & S) | (D & S) = 0 | 2 |

Related: TESTN

Explanation:

TEST determines the parity (number of high (1) bits) and the zero or non-zero state of Dest, or of Dest bitwise ANDed with Src, and stores the results in the C and/or Z flag.

If the WC or WCZ effect is specified, the C flag is set (1) if the number of high (1) bits in Dest (or Dest ANDed with Src) is odd, or is cleared (0) if it is even.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Dest (or Dest ANDed with Src) is zero, or is cleared (0) if it is not zero.

## TESTB / TESTBN

Test bit or bit not

Bit Operation Instruction - Test bit of D or !D and either store, AND, OR, or XOR the result into flags.

TESTB	Dest, {#}Src	WC|WZ

TESTB	Dest, {#}Src	ANDC|ANDZ

TESTB	Dest, {#}Src	ORC|ORZ

TESTB	Dest, {#}Src	XORC|XORZ

TESTBN	Dest, {#}Src	WC|WZ

TESTBN	Dest, {#}Src	ANDC|ANDZ

TESTBN	Dest, {#}Src	ORC|ORZ

TESTBN	Dest, {#}Src	XORC|XORZ

Result: The state of Dest's bit Src is read, possibly inverted, and either stored as-is, or bitwise ANDed, ORed, or XORed into C or Z.

- Dest is the register whose value will have a single bit tested.

- Src is a register or 5-bit literal whose value identifies the bit (0–31) of Dest to test.

- WC, WZ, ANDC, ANDZ, ORC, ORZ, XORC, and XORZ is a required effect to update or bitwise manipulate the C or Z flag.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0100000 | CZI | DDDDDDDDD | SSSSSSSSS | — | IN[D[4:0]] | IN[D[4:0]] | 2 |
| EEEE | 0100010 | CZI | DDDDDDDDD | SSSSSSSSS | — | C AND IN[D[4:0]] | Z AND IN[D[4:0]] | 2 |
| EEEE | 0100100 | CZI | DDDDDDDDD | SSSSSSSSS | — | C OR IN[D[4:0]] | Z OR IN[D[4:0]] | 2 |
| EEEE | 0100110 | CZI | DDDDDDDDD | SSSSSSSSS | — | C XOR IN[D[4:0]] | Z XOR IN[D[4:0]] | 2 |
| EEEE | 0100001 | CZI | DDDDDDDDD | SSSSSSSSS | — | !IN[D[4:0]] | !IN[D[4:0]] | 2 |
| EEEE | 0100011 | CZI | DDDDDDDDD | SSSSSSSSS | — | C AND !IN[D[4:0]] | Z AND !IN[D[4:0]] | 2 |
| EEEE | 0100101 | CZI | DDDDDDDDD | SSSSSSSSS | — | C OR !IN[D[4:0]] | Z OR !IN[D[4:0]] | 2 |
| EEEE | 0100111 | CZI | DDDDDDDDD | SSSSSSSSS | — | C XOR !IN[D[4:0]] | Z XOR !IN[D[4:0]] | 2 |

Related: TESTP and TESTPN

Explanation:

TESTB or TESTBN reads the state (0/1) of a bit in Dest designated by Src, possibly inverts that result, and either stores it as-is, or bitwise ANDs, ORs, or XORs it into the C or Z flag.

Src[4:0] indicates the bit number (0–31) to test.

If the WC or WZ effect is specified, the C or Z flag is overwritten with the state or inverse state of the bit.

If the ANDC or ANDZ effect is specified, the C or Z flag is bitwise ANDed with the state or inverse state of the bit.

If the ORC or ORZ effect is specified, the C or Z flag is bitwise ORed with the state or inverse state of the bit.

If the XORC or XORZ effect is specified, the C or Z flag is bitwise XORed with the state or inverse state of the bit.

## TESTN

Test not

Bit Operation Instruction - Test D by bitwise ANDing with !S to affect flags only.

TESTN Dest, {#}Src {WC|WZ|WCZ}

Result: The parity and zero-state of Dest bitwise ANDed with !Src is stored in the C and Z flags.

- Dest is the register whose value will be tested.

- Src is an optional register, 9-bit literal, or 32-bit augmented literal whose inverse value is ANDed with Dest.

- WC, WZ, and WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0111111 | CZI | DDDDDDDDD | SSSSSSSSS | — | Parity of (D & !S) | (D & !S) = 0 | 2 |

Related: TEST

Explanation:

TESTN determines the parity (number of high (1) bits) and the zero or non-zero state of Dest bitwise ANDed with !Src and stores the results in the C and/or Z flag.

If the WC or WCZ effect is specified, the C flag is set (1) if the number of high (1) bits in Dest ANDed with !Src is odd, or is cleared (0) if it is even.

If the WZ or WCZ effect is specified, the Z flag is set (1) if Dest ANDed with !Src is zero, or is cleared (0) if it is not zero.

## TESTP / TESTPN

Test pin or pin not

I/O Pin Instruction - Test pin and either store, AND, OR, or XOR the result or inverse result into C/Z.

TESTP	{#}Dest	WC|WZ

TESTP	{#}Dest	ANDC|ANDZ

TESTP	{#}Dest	ORC|ORZ

TESTP	{#}Dest	XORC|XORZ

TESTPN	{#}Dest	WC|WZ

TESTPN	{#}Dest	ANDC|ANDZ

TESTPN	{#}Dest	ORC|ORZ

TESTPN	{#}Dest	XORC|XORZ

Result: The state of the I/O pin described by Dest is read, possibly inverted, and either stored as-is, or bitwise ANDed, ORed, or XORed into C or Z.

- Dest is the register or 6-bit literal whose value identifies the I/O pin to test.

- WC, WZ, ANDC, ANDZ, ORC, ORZ, XORC, and XORZ is a required effect to update or bitwise manipulate the C or Z flag.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000000 | — | IN[D[5:0]] | IN[D[5:0]] | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000010 | — | C AND IN[D[5:0]] | Z AND IN[D[5:0]] | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000100 | — | C OR IN[D[5:0]] | Z OR IN[D[5:0]] | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000110 | — | C XOR IN[D[5:0]] | Z XOR IN[D[5:0]] | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000001 | — | !IN[D[5:0]] | !IN[D[5:0]] | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000011 | — | C AND !IN[D[5:0]] | Z AND !IN[D[5:0]] | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000101 | — | C OR !IN[D[5:0]] | Z OR !IN[D[5:0]] | 2 |
| EEEE | 1101011 | CZL | DDDDDDDDD | 001000111 | — | C XOR !IN[D[5:0]] | Z XOR !IN[D[5:0]] | 2 |

Related: TESTB and TESTBN

Explanation:

TESTP or TESTPN reads the state (0/1) of the I/O pin designated by Dest, possibly inverts that result, and either stores it as-is, or bitwise ANDs, ORs, or XORs it into the C or Z flag.  Dest[5:0] indicates the pin number (0–63) to test.

If the WC or WZ effect is specified, the C or Z flag is overwritten with the state or inverse state of the pin.

If the ANDC or ANDZ effect is specified, the C or Z flag is bitwise ANDed with the state or inverse state of the pin.

If the ORC or ORZ effect is specified, the C or Z flag is bitwise ORed with the state or inverse state of the pin.

If the XORC or XORZ effect is specified, the C or Z flag is bitwise XORed with the state or inverse state of the pin.

## TJF / TJNF

Test, jump if full or not full

Flow Control Instruction - Test value and jump if full (-1; $FFFF_FFFF) or not full (<> -1; <> $FFFF_FFFF).

TJF	Dest, {#}Src

TJNF	Dest, {#}Src

Result: Dest is tested and if it's full (or not full in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Dest is a register whose value is tested for full or not full.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011101 | 00I | DDDDDDDDD | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011101 | 01I | DDDDDDDDD | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when Dest is full (or not full in syntax 2).

Explanation:

TJF or TJNF tests the value in Dest and jumps to the address described by Src if the result is full (-1; $FFFF_FFFF; in syntax 1) or not full (<> -1; <> $FFFF_FFFF; in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the TJF / TJNF.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## TJS / TJNS

Test, jump if signed or not signed

Flow Control Instruction - Test value and jump if signed or not signed.

TJS	Dest, {#}Src

TJNS	Dest, {#}Src

Result: Dest is tested and if it's signed (or not signed in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Dest is a register whose value is tested for sign or no sign.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011101 | 10I | DDDDDDDDD | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011101 | 11I | DDDDDDDDD | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when Dest is signed (or not signed in syntax 2).

Explanation:

TJS or TJNS tests the value in Dest and jumps to the address described by Src if the result is signed (Dest[31] = 1) or not signed (Dest[31] = 0).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the TJS / TJNS.  The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## TJV

Test, jump if overflow

Flow Control Instruction - Test value and jump if overflowed.

TJV Dest, {#}Src

Result: Dest and C are tested and if there's been an overflow, PC is set to a new relative (#Src) or absolute (Src) address.

- Dest is a register whose value is tested for overflow (Dest[31] != C).

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011110 | 00I | DDDDDDDDD | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when Dest has overflowed.

Explanation:

TJV tests the value in Dest against C and jumps to the address described by Src if Dest has overflowed (Dest[31] != C).  This instruction requires that C be updated (to the correct sign) by the previous ADDS / ADDSX / SUBS / SUBSX / CMPS / CMPSX / SUMx instruction.

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the TJV. The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## TJZ / TJNZ

Test, jump if zero or not zero

Flow Control Instruction - Test value and jump if zero or not zero.

TJZ	Dest, {#}Src

TJNZ	Dest, {#}Src

Result: Dest is tested and if it's zero (or not zero in syntax 2), PC is set to a new relative (#Src) or absolute (Src) address.

- Dest is a register whose value is tested for zero or not zero.

- Src is a register, 9-bit literal, or 20-bit augmented literal whose value is the absolute or relative address to set PC to.  Use # for relative addressing; omit # for absolute addressing.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1011100 | 10I | DDDDDDDDD | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |
| EEEE | 1011100 | 11I | DDDDDDDDD | SSSSSSSSS | PC1 | — | — | 2 or 4 / 2 or 13–20 |

1 PC is written only when Dest is zero (or not zero in syntax 2).

Explanation:

TJZ or TJNZ tests the value in Dest and jumps to the address described by Src if the result is zero (syntax 1) or not zero (in syntax 2).

The address (Src) can be absolute or relative.  To specify an absolute address, Src must be a register containing a 20-bit address value.  To specify a relative address, use #Label for a 9-bit signed offset (a range of -256 to +255 instructions) or use ##Label (or insert a prior AUGS instruction) for a 20-bit signed offset (a range of -524288 to +524287).  Offsets are relative to the instruction following the TJZ / TJNZ. The signed offset value is in units of whole instructions— it is added to PC as-is when in Cog/LUT execution mode and is multiplied by 4 then added to PC when in Hub execution mode (long-aligned Hub code not required).

## WAITATN

Wait attention

Event Monitor Instruction - Wait for and clear attention flag.

WAITATN {WC|WZ|WCZ}

Result: Wait for attention event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000011110 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: COGATN, POLLATN, JATN, and JNATN

Explanation:

WAITATN waits for an attention event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITATN can time-out if the attention event doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITATN.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The attention event flag is set whenever another cog issues an attention request for this cog.  The attention event flag is cleared upon cog start, or execution of POLLATN, WAITATN, JATN, or JNATN instructions.

## WAITCT1/2/3

Wait counter 1/2/3

Event Monitor Instruction - Wait for and clear counter 1, 2, or 3 event flag.

WAITCT1 {WC|WZ|WCZ}

WAITCT2 {WC|WZ|WCZ}

WAITCT3 {WC|WZ|WCZ}

Result: Wait for counter 1, 2, or 3 event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000010001 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |
| EEEE | 1101011 | CZ0 | 000010010 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |
| EEEE | 1101011 | CZ0 | 000010011 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: ADDCTx, POLLCTx, JCTx, and JNCTx

Explanation:

WAITCT1, WAITCT2, or WAITCT3 waits for a counter 1, 2, or 3 event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITCTx can time-out if the counter event doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITCTx.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The counter 1, 2, or 3 event flag is set whenever the System Counter (CT) passes the value in the CT1, CT2, or CT3 event trigger register, respectively; i.e. MSB of (CT - CTx is 0).  The counter event flags are cleared upon execution of the corresponding ADDCTx, POLLCTx, WAITCTx, JCTx, or JNCTx instructions.

## WAITFBW

Wait FIFO block wrap

Event Monitor Instruction - Wait for and clear FIFO-interface-block-wrap event flag.

WAITFBW {WC|WZ|WCZ}

Result: Wait for FIFO-interface-block-wrap event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000011001 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: RDFAST, WRFAST, FBLOCK, POLLFBW, JFBW, and JNFBW

Explanation:

WAITFBW waits for a FIFO-interface-block-wrap event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITFBW can time-out if the FIFO-interface-block-wrap event doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITFBW.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The FIFO-interface-block-wrap event flag is set whenever the Hub RAM FIFO interface exhausts its block count and reloads its block count and start address.  The FIFO-interface-block-wrap event flag is cleared upon execution of RDFAST, WRFAST, FBLOCK, POLLFBW, WAITFBW, JFBW, or JNFBW instructions.

## WAITINT

Wait interrupt

Event Monitor Instruction - Wait for and clear interrupt-occurred event flag.

WAITINT {WC|WZ|WCZ}

Result: Wait for interrupt-occurred event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000010000 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: POLLINT, JINT, and JNINT

Explanation:

WAITINT waits for an interrupt-occurred event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITINT can time-out if the interrupt-occurred event doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITINT.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The interrupt-occurred event flag is set whenever interrupt 1, 2, or 3 occurs— debug interrupts are ignored.  The interrupt-occurred event flag is cleared upon cog start, or execution of POLLINT, WAITINT, JINT, or JNINT instructions.

## WAITPAT

Wait pattern

Event Monitor Instruction - Wait for and clear pin-pattern-detected event flag.

WAITPAT {WC|WZ|WCZ}

Result: Wait for pin-pattern-detected event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000011000 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: SETPAT, POLLPAT, JPAT, and JNPAT

Explanation:

WAITPAT waits for a pin-pattern-detected event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITPAT can time-out if the pin-pattern-detected event doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITPAT.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The pin-pattern-detected event flag is set whenever the masked input pins match or don't match the pattern described by a previous SETPAT instruction.  The pin-pattern-detected event flag is cleared upon execution of SETPAT, POLLPAT, WAITPAT, JPAT, or JNPAT instructions.

## WAITSE1/2/3/4

Wait selectable event 1/2/3/4

Event Monitor Instruction - Wait for and clear selectable event 1, 2, 3, or 4 flag.

WAITSE1 {WC|WZ|WCZ}

WAITSE2 {WC|WZ|WCZ}

WAITSE3 {WC|WZ|WCZ}

WAITSE4 {WC|WZ|WCZ}

Result: Wait for selectable event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000010100 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |
| EEEE | 1101011 | CZ0 | 000010101 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |
| EEEE | 1101011 | CZ0 | 000010110 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |
| EEEE | 1101011 | CZ0 | 000010111 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: SETSEx, POLLSEx, JSEx, and JNSEx

Explanation:

WAITSE1, WAITSE2, WAITSE3, or WAITSE4 waits for a selectable event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITSEx can time-out if the selectable event doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITSEx.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The selectable event 1, 2, 3, or 4 flag is set whenever the corresponding configured event occurs.  The selectable event flag is cleared upon execution of the corresponding SETSEx, POLLSEx, WAITSEx, JSEx, or JNSEx instructions.

## WAITXFI

Wait streamer finished

Event Monitor Instruction - Wait for and clear streamer-finished event flag.

WAITXFI {WC|WZ|WCZ}

Result: Wait for streamer-finished event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000011011 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: XINIT, XZERO, XCONT, POLLXFI, JXFI, and JNXFI

Explanation:

WAITXFI waits for a streamer-finished event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITXFI can time-out if the streamer-finished doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITXFI.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The streamer-finished event flag is set whenever the streamer runs out of commands to process.  The streamer-finished event flag is cleared upon execution of XINIT, XZERO, XCONT, POLLXFI, WAITXFI, JXFI, or JNXFI instructions.

## WAITXMT

Wait streamer empty

Event Monitor Instruction - Wait for and clear streamer-empty event flag.

WAITXMT {WC|WZ|WCZ}

Result: Wait for streamer-empty event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000011010 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: XINIT, XZERO, XCONT, POLLXMT, JXMT, and JNXMT

Explanation:

WAITXMT waits for a streamer-empty event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITXMT can time-out if the streamer-empty event doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITXMT.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The streamer-empty event flag is set whenever the streamer is ready for a new command.  The streamer-empty event flag is cleared upon execution of XINIT, XZERO, XCONT, POLLXMT, WAITXMT, JXMT, or JNXMT instructions.

## WAITXRL

Wait streamer rollover LUT

Event Monitor Instruction - Wait for and clear streamer-LUT-RAM-rollover event flag

WAITXRL {WC|WZ|WCZ}

Result: Wait for streamer-LUT-RAM-rollover event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000011101 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: XINIT, XZERO, XCONT, POLLXRL, JXRL, and JNXRL

Explanation:

WAITXRL waits for a streamer-LUT-RAM-rollover event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITXRL can time-out if the streamer-LUT-RAM-rollover event doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITXRL.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The streamer-LUT-RAM-rollover event flag is set whenever location $1FF of the Lookup RAM is read by the streamer.  The streamer-LUT-RAM-rollover event flag is cleared upon cog start or upon execution of POLLXRL, WAITXRL, JXRL, or JNXRL instructions.

## WAITXRO

Wait streamer rollover NCO

Event Monitor Instruction - Wait for and clear streamer-NCO-rollover event flag

WAITXRO {WC|WZ|WCZ}

Result: Wait for streamer-NCO-rollover event, then clear the flag; optionally aborting on timeout and setting C and/or Z on abort.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | CZ0 | 000011100 | 000100100 | — | Timeout Abort | Timeout Abort | 2+ |

Related: XINIT, XZERO, XCONT, POLLXRO, JXRO, and JNXRO.

Explanation:

WAITXRO waits for a streamer-NCO-rollover event to occur (unless the event flag is already set), then clears the event flag (unless it's being set again by the event sensor) and resumes execution at the next instruction.  Optionally, WAITXRO can time-out if the streamer-NCO-rollover event doesn't occur soon enough; setting C and/or Z flags and then resuming execution at the next instruction.

To set the optional timeout, insert a SETQ (with a future System Counter target value) right before WAITXRO.

The WC, WZ, or WCZ effect is recommended only if the optional timeout is specified, in which case the C flag and/or Z flag is set (1) if a timeout occurred before the event, or is cleared (0) if the event occurred before the timeout.

During a wait, the pipeline is stalled; no instructions execute and no interrupts are processed in the cog until the wait condition ends.

The streamer-NCO-rollover event flag is set whenever the streamer's numerically-controlled oscillator (NCO) rolls over.  The streamer-NCO-rollover event flag is cleared upon execution of XINIT, XZERO, XCONT, POLLXRO, WAITXRO, JXRO, or JNXRO instructions.

## WRC / WRNC

Write C or not C

Bit Operation Instruction - Write C or not C to register.

WRC	Dest

WRNC	Dest

Result: Dest value is replaced with state of C or !C.

- Dest is the register whose value will be replaced with the state of C or !C.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | 000 | DDDDDDDDD | 001101100 | D | — | — | 2 |
| EEEE | 1101011 | 000 | DDDDDDDDD | 001101101 | D | — | — | 2 |

Explanation:

WRC or WRNC writes the state or inverse state of C (0 or 1) to Dest.

## WRZ / WRNZ

Write Z or not Z

Bit Operation Instruction - Write Z or not Z to register.

WRZ	Dest

WRNZ	Dest

Result: Dest value is replaced with state of Z or !Z.

- Dest is the register whose value will be replaced with the state of Z or !Z.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 1101011 | 000 | DDDDDDDDD | 001101110 | D | — | — | 2 |
| EEEE | 1101011 | 000 | DDDDDDDDD | 001101111 | D | — | — | 2 |

Explanation:

WRZ or WRNZ writes the state or inverse state of Z (0 or 1) to Dest.

## XOR

Exclusive or

Bit Operation Instruction - Bitwise XOR a value with another.

XOR Dest, {#}Src {WC|WZ|WCZ}

Result: Dest XOR Src is stored in Dest and flags are optionally updated with parity and zero status.

- Dest is the register containing the value to bitwise XOR with Src and is the destination in which to write the result.

- Src is a register, 9-bit literal, or 32-bit augmented literal whose value will be bitwise XORed into Dest.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0101011 | CZI | DDDDDDDDD | SSSSSSSSS | D | Parity of Result | Result = 0 | 2 |

Explanation:

XOR performs a bitwise XOR of the value in Src into that of Dest.

If the WC or WCZ effect is specified, the C flag is set (1) if the result contains an odd number of high (1) bits, or is cleared (0) if it contains an even number of high bits.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the Dest XOR Src result equals zero, or is cleared (0) if it is non-zero.

## ZEROX

Zero extend

Math Instruction - Zero-extend value beyond designated bit.

ZEROX Dest, {#}Src {WC|WZ|WCZ}

Result: The Dest value is zero-extended above the bit indicated by Src and is stored in Dest.  Optionally the C and Z flags are updated to the resulting MSB and zero status.

- Dest is a register containing the value to zero-extend above bit Src[4:0] and is where the result is written.

- Src is a register or 9-bit literal whose value (lower 5 bits) identifies the bit of Dest to zero-extend beyond.

- WC, WZ, or WCZ are optional effects to update flags.

| COND | INSTR | FX | DEST | SRC | Write | C Flag | Z Flag | Clocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EEEE | 0111010 | CZI | DDDDDDDDD | SSSSSSSSS | D | MSB of result | Result = 0 | 2 |

Related: SIGNX

Explanation:

ZEROX fills the bits of Dest, above the bit indicated by Src[4:0], with zeros; i.e. zero-extending the value.  This is handy when converting encoded or received unsigned values from a small bit width to a large bit with; i.e. 32 bits.

If the WC or WCZ effect is specified, the C flag is set to the result's MSB value.

If the WZ or WCZ effect is specified, the Z flag is set (1) if the result is zero, or is cleared (0) if it is non-zero.

# PROPELLER 2 ASSEMBLY LANGUAGE (PASM2) IN BRIEF

| ###### Math and Logic Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Reg, LUT, & Hub |
| ABS D {WC/WZ/WCZ} | Get absolute value of D into D. D = ABS(D). C = D[31]. * | 2 |
| ABS D,{#}S {WC/WZ/WCZ} | Get absolute value of S into D. D = ABS(S). C = S[31]. * | 2 |
| ADD D,{#}S {WC/WZ/WCZ} | Add S into D. D = D + S. C = carry of (D + S). * | 2 |
| ADDS D,{#}S {WC/WZ/WCZ} | Add S into D, signed. D = D + S. C = correct sign of (D + S). * | 2 |
| ADDSX D,{#}S {WC/WZ/WCZ} | Add (S + C) into D, signed and extended. D = D + S + C. C = correct sign of (D + S + C). Z = Z AND (result == 0). | 2 |
| ADDX D,{#}S {WC/WZ/WCZ} | Add (S + C) into D, extended. D = D + S + C. C = carry of (D + S + C). Z = Z AND (result == 0). | 2 |
| AND D,{#}S {WC/WZ/WCZ} | AND S into D. D = D & S. C = parity of result. * | 2 |
| ANDN D,{#}S {WC/WZ/WCZ} | AND !S into D. D = D & !S. C = parity of result. * | 2 |
| BITC D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = C. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. | 2 |
| BITH D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = 1. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. | 2 |
| BITL D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = 0. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. | 2 |
| BITNC D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = !C. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. | 2 |
| BITNOT D,{#}S {WCZ} | Toggle bits D[S[9:5]+S[4:0]:S[4:0]]. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. | 2 |
| BITNZ D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = !Z. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. | 2 |
| BITRND D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = RNDs. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. | 2 |
| BITZ D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = Z. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. | 2 |
| BMASK D | Get LSB-justified bit mask of size (D[4:0] + 1) into D. D = ($0000_0002 << D[4:0]) - 1. | 2 |
| BMASK D,{#}S | Get LSB-justified bit mask of size (S[4:0] + 1) into D. D = ($0000_0002 << S[4:0]) - 1. | 2 |
| CMP D,{#}S {WC/WZ/WCZ} | Compare D to S. C = borrow of (D - S). Z = (D == S). | 2 |
| CMPM D,{#}S {WC/WZ/WCZ} | Compare D to S, get MSB of difference into C. C = MSB of (D - S). Z = (D == S). | 2 |
| CMPR D,{#}S {WC/WZ/WCZ} | Compare S to D (reverse). C = borrow of (S - D). Z = (D == S). | 2 |
| CMPS D,{#}S {WC/WZ/WCZ} | Compare D to S, signed. C = correct sign of (D - S). Z = (D == S). | 2 |
| CMPSUB D,{#}S {WC/WZ/WCZ} | Compare and subtract S from D if D >= S. If D => S then D = D - S and C = 1, else D same and C = 0. * | 2 |
| CMPSX D,{#}S {WC/WZ/WCZ} | Compare D to (S + C), signed and extended. C = correct sign of (D - (S + C)). Z = Z AND (D == S + C). | 2 |
| CMPX D,{#}S {WC/WZ/WCZ} | Compare D to (S + C), extended. C = borrow of (D - (S + C)). Z = Z AND (D == S + C). | 2 |
| CRCBIT D,{#}S | Iterate CRC value in D using C and polynomial in S. If (C XOR D[0]) then D = (D >> 1) XOR S, else D = (D >> 1). | 2 |
| CRCNIB D,{#}S | Iterate CRC value in D using Q[31:28] and polynomial in S. Like CRCBIT x 4. Q = Q << 4. Use 'REP #n,#1'+SETQ+CRCNIB+CRCNIB+CRCNIB... | 2 |
| DECMOD D,{#}S {WC/WZ/WCZ} | Decrement with modulus. If D = 0 then D = S and C = 1, else D = D - 1 and C = 0. * | 2 |
| DECOD D | Decode D[4:0] into D. D = 1 << D[4:0]. | 2 |
| DECOD D,{#}S | Decode S[4:0] into D. D = 1 << S[4:0]. | 2 |
| ENCOD D {WC/WZ/WCZ} | Get bit position of top-most '1' in D into D. D = position of top '1' in S (0..31). C = (S != 0). * | 2 |
| ENCOD D,{#}S {WC/WZ/WCZ} | Get bit position of top-most '1' in S into D. D = position of top '1' in S (0..31). C = (S != 0). * | 2 |
| FGE D,{#}S {WC/WZ/WCZ} | Force D >= S. If D < S then D = S and C = 1, else D same and C = 0. * | 2 |
| FGES D,{#}S {WC/WZ/WCZ} | Force D >= S, signed. If D < S then D = S and C = 1, else D same and C = 0. * | 2 |
| FLE D,{#}S {WC/WZ/WCZ} | Force D <= S. If D > S then D = S and C = 1, else D same and C = 0. * | 2 |
| FLES D,{#}S {WC/WZ/WCZ} | Force D <= S, signed. If D > S then D = S and C = 1, else D same and C = 0. * | 2 |
| GETBYTE D | Get byte established by prior ALTGB instruction into D. | 2 |
| GETBYTE D,{#}S,#N | Get byte N of S into D. D = {24'b0, S.BYTE[N]). | 2 |
| GETNIB D | Get nibble established by prior ALTGN instruction into D. | 2 |
| GETNIB D,{#}S,#N | Get nibble N of S into D. D = {28'b0, S.NIBBLE[N]). | 2 |
| GETWORD D | Get word established by prior ALTGW instruction into D. | 2 |
| GETWORD D,{#}S,#N | Get word N of S into D. D = {16'b0, S.WORD[N]). | 2 |
| INCMOD D,{#}S {WC/WZ/WCZ} | Increment with modulus. If D = S then D = 0 and C = 1, else D = D + 1 and C = 0. * | 2 |
| LOC PA/PB/PTRA/PTRB,#{\}A | Get {12'b0, address[19:0]} into PA/PB/PTRA/PTRB (per W). If R = 1, address = PC + A, else address = A. "\" forces R = 0. | 2 |
| MERGEB D | Merge bits of bytes in D. D = {D[31], D[23], D[15], D[7], ...D[24], D[16], D[8], D[0]}. | 2 |
| MERGEW D | Merge bits of words in D. D = {D[31], D[15], D[30], D[14], ...D[17], D[1], D[16], D[0]}. | 2 |
| MODC c {WC} | Modify C according to cccc. C = cccc[{C,Z}]. | 2 |
| MODCZ c,z {WC/WZ/WCZ} | Modify C and Z according to cccc and zzzz. C = cccc[{C,Z}], Z = zzzz[{C,Z}]. | 2 |
| MODZ z {WZ} | Modify Z according to zzzz. Z = zzzz[{C,Z}]. | 2 |
| MOV D,{#}S {WC/WZ/WCZ} | Move S into D. D = S. C = S[31]. * | 2 |
| MOVBYTS D,{#}S | Move bytes within D, per S. D = {D.BYTE[S[7:6]], D.BYTE[S[5:4]], D.BYTE[S[3:2]], D.BYTE[S[1:0]]}. | 2 |
| MUL D,{#}S {WZ} | D = unsigned (D[15:0] * S[15:0]). Z = (S == 0) ¦ (D == 0). | 2 |
| MULS D,{#}S {WZ} | D = signed (D[15:0] * S[15:0]). Z = (S == 0) ¦ (D == 0). | 2 |
| MUXC D,{#}S {WC/WZ/WCZ} | Mux C into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{ C}}). C = parity of result. * | 2 |
| MUXNC D,{#}S {WC/WZ/WCZ} | Mux !C into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{!C}}). C = parity of result. * | 2 |
| MUXNIBS D,{#}S | For each non-zero nibble in S, copy that nibble into the corresponding D nibble, else leave that D nibble the same. | 2 |
| MUXNITS D,{#}S | For each non-zero bit pair in S, copy that bit pair into the corresponding D bits, else leave that D bit pair the same. | 2 |
| MUXNZ D,{#}S {WC/WZ/WCZ} | Mux !Z into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{!Z}}). C = parity of result. * | 2 |
| MUXQ D,{#}S | Used after SETQ. For each '1' bit in Q, copy the corresponding bit in S into D. D = (D & !Q) ¦ (S & Q). | 2 |
| MUXZ D,{#}S {WC/WZ/WCZ} | Mux Z into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{ Z}}). C = parity of result. * | 2 |
| NEG D {WC/WZ/WCZ} | Negate D. D = -D. C = MSB of result. * | 2 |
| NEG D,{#}S {WC/WZ/WCZ} | Negate S into D. D = -S. C = MSB of result. * | 2 |
| NEGC D {WC/WZ/WCZ} | Negate D by C. If C = 1 then D = -D, else D = D. C = MSB of result. * | 2 |
| NEGC D,{#}S {WC/WZ/WCZ} | Negate S by C into D. If C = 1 then D = -S, else D = S. C = MSB of result. * | 2 |
| NEGNC D {WC/WZ/WCZ} | Negate D by !C. If C = 0 then D = -D, else D = D. C = MSB of result. * | 2 |
| NEGNC D,{#}S {WC/WZ/WCZ} | Negate S by !C into D. If C = 0 then D = -S, else D = S. C = MSB of result. * | 2 |
| NEGNZ D {WC/WZ/WCZ} | Negate D by !Z. If Z = 0 then D = -D, else D = D. C = MSB of result. * | 2 |
| NEGNZ D,{#}S {WC/WZ/WCZ} | Negate S by !Z into D. If Z = 0 then D = -S, else D = S. C = MSB of result. * | 2 |
| NEGZ D {WC/WZ/WCZ} | Negate D by Z. If Z = 1 then D = -D, else D = D. C = MSB of result. * | 2 |
| NEGZ D,{#}S {WC/WZ/WCZ} | Negate S by Z into D. If Z = 1 then D = -S, else D = S. C = MSB of result. * | 2 |
| NOT D {WC/WZ/WCZ} | Get !D into D. D = !D. C = !D[31]. * | 2 |
| NOT D,{#}S {WC/WZ/WCZ} | Get !S into D. D = !S. C = !S[31]. * | 2 |
| ONES D {WC/WZ/WCZ} | Get number of '1's in D into D. D = number of '1's in S (0..32). C = LSB of result. * | 2 |
| ONES D,{#}S {WC/WZ/WCZ} | Get number of '1's in S into D. D = number of '1's in S (0..32). C = LSB of result. * | 2 |
| OR D,{#}S {WC/WZ/WCZ} | OR S into D. D = D ¦ S. C = parity of result. * | 2 |
| RCL D,{#}S {WC/WZ/WCZ} | Rotate carry left. D = [63:32] of ({D[31:0], {32{C}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. * | 2 |
| RCR D,{#}S {WC/WZ/WCZ} | Rotate carry right. D = [31:0] of ({{32{C}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. * | 2 |
| RCZL D {WC/WZ/WCZ} | Rotate C,Z left through D. D = {D[29:0], C, Z}. C = D[31], Z = D[30]. | 2 |
| RCZR D {WC/WZ/WCZ} | Rotate C,Z right through D. D = {C, Z, D[31:2]}. C = D[1], Z = D[0]. | 2 |
| REV D | Reverse D bits. D = D[0:31]. | 2 |
| RGBEXP D | Expand 5:6:5 RGB value in D[15:0] into 8:8:8 value in D[31:8]. D = {D[15:11,15:13], D[10:5,10:9], D[4:0,4:2], 8'b0}. | 2 |
| RGBSQZ D | Squeeze 8:8:8 RGB value in D[31:8] into 5:6:5 value in D[15:0]. D = {15'b0, D[31:27], D[23:18], D[15:11]}. | 2 |
| ROL D,{#}S {WC/WZ/WCZ} | Rotate left. D = [63:32] of ({D[31:0], D[31:0]} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. * | 2 |
| ROLBYTE D | Rotate-left byte established by prior ALTGB instruction into D. | 2 |
| ROLBYTE D,{#}S,#N | Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]). | 2 |
| ROLNIB D | Rotate-left nibble established by prior ALTGN instruction into D. | 2 |
| ROLNIB D,{#}S,#N | Rotate-left nibble N of S into D. D = {D[27:0], S.NIBBLE[N]). | 2 |
| ROLWORD D | Rotate-left word established by prior ALTGW instruction into D. | 2 |
| ROLWORD D,{#}S,#N | Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]). | 2 |
| ROR D,{#}S {WC/WZ/WCZ} | Rotate right. D = [31:0] of ({D[31:0], D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. * | 2 |
| SAL D,{#}S {WC/WZ/WCZ} | Shift arithmetic left. D = [63:32] of ({D[31:0], {32{D[0]}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. * | 2 |
| SAR D,{#}S {WC/WZ/WCZ} | Shift arithmetic right. D = [31:0] of ({{32{D[31]}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. * | 2 |
| SCA D,{#}S {WZ} | Next instruction's S value = unsigned (D[15:0] * S[15:0]) >> 16. * | 2 |
| SCAS D,{#}S {WZ} | Next instruction's S value = signed (D[15:0] * S[15:0]) >> 14. In this scheme, $4000 = 1.0 and $C000 = -1.0. * | 2 |
| SETBYTE {#}S | Set S[7:0] into byte established by prior ALTSB instruction. | 2 |
| SETBYTE D,{#}S,#N | Set S[7:0] into byte N in D, keeping rest of D same. | 2 |
| SETD D,{#}S | Set D field of D to S[8:0]. D = {D[31:18], S[8:0], D[8:0]}. | 2 |
| SETNIB {#}S | Set S[3:0] into nibble established by prior ALTSN instruction. | 2 |
| SETNIB D,{#}S,#N | Set S[3:0] into nibble N in D, keeping rest of D same. | 2 |
| SETR D,{#}S | Set R field of D to S[8:0]. D = {D[31:28], S[8:0], D[18:0]}. | 2 |
| SETS D,{#}S | Set S field of D to S[8:0]. D = {D[31:9], S[8:0]}. | 2 |
| SETWORD {#}S | Set S[15:0] into word established by prior ALTSW instruction. | 2 |
| SETWORD D,{#}S,#N | Set S[15:0] into word N in D, keeping rest of D same. | 2 |
| SEUSSF D | Relocate and periodically invert bits within D. Returns to original value on 32nd iteration. Forward pattern. | 2 |
| SEUSSR D | Relocate and periodically invert bits within D. Returns to original value on 32nd iteration. Reverse pattern. | 2 |
| SHL D,{#}S {WC/WZ/WCZ} | Shift left. D = [63:32] of ({D[31:0], 32'b0} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. * | 2 |
| SHR D,{#}S {WC/WZ/WCZ} | Shift right. D = [31:0] of ({32'b0, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. * | 2 |
| SIGNX D,{#}S {WC/WZ/WCZ} | Sign-extend D from bit S[4:0]. C = MSB of result. * | 2 |
| SPLITB D | Split every 4th bit of D into bytes. D = {D[31], D[27], D[23], D[19], ...D[12], D[8], D[4], D[0]}. | 2 |
| SPLITW D | Split odd/even bits of D into words. D = {D[31], D[29], D[27], D[25], ...D[6], D[4], D[2], D[0]}. | 2 |
| SUB D,{#}S {WC/WZ/WCZ} | Subtract S from D. D = D - S. C = borrow of (D - S). * | 2 |
| SUBR D,{#}S {WC/WZ/WCZ} | Subtract D from S (reverse). D = S - D. C = borrow of (S - D). * | 2 |
| SUBS D,{#}S {WC/WZ/WCZ} | Subtract S from D, signed. D = D - S. C = correct sign of (D - S). * | 2 |
| SUBSX D,{#}S {WC/WZ/WCZ} | Subtract (S + C) from D, signed and extended. D = D - (S + C). C = correct sign of (D - (S + C)). Z = Z AND (result == 0). | 2 |
| SUBX D,{#}S {WC/WZ/WCZ} | Subtract (S + C) from D, extended. D = D - (S + C). C = borrow of (D - (S + C)). Z = Z AND (result == 0). | 2 |
| SUMC D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by C. If C = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * | 2 |
| SUMNC D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by !C. If C = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * | 2 |
| SUMNZ D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by !Z. If Z = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * | 2 |
| SUMZ D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by Z. If Z = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * | 2 |
| TEST D {WC/WZ/WCZ} | Test D. C = parity of D. Z = (D == 0). | 2 |
| TEST D,{#}S {WC/WZ/WCZ} | Test D with S. C = parity of (D & S). Z = ((D & S) == 0). | 2 |
| TESTB D,{#}S WC/WZ | Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]]. | 2 |
| TESTB D,{#}S ORC/ORZ | Test bit S[4:0] of D, OR into C/Z. C/Z = C/Z OR D[S[4:0]]. | 2 |
| TESTB D,{#}S ANDC/ANDZ | Test bit S[4:0] of D, AND into C/Z. C/Z = C/Z AND D[S[4:0]]. | 2 |
| TESTB D,{#}S XORC/XORZ | Test bit S[4:0] of D, XOR into C/Z. C/Z = C/Z XOR D[S[4:0]]. | 2 |
| TESTBN D,{#}S WC/WZ | Test bit S[4:0] of !D, write to C/Z. C/Z = !D[S[4:0]]. | 2 |
| TESTBN D,{#}S ORC/ORZ | Test bit S[4:0] of !D, OR into C/Z. C/Z = C/Z OR !D[S[4:0]]. | 2 |
| TESTBN D,{#}S ANDC/ANDZ | Test bit S[4:0] of !D, AND into C/Z. C/Z = C/Z AND !D[S[4:0]]. | 2 |
| TESTBN D,{#}S XORC/XORZ | Test bit S[4:0] of !D, XOR into C/Z. C/Z = C/Z XOR !D[S[4:0]]. | 2 |
| TESTN D,{#}S {WC/WZ/WCZ} | Test D with !S. C = parity of (D & !S). Z = ((D & !S) == 0). | 2 |
| WRC D | Write 0 or 1 to D, according to C. D = {31'b0, C). | 2 |
| WRNC D | Write 0 or 1 to D, according to !C. D = {31'b0, !C). | 2 |
| WRNZ D | Write 0 or 1 to D, according to !Z. D = {31'b0, !Z). | 2 |
| WRZ D | Write 0 or 1 to D, according to Z. D = {31'b0, Z). | 2 |
| XOR D,{#}S {WC/WZ/WCZ} | XOR S into D. D = D ^ S. C = parity of result. * | 2 |
| XORO32 D | Iterate D with xoroshiro32+ PRNG algorithm and put PRNG result into next instruction's S. | 2 |
| ZEROX D,{#}S {WC/WZ/WCZ} | Zero-extend D above bit S[4:0]. C = MSB of result. * | 2 |

| ###### Pin & Smart Pin Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Cog, LUT & Hub |
| Pin |  |  |
| DIRC {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = C. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. | 2 |
| DIRH {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = 1. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. | 2 |
| DIRL {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = 0. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. | 2 |
| DIRNC {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = !C. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. | 2 |
| DIRNOT {#}D {WCZ} | Toggle DIR bits of pins D[10:6]+D[5:0]..D[5:0]. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. | 2 |
| DIRNZ {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = !Z. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. | 2 |
| DIRRND {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = RNDs. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. | 2 |
| DIRZ {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = Z. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. | 2 |
| DRVC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = C. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| DRVH {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 1. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| DRVL {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 0. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| DRVNC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !C. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| DRVNOT {#}D {WCZ} | Toggle OUT bits of pins D[10:6]+D[5:0]..D[5:0]. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| DRVNZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !Z. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| DRVRND {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = RNDs. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| DRVZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = Z. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| FLTC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = C. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| FLTH {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 1. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| FLTL {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 0. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| FLTNC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !C. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| FLTNOT {#}D {WCZ} | Toggle OUT bits of pins D[10:6]+D[5:0]..D[5:0]. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| FLTNZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !Z. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| FLTRND {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = RNDs. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| FLTZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = Z. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| OUTC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = C. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| OUTH {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| OUTL {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| OUTNC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !C. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| OUTNOT {#}D {WCZ} | Toggle OUT bits of pins D[10:6]+D[5:0]..D[5:0]. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| OUTNZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !Z. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| OUTRND {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = RNDs. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| OUTZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = Z. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. | 2 |
| TESTP {#}D WC/WZ | Test IN bit of pin D[5:0], write to C/Z. C/Z = IN[D[5:0]]. | 2 |
| TESTP {#}D ORC/ORZ | Test IN bit of pin D[5:0], OR into C/Z. C/Z = C/Z OR IN[D[5:0]]. | 2 |
| TESTP {#}D ANDC/ANDZ | Test IN bit of pin D[5:0], AND into C/Z. C/Z = C/Z AND IN[D[5:0]]. | 2 |
| TESTP {#}D XORC/XORZ | Test IN bit of pin D[5:0], XOR into C/Z. C/Z = C/Z XOR IN[D[5:0]]. | 2 |
| TESTPN {#}D WC/WZ | Test !IN bit of pin D[5:0], write to C/Z. C/Z = !IN[D[5:0]]. | 2 |
| TESTPN {#}D ORC/ORZ | Test !IN bit of pin D[5:0], OR into C/Z. C/Z = C/Z OR !IN[D[5:0]]. | 2 |
| TESTPN {#}D ANDC/ANDZ | Test !IN bit of pin D[5:0], AND into C/Z. C/Z = C/Z AND !IN[D[5:0]]. | 2 |
| TESTPN {#}D XORC/XORZ | Test !IN bit of pin D[5:0], XOR into C/Z. C/Z = C/Z XOR !IN[D[5:0]]. | 2 |
| Smart Pin |  |  |
| AKPIN {#}S | Acknowledge smart pins S[10:6]+S[5:0]..S[5:0]. Wraps within A/B pins. Prior SETQ overrides S[10:6]. | 2 |
| GETSCP D | Get four-channel oscilloscope samples into D. D = {ch3[7:0],ch2[7:0],ch1[7:0],ch0[7:0]}. | 2 |
| RDPIN D,{#}S {WC} | Read smart pin S[5:0] result "Z" into D, acknowledge smart pin. C = modal result. | 2 |
| RQPIN D,{#}S {WC} | Read smart pin S[5:0] result "Z" into D, don't acknowledge smart pin ("Q" in RQPIN means "quiet"). C = modal result. | 2 |
| SETDACS {#}D | DAC3 = D[31:24], DAC2 = D[23:16], DAC1 = D[15:8], DAC0 = D[7:0]. | 2 |
| SETSCP {#}D | Set four-channel oscilloscope enable to D[6] and set input pin base to D[5:2]. | 2 |
| WRPIN {#}D,{#}S | Set mode of smart pins S[10:6]+S[5:0]..S[5:0] to D, acknowledge smart pins. Wraps within A/B pins. Prior SETQ overrides S[10:6]. | 2 |
| WXPIN {#}D,{#}S | Set "X" of smart pins S[10:6]+S[5:0]..S[5:0] to D, acknowledge smart pins. Wraps within A/B pins. Prior SETQ overrides S[10:6]. | 2 |
| WYPIN {#}D,{#}S | Set "Y" of smart pins S[10:6]+S[5:0]..S[5:0] to D, acknowledge smart pins. Wraps within A/B pins. Prior SETQ overrides S[10:6]. | 2 |

| ###### Branch Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Cog & LUT / Hub |
| CALL #{\}A | Call to A by pushing {C, Z, 10'b0, PC[19:0]} onto stack. If R = 1 then PC += A, else PC = A. "\" forces R = 0. | 4 / 13...20 |
| CALL D {WC/WZ/WCZ} | Call to D by pushing {C, Z, 10'b0, PC[19:0]} onto stack. C = D[31], Z = D[30], PC = D[19:0]. | 4 / 13...20 |
| CALLA #{\}A | Call to A by writing {C, Z, 10'b0, PC[19:0]} to hub long at PTRA++. If R = 1 then PC += A, else PC = A. "\" forces R = 0. | 5...12 1 / 14...32 1 |
| CALLA D {WC/WZ/WCZ} | Call to D by writing {C, Z, 10'b0, PC[19:0]} to hub long at PTRA++. C = D[31], Z = D[30], PC = D[19:0]. | 5...12 1 / 14...32 1 |
| CALLB #{\}A | Call to A by writing {C, Z, 10'b0, PC[19:0]} to hub long at PTRB++. If R = 1 then PC += A, else PC = A. "\" forces R = 0. | 5...12 1 / 14...32 1 |
| CALLB D {WC/WZ/WCZ} | Call to D by writing {C, Z, 10'b0, PC[19:0]} to hub long at PTRB++. C = D[31], Z = D[30], PC = D[19:0]. | 5...12 1 / 14...32 1 |
| CALLD D,{#}S {WC/WZ/WCZ} | Call to S** by writing {C, Z, 10'b0, PC[19:0]} to D. C = S[31], Z = S[30]. | 4 / 13...20 |
| CALLD PA/PB/PTRA/PTRB,#{\}A | Call to A by writing {C, Z, 10'b0, PC[19:0]} to PA/PB/PTRA/PTRB (per W). If R = 1 then PC += A, else PC = A. "\" forces R = 0. | 4 / 13...20 |
| CALLPA {#}D,{#}S | Call to S** by pushing {C, Z, 10'b0, PC[19:0]} onto stack, copy D to PA. | 4 / 13...20 |
| CALLPB {#}D,{#}S | Call to S** by pushing {C, Z, 10'b0, PC[19:0]} onto stack, copy D to PB. | 4 / 13...20 |
| DJF D,{#}S | Decrement D and jump to S** if result is $FFFF_FFFF. | 2 or 4 / 2 or 13...20 |
| DJNF D,{#}S | Decrement D and jump to S** if result is not $FFFF_FFFF. | 2 or 4 / 2 or 13...20 |
| DJNZ D,{#}S | Decrement D and jump to S** if result is not zero. | 2 or 4 / 2 or 13...20 |
| DJZ D,{#}S | Decrement D and jump to S** if result is zero. | 2 or 4 / 2 or 13...20 |
| EXECF {#}D | Jump to D[9:0] in cog/LUT and set SKIPF pattern to D[31:10]. PC = {10'b0, D[9:0]}. | 4 / 4 |
| IJNZ D,{#}S | Increment D and jump to S** if result is not zero. | 2 or 4 / 2 or 13...20 |
| IJZ D,{#}S | Increment D and jump to S** if result is zero. | 2 or 4 / 2 or 13...20 |
| JMP #{\}A | Jump to A. If R = 1 then PC += A, else PC = A. "\" forces R = 0. | 4 / 13...20 |
| JMP D {WC/WZ/WCZ} | Jump to D. C = D[31], Z = D[30], PC = D[19:0]. | 4 / 13...20 |
| JMPREL {#}D | Jump ahead/back by D instructions. For cogex, PC += D[19:0]. For hubex, PC += D[17:0] << 2. | 4 / 13...20 |
| REP {#}D,{#}S | Execute next D[8:0] instructions S times. If S = 0, repeat instructions infinitely. If D[8:0] = 0, nothing repeats. | 2 / 2 |
| RESI0 | Resume from INT0. (CALLD $1FE,$1FF WCZ) | 4 / 13...20 |
| RESI1 | Resume from INT1. (CALLD $1F4,$1F5 WCZ) | 4 / 13...20 |
| RESI2 | Resume from INT2. (CALLD $1F2,$1F3 WCZ) | 4 / 13...20 |
| RESI3 | Resume from INT3. (CALLD $1F0,$1F1 WCZ) | 4 / 13...20 |
| RET {WC/WZ/WCZ} | Return by popping stack (K). C = K[31], Z = K[30], PC = K[19:0]. | 4 / 13...20 |
| RETA {WC/WZ/WCZ} | Return by reading hub long (L) at --PTRA. C = L[31], Z = L[30], PC = L[19:0]. | 11...18 1 / 20...40 1 |
| RETB {WC/WZ/WCZ} | Return by reading hub long (L) at --PTRB. C = L[31], Z = L[30], PC = L[19:0]. | 11...18 1 / 20...40 1 |
| RETI0 | Return from INT0. (CALLD $1FF,$1FF WCZ) | 4 / 13...20 |
| RETI1 | Return from INT1. (CALLD $1FF,$1F5 WCZ) | 4 / 13...20 |
| RETI2 | Return from INT2. (CALLD $1FF,$1F3 WCZ) | 4 / 13...20 |
| RETI3 | Return from INT3. (CALLD $1FF,$1F1 WCZ) | 4 / 13...20 |
| SKIP {#}D | Skip instructions per D. Subsequent instructions 0..31 get cancelled for each '1' bit in D[0]..D[31]. | 2 / 2 |
| SKIPF {#}D | Skip cog/LUT instructions fast per D. Like SKIP, but instead of cancelling instructions, the PC leaps over them. | 2 / ILLEGAL |
| TJF D,{#}S | Test D and jump to S** if D is full (D = $FFFF_FFFF). | 2 or 4 / 2 or 13...20 |
| TJNF D,{#}S | Test D and jump to S** if D is not full (D != $FFFF_FFFF). | 2 or 4 / 2 or 13...20 |
| TJNS D,{#}S | Test D and jump to S** if D is not signed (D[31] = 0). | 2 or 4 / 2 or 13...20 |
| TJNZ D,{#}S | Test D and jump to S** if D is not zero. | 2 or 4 / 2 or 13...20 |
| TJS D,{#}S | Test D and jump to S** if D is signed (D[31] = 1). | 2 or 4 / 2 or 13...20 |
| TJV D,{#}S | Test D and jump to S** if D overflowed (D[31] != C, C = 'correct sign' from last addition/subtraction). | 2 or 4 / 2 or 13...20 |
| TJZ D,{#}S | Test D and jump to S** if D is zero. | 2 or 4 / 2 or 13...20 |

1 +1 if crosses hub long

| ###### Hub Control, FIFO, & RAM Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Cog & LUT / Hub |
| Hub Control |  |  |
| COGID {#}D {WC} | If D is register and no WC, get cog ID (0 to 15) into D. If WC, check status of cog D[3:0], C = 1 if on. | 2...9, +2 if result / same |
| COGINIT {#}D,{#}S {WC} | Start cog selected by D. S[19:0] sets hub startup address and PTRB of cog. Prior SETQ sets PTRA of cog. | 2...9, +2 if result / same |
| COGSTOP {#}D | Stop cog D[3:0]. | 2...9 / same |
| LOCKNEW D {WC} | Request a LOCK. D will be written with the LOCK number (0 to 15). C = 1 if no LOCK available. | 4...11 / same |
| LOCKREL {#}D {WC} | Release LOCK D[3:0]. If D is a register and WC, get current/last cog id of LOCK owner into D and LOCK status into C. | 2...9, +2 if result / same |
| LOCKRET {#}D | Return LOCK D[3:0] for reallocation. | 2...9 / same |
| LOCKTRY {#}D {WC} | Try to get LOCK D[3:0]. C = 1 if got LOCK. LOCKREL releases LOCK. LOCK is also released if owner cog stops or restarts. | 2...9, +2 if result / same |
| HUBSET {#}D | Set hub configuration to D. | 2...9 / same |
| Hub FIFO |  |  |
| GETPTR D | Get current FIFO hub pointer into D. | 2 / FIFO IN USE |
| FBLOCK {#}D,{#}S | Set next block for when block wraps. D[13:0] = block size in 64-byte units (0 = max), S[19:0] = block start address. | 2 / FIFO IN USE |
| RDFAST {#}D,{#}S | Begin new fast hub read via FIFO. D[31] = no wait, D[13:0] = block size in 64-byte units (0 = max), S[19:0] = block start address. | 2 or WRFAST finish + 10...17 / FIFO IN USE |
| WRFAST {#}D,{#}S | Begin new fast hub write via FIFO. D[31] = no wait, D[13:0] = block size in 64-byte units (0 = max), S[19:0] = block start address. | 2 or WRFAST finish + 3 / FIFO IN USE |
| RFBYTE D {WC/WZ/WCZ} | Used after RDFAST. Read zero-extended byte from FIFO into D. C = MSB of byte. * | 2 / FIFO IN USE |
| RFLONG D {WC/WZ/WCZ} | Used after RDFAST. Read long from FIFO into D. C = MSB of long. * | 2 / FIFO IN USE |
| RFVAR D {WC/WZ/WCZ} | Used after RDFAST. Read zero-extended 1..4-byte value from FIFO into D. C = 0. * | 2 / FIFO IN USE |
| RFVARS D {WC/WZ/WCZ} | Used after RDFAST. Read sign-extended 1..4-byte value from FIFO into D. C = MSB of value. * | 2 / FIFO IN USE |
| RFWORD D {WC/WZ/WCZ} | Used after RDFAST. Read zero-extended word from FIFO into D. C = MSB of word. * | 2 / FIFO IN USE |
| WFBYTE {#}D | Used after WRFAST. Write byte in D[7:0] into FIFO. | 2 / FIFO IN USE |
| WFLONG {#}D | Used after WRFAST. Write long in D[31:0] into FIFO. | 2 / FIFO IN USE |
| WFWORD {#}D | Used after WRFAST. Write word in D[15:0] into FIFO. | 2 / FIFO IN USE |
| Hub RAM |  |  |
| POPA D {WC/WZ/WCZ} | Read long from hub address --PTRA into D. C = MSB of long. * | 9...16 1 / 9...26 1 |
| POPB D {WC/WZ/WCZ} | Read long from hub address --PTRB into D. C = MSB of long. * | 9...16 1 / 9...26 1 |
| RDBYTE D,{#}S/P {WC/WZ/WCZ} | Read zero-extended byte from hub address {#}S/PTRx into D. C = MSB of byte. * | 9...16 / 9...26 |
| RDLONG D,{#}S/P {WC/WZ/WCZ} | Read long from hub address {#}S/PTRx into D. C = MSB of long. * Prior SETQ/SETQ2 invokes cog/LUT block transfer. | 9...16 1 / 9...26 1 |
| RDWORD D,{#}S/P {WC/WZ/WCZ} | Read zero-extended word from hub address {#}S/PTRx into D. C = MSB of word. * | 9...16 1 / 9...26 1 |
| PUSHA {#}D | Write long in D[31:0] to hub address PTRA++. | 3...10 1 / 3...20 1 |
| PUSHB {#}D | Write long in D[31:0] to hub address PTRB++. | 3...10 1 / 3...20 1 |
| WMLONG D,{#}S/P | Write only non-$00 bytes in D[31:0] to hub address {#}S/PTRx. Prior SETQ/SETQ2 invokes cog/LUT block transfer. | 3...10 1 / 3...20 1 |
| WRBYTE {#}D,{#}S/P | Write byte in D[7:0] to hub address {#}S/PTRx. | 3...10 / 3...20 |
| WRLONG {#}D,{#}S/P | Write long in D[31:0] to hub address {#}S/PTRx. Prior SETQ/SETQ2 invokes cog/LUT block transfer. | 3...10 1 / 3...20 1 |
| WRWORD {#}D,{#}S/P | Write word in D[15:0] to hub address {#}S/PTRx. | 3...10 1 / 3...20 1 |

1 +1 if crosses hub long

| ###### Event Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Cog & LUT / Hub |
| ADDCT1 D,{#}S | Set CT1 event to trigger on CT = D + S. Adds S into D. | 2 |
| ADDCT2 D,{#}S | Set CT2 event to trigger on CT = D + S. Adds S into D. | 2 |
| ADDCT3 D,{#}S | Set CT3 event to trigger on CT = D + S. Adds S into D. | 2 |
| COGATN {#}D | Strobe "attention" of all cogs whose corresponding bits are high in D[15:0]. | 2 |
| JATN {#}S | Jump to S** if ATN event flag is set. | 2 or 4 / 2 or 13...20 |
| JCT1 {#}S | Jump to S** if CT1 event flag is set. | 2 or 4 / 2 or 13...20 |
| JCT2 {#}S | Jump to S** if CT2 event flag is set. | 2 or 4 / 2 or 13...20 |
| JCT3 {#}S | Jump to S** if CT3 event flag is set. | 2 or 4 / 2 or 13...20 |
| JFBW {#}S | Jump to S** if FBW event flag is set. | 2 or 4 / 2 or 13...20 |
| JINT {#}S | Jump to S** if INT event flag is set. | 2 or 4 / 2 or 13...20 |
| JNATN {#}S | Jump to S** if ATN event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNCT1 {#}S | Jump to S** if CT1 event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNCT2 {#}S | Jump to S** if CT2 event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNCT3 {#}S | Jump to S** if CT3 event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNFBW {#}S | Jump to S** if FBW event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNINT {#}S | Jump to S** if INT event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNPAT {#}S | Jump to S** if PAT event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNQMT {#}S | Jump to S** if QMT event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNSE1 {#}S | Jump to S** if SE1 event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNSE2 {#}S | Jump to S** if SE2 event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNSE3 {#}S | Jump to S** if SE3 event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNSE4 {#}S | Jump to S** if SE4 event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNXFI {#}S | Jump to S** if XFI event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNXMT {#}S | Jump to S** if XMT event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNXRL {#}S | Jump to S** if XRL event flag is clear. | 2 or 4 / 2 or 13...20 |
| JNXRO {#}S | Jump to S** if XRO event flag is clear. | 2 or 4 / 2 or 13...20 |
| JPAT {#}S | Jump to S** if PAT event flag is set. | 2 or 4 / 2 or 13...20 |
| JQMT {#}S | Jump to S** if QMT event flag is set. | 2 or 4 / 2 or 13...20 |
| JSE1 {#}S | Jump to S** if SE1 event flag is set. | 2 or 4 / 2 or 13...20 |
| JSE2 {#}S | Jump to S** if SE2 event flag is set. | 2 or 4 / 2 or 13...20 |
| JSE3 {#}S | Jump to S** if SE3 event flag is set. | 2 or 4 / 2 or 13...20 |
| JSE4 {#}S | Jump to S** if SE4 event flag is set. | 2 or 4 / 2 or 13...20 |
| JXFI {#}S | Jump to S** if XFI event flag is set. | 2 or 4 / 2 or 13...20 |
| JXMT {#}S | Jump to S** if XMT event flag is set. | 2 or 4 / 2 or 13...20 |
| JXRL {#}S | Jump to S** if XRL event flag is set. | 2 or 4 / 2 or 13...20 |
| JXRO {#}S | Jump to S** if XRO event flag is set. | 2 or 4 / 2 or 13...20 |
| POLLATN {WC/WZ/WCZ} | Get ATN event flag into C/Z, then clear it. | 2 |
| POLLCT1 {WC/WZ/WCZ} | Get CT1 event flag into C/Z, then clear it. | 2 |
| POLLCT2 {WC/WZ/WCZ} | Get CT2 event flag into C/Z, then clear it. | 2 |
| POLLCT3 {WC/WZ/WCZ} | Get CT3 event flag into C/Z, then clear it. | 2 |
| POLLFBW {WC/WZ/WCZ} | Get FBW event flag into C/Z, then clear it. | 2 |
| POLLINT {WC/WZ/WCZ} | Get INT event flag into C/Z, then clear it. | 2 |
| POLLPAT {WC/WZ/WCZ} | Get PAT event flag into C/Z, then clear it. | 2 |
| POLLQMT {WC/WZ/WCZ} | Get QMT event flag into C/Z, then clear it. | 2 |
| POLLSE1 {WC/WZ/WCZ} | Get SE1 event flag into C/Z, then clear it. | 2 |
| POLLSE2 {WC/WZ/WCZ} | Get SE2 event flag into C/Z, then clear it. | 2 |
| POLLSE3 {WC/WZ/WCZ} | Get SE3 event flag into C/Z, then clear it. | 2 |
| POLLSE4 {WC/WZ/WCZ} | Get SE4 event flag into C/Z, then clear it. | 2 |
| POLLXFI {WC/WZ/WCZ} | Get XFI event flag into C/Z, then clear it. | 2 |
| POLLXMT {WC/WZ/WCZ} | Get XMT event flag into C/Z, then clear it. | 2 |
| POLLXRL {WC/WZ/WCZ} | Get XRL event flag into C/Z, then clear it. | 2 |
| POLLXRO {WC/WZ/WCZ} | Get XRO event flag into C/Z, then clear it. | 2 |
| SETPAT {#}D,{#}S | Set pin pattern for PAT event. C selects INA/INB, Z selects =/!=, D provides mask value, S provides match value. | 2 |
| SETSE1 {#}D | Set SE1 event configuration to D[8:0]. | 2 |
| SETSE2 {#}D | Set SE2 event configuration to D[8:0]. | 2 |
| SETSE3 {#}D | Set SE3 event configuration to D[8:0]. | 2 |
| SETSE4 {#}D | Set SE4 event configuration to D[8:0]. | 2 |
| WAITATN {WC/WZ/WCZ} | Wait for ATN event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITCT1 {WC/WZ/WCZ} | Wait for CT1 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITCT2 {WC/WZ/WCZ} | Wait for CT2 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITCT3 {WC/WZ/WCZ} | Wait for CT3 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITFBW {WC/WZ/WCZ} | Wait for FBW event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITINT {WC/WZ/WCZ} | Wait for INT event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITPAT {WC/WZ/WCZ} | Wait for PAT event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITSE1 {WC/WZ/WCZ} | Wait for SE1 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITSE2 {WC/WZ/WCZ} | Wait for SE2 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITSE3 {WC/WZ/WCZ} | Wait for SE3 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITSE4 {WC/WZ/WCZ} | Wait for SE4 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITXFI {WC/WZ/WCZ} | Wait for XFI event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITXMT {WC/WZ/WCZ} | Wait for XMT event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITXRL {WC/WZ/WCZ} | Wait for XRL event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |
| WAITXRO {WC/WZ/WCZ} | Wait for XRO event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. | 2+ |

| ###### Interrupt Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Cog, LUT & Hub |
| ALLOWI | Allow interrupts (default). | 2 |
| BRK {#}D | If in debug ISR, set next break condition to D. Else, set BRK code to D[7:0] and unconditionally trigger BRK interrupt, if enabled. | 2 |
| COGBRK {#}D | If in debug ISR, trigger asynchronous breakpoint in cog D[3:0]. Cog D[3:0] must have asynchronous breakpoint enabled. | 2 |
| GETBRK D WC/WZ/WCZ | Get breakpoint/cog status into D according to WC/WZ/WCZ. See documentation for details. | 2 |
| NIXINT1 | Cancel INT1. | 2 |
| NIXINT2 | Cancel INT2. | 2 |
| NIXINT3 | Cancel INT3. | 2 |
| SETINT1 {#}D | Set INT1 source to D[3:0]. | 2 |
| SETINT2 {#}D | Set INT2 source to D[3:0]. | 2 |
| SETINT3 {#}D | Set INT3 source to D[3:0]. | 2 |
| STALLI | Stall Interrupts. | 2 |
| TRGINT1 | Trigger INT1, regardless of STALLI mode. | 2 |
| TRGINT2 | Trigger INT2, regardless of STALLI mode. | 2 |
| TRGINT3 | Trigger INT3, regardless of STALLI mode. | 2 |

| ###### Register Indirection Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Cog & LUT / Hub |
| ALTB D,{#}S | Alter D field of next instruction to D[13:5]. | 2 |
| ALTB D,{#}S | Alter D field of next instruction to (D[13:5] + S) & $1FF. D += sign-extended S[17:9]. | 2 |
| ALTD D | Alter D field of next instruction to D[8:0]. | 2 |
| ALTD D,{#}S | Alter D field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. | 2 |
| ALTGB D | Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = D[10:2], N field = D[1:0]. | 2 |
| ALTGB D,{#}S | Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9]. | 2 |
| ALTGN D | Alter subsequent GETNIB/ROLNIB instruction. Next S field = D[11:3], N field = D[2:0]. | 2 |
| ALTGN D,{#}S | Alter subsequent GETNIB/ROLNIB instruction. Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9]. | 2 |
| ALTGW D | Alter subsequent GETWORD/ROLWORD instruction. Next S field = D[9:1], N field = D[0]. | 2 |
| ALTGW D,{#}S | Alter subsequent GETWORD/ROLWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9]. | 2 |
| ALTI D | Execute D in place of next instruction. D stays same. | 2 |
| ALTI D,{#}S | Substitute next instruction's I/R/D/S fields with fields from D, per S. Modify D per S. | 2 |
| ALTR D | Alter result register address (normally D field) of next instruction to D[8:0]. | 2 |
| ALTR D,{#}S | Alter result register address (normally D field) of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. | 2 |
| ALTS D | Alter S field of next instruction to D[8:0]. | 2 |
| ALTS D,{#}S | Alter S field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. | 2 |
| ALTSB D | Alter subsequent SETBYTE instruction. Next D field = D[10:2], N field = D[1:0]. | 2 |
| ALTSB D,{#}S | Alter subsequent SETBYTE instruction. Next D field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9]. | 2 |
| ALTSN D | Alter subsequent SETNIB instruction. Next D field = D[11:3], N field = D[2:0]. | 2 |
| ALTSN D,{#}S | Alter subsequent SETNIB instruction. Next D field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9]. | 2 |
| ALTSW D | Alter subsequent SETWORD instruction. Next D field = D[9:1], N field = D[0]. | 2 |
| ALTSW D,{#}S | Alter subsequent SETWORD instruction. Next D field = (D[9:1] + S) & $1FF, N field = D[0]. D += sign-extended S[17:9]. | 2 |

| ###### CORDIC Solver Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Cog, LUT & Hub |
| GETQX D {WC/WZ/WCZ} | Retrieve CORDIC result X into D. Waits, in case result not ready. C = X[31]. 1 | 2...58 |
| GETQY D {WC/WZ/WCZ} | Retrieve CORDIC result Y into D. Waits, in case result not ready. C = Y[31]. 1 | 2...58 |
| QDIV {#}D,{#}S | Begin CORDIC unsigned division of {SETQ value or 32'b0, D} / S. GETQX/GETQY retrieves quotient/remainder. | 2...9 |
| QEXP {#}D | Begin CORDIC logarithm-to-number conversion of D. GETQX retrieves number. | 2...9 |
| QFRAC {#}D,{#}S | Begin CORDIC unsigned division of {D, SETQ value or 32'b0} / S. GETQX/GETQY retrieves quotient/remainder. | 2...9 |
| QLOG {#}D | Begin CORDIC number-to-logarithm conversion of D. GETQX retrieves log {5'whole_exponent, 27'fractional_exponent}. | 2...9 |
| QMUL {#}D,{#}S | Begin CORDIC unsigned multiplication of D * S. GETQX/GETQY retrieves lower/upper product. | 2...9 |
| QROTATE {#}D,{#}S | Begin CORDIC rotation of point (D, SETQ value or 32'b0) by angle S. GETQX/GETQY retrieves X/Y. | 2...9 |
| QSQRT {#}D,{#}S | Begin CORDIC square root of {S, D}. GETQX retrieves root. | 2...9 |
| QVECTOR {#}D,{#}S | Begin CORDIC vectoring of point (D, S). GETQX/GETQY retrieves length/angle. | 2...9 |

1 Z = (result == 0)

| ###### Color Space Converter and Pixel Mixer Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Cog, LUT & Hub |
| Color Space Converter |  |  |
| SETCFRQ {#}D | Set the colorspace converter "CFRQ" parameter to D[31:0]. | 2 |
| SETCI {#}D | Set the colorspace converter "CI" parameter to D[31:0]. | 2 |
| SETCMOD {#}D | Set the colorspace converter "CMOD" parameter to D[8:0]. | 2 |
| SETCQ {#}D | Set the colorspace converter "CQ" parameter to D[31:0]. | 2 |
| SETCY {#}D | Set the colorspace converter "CY" parameter to D[31:0]. | 2 |
| Pixel Mixer |  |  |
| ADDPIX D,{#}S | Add bytes of S into bytes of D, with $FF saturation. | 7 |
| BLNPIX D,{#}S | Alpha-blend bytes of S into bytes of D, using SETPIV value. | 7 |
| MIXPIX D,{#}S | Mix bytes of S into bytes of D, using SETPIX and SETPIV values. | 7 |
| MULPIX D,{#}S | Multiply bytes of S into bytes of D, where $FF = 1.0 and $00 = 0.0. | 7 |
| SETPIV {#}D | Set BLNPIX/MIXPIX blend factor to D[7:0]. | 2 |
| SETPIX {#}D | Set MIXPIX mode to D[5:0]. | 2 |

| ###### Lookup Table, Streamer, and Misc Instructions |  |  |
| --- | --- | --- |
| Instruction | Description | Clocks Cog & LUT / Hub |
| Lookup Table |  |  |
| RDLUT D,{#}S/P {WC/WZ/WCZ} | Read data from LUT address {#}S/PTRx into D. C = MSB of data. * | 3 |
| SETLUTS {#}D | If D[0] = 1 then enable LUT sharing, where LUT writes within the adjacent odd/even companion cog are copied to this cog's LUT. | 2 |
| WRLUT {#}D,{#}S/P | Write D to LUT address {#}S/PTRx. | 2 |
| Streamer |  |  |
| GETXACC D | Get the streamer's Goertzel X accumulator into D and the Y accumulator into the next instruction's S, clear accumulators. | 2 |
| SETXFRQ {#}D | Set streamer NCO frequency to D. | 2 |
| XCONT {#}D,{#}S | Buffer new streamer command to be issued on final NCO rollover of current command, continuing phase. | 2+ |
| XINIT {#}D,{#}S | Issue streamer command immediately, zeroing phase. | 2 |
| XSTOP | Stop streamer immediately. | 2 |
| XZERO {#}D,{#}S | Buffer new streamer command to be issued on final NCO rollover of current command, zeroing phase. | 2+ |
| Miscellaneous |  |  |
| AUGD #n | Queue #n to be used as upper 23 bits for next #D occurrence, so that the next 9-bit #D will be augmented to 32 bits. | 2 |
| AUGS #n | Queue #n to be used as upper 23 bits for next #S occurrence, so that the next 9-bit #S will be augmented to 32 bits. | 2 |
| GETCT D {WC} | Get CT[31:0] or CT[63:32] if WC into D. GETCT WC + GETCT gets full CT. CT=0 on reset, CT++ on every clock. C = same. | 2 |
| GETRND WC/WZ/WCZ | Get RND into C/Z. C = RND[31], Z = RND[30], unique per cog. | 2 |
| GETRND D {WC/WZ/WCZ} | Get RND into D/C/Z. RND is the PRNG that updates on every clock. D = RND[31:0], C = RND[31], Z = RND[30], unique per cog. | 2 |
| NOP | No operation. | 2 |
| POP D {WC/WZ/WCZ} | Pop stack (K). D = K. C = K[31]. * | 2 |
| PUSH {#}D | Push D onto stack. | 2 |
| SETQ {#}D | Set Q to D. Use before RDLONG/WRLONG/WMLONG to set block transfer. Also used before MUXQ/COGINIT/QDIV/QFRAC/QROTATE/WAITxxx. | 2 |
| SETQ2 {#}D | Set Q to D. Use before RDLONG/WRLONG/WMLONG to set LUT block transfer. | 2 |
| WAITX {#}D {WC/WZ/WCZ} | Wait 2 + D clocks if no WC/WZ/WCZ. If WC/WZ/WCZ, wait 2 + (D & RND) clocks. C/Z = 0. | 2 + D |

# PROPELLER 2 RESERVED WORDS (SPIN2 + PASM2)

Predefined symbols recognized by the compiler to have special meaning.

## _ (leading underscore)

| _C _C_AND_NZ _C_AND_Z _C_EQ_Z | _C_NE_Z _C_OR_NZ _C_OR_Z _CLR | _E _GE _GT _LE | _LT _NC _NC_AND_NZ _NC_AND_Z | _NC_OR_NZ _NC_OR_Z _NE _NZ | _NZ_AND_C _NZ_AND_NC _NZ_OR_C _NZ_OR_NC | _RET_ _SET _Z _Z_AND_C | _Z_AND_NC _Z_EQ_C _Z_NE_C _Z_OR_C | _Z_OR_NC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

## A - B

| ABORT ABS ADD ADDBITS ADDCT1 | ADDCT2 ADDCT3 ADDPINS ADDPIX ADDS | ADDSX ADDX AKPIN ALIGNL ALIGNW | ALLOWI ALT ALTB ALTD ALTGB | ALTGN ALTGW ALTI ALTR ALTS | ALTSB ALTSN ALTSW AND ANDC | ANDN ANDZ ARCHIVE ASMCLK AUGD | AUGS BACKCOLOR BITC BITH BITL | BITMAP BITNC BITNOT BITNZ BITRND | BITZ BLACK BLNPIX BLUE BMASK | BOX BRK BYTE BYTEFILL BYTEMOVE | BYTES_1BIT BYTES_2BIT BYTES_4BIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## C - D

| CALL CALLA CALLB CALLD CALLPA CALLPB CARTESIAN CASE CASE_FAST CHANNEL | CIRCLE CLEAR CLKFREQ CLKMODE CLKSET CLOSE CMP CMPM CMPR CMPS | CMPSUB CMPSX CMPX COGATN COGBRK COGCHK COGEXEC COGEXEC_NEW COGEXEC_NEW_PAIR COGID | COGINIT COGSPIN COGSTOP COLOR CON CRCBIT CRCNIB CYAN DAT DEBUG | DEBUG_BAUD DEBUG_COGS DEBUG_DELAY DEBUG_DISPLAY_LEFT DEBUG_DISPLAY_TOP DEBUG_HEIGHT DEBUG_LEFT DEBUG_LOG_SIZE DEBUG_PIN DEBUG_TIMESTAMP | DEBUG_TOP DEBUG_WIDTH DEBUG_WINDOWS_OFF DECMOD DECOD DEPTH DEV DIRA DIRB DIRC | DIRH DIRL DIRNC DIRNOT DIRNZ DIRRND DIRZ DJF DJNF DJNZ | DJZ DLY DOT DOTSIZE DRVC DRVH DRVL DRVNC DRVNOT | DRVRND DRVZ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

## E - F

| ELSE ELSEIF ELSEIFNOT ENCOD END | EVENT_ATN EVENT_CT1 EVENT_CT2 EVENT_CT3 EVENT_FBW | EVENT_INT EVENT_PAT EVENT_QMT EVENT_SE1 EVENT_SE2 | EVENT_SE3 EVENT_SE4 EVENT_XFI EVENT_XMT EVENT_XRL | EVENT_XRO EXECF FABS FALSE FBLOCK | FDEC FDEC_ FDEC_ARRAY FDEC_ARRAY_ FDEC_REG_ARRAY | FDEC_REG_ARRAY_ FFT FGE FGES FILE | FIT FLE FLES FLOAT FLTC | FLTH FLTL FLTNC FLTNOT FLTNZ | FLTRND FLTZ FRAC FROM FSQRT | FVAR FVARS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## G - H

| GETBRK GETBYTE GETCT | GETMS GETNIB GETPTR | GETQX GETQY GETREGS | GETRND GETSCP GETSEC | GETWORD GETXACC GREEN | GREY HIDEXY HOLDOFF | HSV16 HSV16W HSV16X | HSV8 HSV8W HSV8X | HUBEXEC HUBEXEC_NEW HUBEXEC_NEW_PAIR | HUBSET |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## I - J

| IF IF_00 IF_0000 IF_0001 IF_0010 IF_0011 IF_01 IF_0100 IF_0101 IF_0110 IF_0111 IF_0X | IF_10 IF_1000 IF_1001 IF_1010 IF_1011 IF_11 IF_1100 IF_1101 IF_1110 IF_1111 IF_1X IF_A | IF_AE IF_ALWAYS IF_B IF_BE IF_C IF_C_AND_NZ IF_C_AND_Z IF_C_EQ_Z IF_C_NE_Z IF_C_OR_NZ IF_C_OR_Z IF_DIFF | IF_E IF_GE IF_GT IF_LE IF_LT IF_NC IF_NC_AND_NZ IF_NC_AND_Z IF_NC_OR_NZ IF_NC_OR_Z IF_NE IF_NOT_00 | IF_NOT_01 IF_NOT_10 IF_NOT_11 IF_NZ IF_NZ_AND_C IF_NZ_AND_NC IF_NZ_OR_C IF_NZ_OR_NC IF_SAME IF_X0 IF_X1 IF_Z | IF_Z_AND_C IF_Z_AND_NC IF_Z_EQ_C IF_Z_NE_C IF_Z_OR_C IF_Z_OR_NC IFNOT IJMP1 IJMP2 IJMP3 IJNZ IJZ | INA INB INCMOD INT_OFF IRET1 IRET2 IRET3 JATN JCT1 JCT2 JCT3 JFBW | JINT JMP JMPREL JNATN JNCT1 JNCT2 JNCT3 JNFBW JNINT JNPAT JNQMT JNSE1 | JNSE2 JNSE3 JNSE4 JNXFI JNXMT JNXRL JNXRO JPAT JQMT JSE1 JSE2 JSE3 | JSE4 JXFI JXMT JXRL JXRO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## L - M

| LINE LINESIZE LOC LOCKCHK LOCKNEW | LOCKREL LOCKRET LOCKTRY LOGIC LOGSCALE | LONG LONGFILL LONGMOVE LONGS_16BIT LONGS_1BIT | LONGS_2BIT LONGS_4BIT LONGS_8BIT LOOKDOWN LOOKDOWNZ | LOOKUP LOOKUPZ LSTR LSTR_ LUMA8 | LUMA8W LUMA8X LUT1 LUT2 LUT4 | LUT8 LUTCOLORS MAG MAGENTA MERGEB | MERGEW MIDI MIXPIX MODC MODCZ | MODZ MOV MOVBYTS MUL MULDIV64 | MULPIX MULS MUXC MUXNC MUXNIBS | MUXNITS MUXNZ MUXQ MUXZ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## N - O

| NAN NEG NEGC NEGNC | NEGNZ NEGX NEGZ NEWCOG | NEXT NIXINT1 NIXINT2 NIXINT3 | NOP NOT OBJ OBOX | ONES OPACITY OR ORANGE | ORC ORG ORGF ORGH | ORIGIN ORZ OTHER OUTA | OUTB OUTC OUTH OUTL | OUTNC OUTNOT OUTNZ OUTRND | OUTZ OVAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## P

| P_ADC P_ADC_100X P_ADC_10X P_ADC_1X P_ADC_30X P_ADC_3X P_ADC_EXT P_ADC_FLOAT P_ADC_GIO P_ADC_SCOPE P_ADC_VIO P_AND_AB P_ASYNC_IO P_ASYNC_RX P_ASYNC_TX P_BITDAC P_CHANNEL P_COMPARE_AB P_COMPARE_AB_FB P_COUNT_HIGHS | P_COUNT_RISES P_COUNTER_HIGHS P_COUNTER_PERIODS P_COUNTER_TICKS P_DAC_124R_3V P_DAC_600R_2V P_DAC_75R_2V P_DAC_990R_3V P_DAC_DITHER_PWM P_DAC_DITHER_RND P_DAC_NOISE P_EVENTS_TICKS P_FILT0_AB P_FILT1_AB P_FILT2_AB P_FILT3_AB P_HIGH_100UA P_HIGH_10UA P_HIGH_150K P_HIGH_15K | P_HIGH_1K5 P_HIGH_1MA P_HIGH_FAST P_HIGH_FLOAT P_HIGH_TICKS P_INVERT_A P_INVERT_B P_INVERT_IN P_INVERT_OUT P_INVERT_OUTPUT P_LEVEL_A P_LEVEL_A_FBN P_LEVEL_B_FBN P_LEVEL_B_FBP P_LOCAL_A P_LOCAL_B P_LOGIC_A P_LOGIC_A_FB P_LOGIC_B_FB P_LOW_100UA | P_LOW_10UA P_LOW_150K P_LOW_15K P_LOW_1K5 P_LOW_1MA P_LOW_FAST P_LOW_FLOAT P_MINUS1_A P_MINUS1_B P_MINUS2_A P_MINUS2_B P_MINUS3_A P_MINUS3_B P_NCO_DUTY P_NCO_FREQ P_NORMAL P_OE P_OR_AB P_OUTBIT_A P_OUTBIT_B | P_PASS_AB P_PERIODS_HIGHS P_PERIODS_TICKS P_PLUS1_A P_PLUS1_B P_PLUS2_A P_PLUS2_B P_PLUS3_A P_PLUS3_B P_PULSE P_PWM_SAWTOOTH P_PWM_SMPS P_PWM_TRIANGLE P_QUADRATURE P_REG_UP P_REG_UP_DOWN P_REPOSITORY P_SCHMITT_A P_SCHMITT_A_FB P_SCHMITT_B_FB | P_STATE_TICKS P_SYNC_IO P_SYNC_RX P_SYNC_TX P_TRANSITION P_TRUE_A P_TRUE_B P_TRUE_IN P_TRUE_OUT P_TRUE_OUTPUT P_TT_00 P_TT_01 P_TT_10 P_TT_11 P_USB_PAIR P_XOR_AB PA PB PC_KEY PC_MOUSE | PI PINCLEAR PINF PINFLOAT PINH PINHIGH PINL PINLOW PINR PINREAD PINSTART PINT PINTOGGLE PINW PINWRITE PLOT POLAR POLLATN POLLCT POLLCT1 | POLLCT2 POLLCT3 POLLFBW POLLINT POLLPAT POLLQMT POLLSE1 POLLSE2 POLLSE3 POLLSE4 POLLXFI POLLXMT POLLXRL POLLXRO POLXY POP POPA POPB POS POSX | PR0 PR1 PR2 PR3 PR4 PR5 PR6 PR7 PRECISE PRECOMPILE PRI PTRA PTRB PUB PUSH PUSHA PUSHB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Q - R

| QCOS QDIV QEXP QFRAC QLOG QMUL | QROTATE QSIN QSQRT QUIT QVECTOR RANGE | RATE RCL RCR RCZL RCZR RDBYTE | RDFAST RDLONG RDLUT RDPIN RDWORD RECV | RED REG REGEXEC REGLOAD REP REPEAT | RES RESI0 RESI1 RESI2 RESI3 RET | RETA RETB RETI0 RETI1 RETI2 RETI3 | RETURN REV RFBYTE RFLONG RFVAR RFVARS | RFWORD RGB16 RGB24 RGB8 RGBEXP RGBI8 | RGBI8W RGBI8X RGBSQZ ROL ROLBYTE ROLNIB | ROLWORD ROR ROTXY ROUND RQPIN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## S - T

| SAL SAMPLES SAR SAVE SBIN SBIN_ SBIN_BYTE_ SBIN_BYTE_ARRAY SBIN_BYTE_ARRAY_ SBIN_LONG SBIN_LONG_ SBIN_LONG_ARRAY SBIN_LONG_ARRAY_ SBIN_REG_ARRAY SBIN_REG_ARRAY_ SBIN_WORD SBIN_WORD_ SBIN_WORD_ARRAY | SBIN_WORD_ARRAY_ SCA SCAS SCOPE SCOPE_XY SCROLL SDEC SDEC_ SDEC_BYTE SDEC_BYTE_ SDEC_BYTE_ARRAY SDEC_BYTE_ARRAY_ SDEC_LONG SDEC_LONG_ SDEC_LONG_ARRAY SDEC_LONG_ARRAY_ SDEC_REG_ARRAY SDEC_REG_ARRAY_ | SDEC_WORD SDEC_WORD_ SDEC_WORD_ARRAY SDEC_WORD_ARRAY_ SEND SET SETBYTE SETCFRQ SETCI SETCMOD SETCQ SETCY SETD SETDACS SETINT1 SETINT2 SETINT3 SETLUTS | SETNIB SETPAT SETPIV SETPIX SETQ SETQ2 SETR SETREGS SETS SETSCP SETSE1 SETSE2 SETSE3 SETSE4 SETWORD SETXFRQ SEUSSF SEUSSR | SHEX SHEX_ SHEX_BYTE SHEX_BYTE_ SHEX_BYTE_ARRAY SHEX_BYTE_ARRAY_ SHEX_LONG SHEX_LONG_ SHEX_LONG_ARRAY SHEX_LONG_ARRAY_ SHEX_REG_ARRAY SHEX_REG_ARRAY_ SHEX_WORD SHEX_WORD_ SHEX_WORD_ARRAY SHEX_WORD_ARRAY_ SHL SHR | SIGNED SIGNX SIZE SKIP SKIPF SPACING SPECTRO SPLITB SPLITW SPRITE SPRITEDEF SQRT STALLI STEP STRCOMP STRING STRSIZE SUB | SUBR SUBS SUBSX SUBX SUMC SUMNC SUMNZ SUMZ TERM TEST TESTB TESTBN TESTN TESTP TESTPN TEXT TEXTANGLE TEXTSIZE | TEXTSTYLE TITLE TJF TJNF TJNS TJNZ TJS TJV TJZ TO TRACE TRGINT1 TRGINT2 TRGINT3 TRIGGER TRUE TRUNC |
| --- | --- | --- | --- | --- | --- | --- | --- |

## U, V, W

| UBIN UBIN_ UBIN_BYTE UBIN_BYTE_ UBIN_BYTE_ARRAY UBIN_BYTE_ARRAY_ UBIN_LONG UBIN_LONG_ UBIN_LONG_ARRAY UBIN_LONG_ARRAY_ UBIN_REG_ARRAY UBIN_REG_ARRAY_ UBIN_WORD UBIN_WORD_ UBIN_WORD_ARRAY | UBIN_WORD_ARRAY_ UDEC UDEC_ UDEC_BYTE UDEC_BYTE_ UDEC_BYTE_ARRAY UDEC_BYTE_ARRAY_ UDEC_LONG UDEC_LONG_ UDEC_LONG_ARRAY UDEC_LONG_ARRAY_ UDEC_REG_ARRAY UDEC_REG_ARRAY_ UDEC_WORD UDEC_WORD_ | UDEC_WORD_ARRAY UDEC_WORD_ARRAY_ UHEX UHEX_ UHEX_BYTE UHEX_BYTE_ UHEX_BYTE_ARRAY UHEX_BYTE_ARRAY_ UHEX_LONG UHEX_LONG_ UHEX_LONG_ARRAY UHEX_LONG_ARRAY_ UHEX_REG_ARRAY UHEX_REG_ARRAY_ UHEX_WORD | UHEX_WORD_ UHEX_WORD_ARRAY UHEX_WORD_ARRAY_ UNTIL UPDATE VAR VARBASE WAITATN WAITCT WAITCT1 WAITCT2 WAITCT3 WAITFBW WAITINT WAITMS | WAITPAT WAITSE1 WAITSE2 WAITSE3 WAITSE4 WAITUS WAITX WAITXFI WAITXMT WAITXRL WAITXRO WC WCZ WFBYTE WFLONG | WFWORD WHILE WHITE WINDOW WMLONG WORD WORDFILL WORDMOVE WORDS_1BIT WORDS_2BIT WORDS_4BIT WORDS_8BIT WRBYTE WRC WRFAST | WRLONG WRLUT WRNC WRNZ WRPIN WRWORD WRZ WXPIN WYPIN WZ |
| --- | --- | --- | --- | --- | --- | --- |

## X, Y, Z

| X_16P_2DAC8_WFWORD X_16P_4DAC4_WFWORD X_1ADC8_0P_1DAC8_WFBYTE X_1ADC8_8P_2DAC8_WFWORD X_1P_1DAC1_WFBYTE X_2ADC8_0P_2DAC8_WFWORD X_2ADC8_16P_4DAC8_WFLONG X_2P_1DAC2_WFBYTE X_2P_2DAC1_WFBYTE X_32P_4DAC8_WFLONG X_4ADC8_0P_4DAC8_WFLONG X_4P_1DAC4_WFBYTE X_4P_2DAC2_WFBYTE X_4P_4DAC1_WFBYTE X_8P_1DAC8_WFBYTE X_8P_2DAC4_WFBYTE | X_8P_4DAC2_WFBYTE X_ALT_OFF X_ALT_ON X_DACS_0_0_0_0 X_DACS_0_0_X_X X_DACS_0_X_X_X X_DACS_0N0_0N0 X_DACS_0N0_X_X X_DACS_1_0_1_0 X_DACS_1_0_X_X X_DACS_1N1_0N0 X_DACS_3_2_1_0 X_DACS_OFF X_DACS_X_0_X_X X_DACS_X_X_0_0 X_DACS_X_X_0_X | X_DACS_X_X_0N0 X_DACS_X_X_1_0 X_DACS_X_X_X_0 X_DDS_GOERTZEL_SINC1 X_DDS_GOERTZEL_SINC2 X_IMM_16X2_1DAC2 X_IMM_16X2_2DAC1 X_IMM_16X2_LUT X_IMM_1X32_4DAC8 X_IMM_2X16_2DAC8 X_IMM_2X16_4DAC4 X_IMM_32X1_1DAC1 X_IMM_32X1_LUT X_IMM_4X8_1DAC8 X_IMM_4X8_2DAC4 X_IMM_4X8_4DAC2 | X_IMM_4X8_LUT X_IMM_8X4_1DAC4 X_IMM_8X4_2DAC2 X_IMM_8X4_4DAC1 X_IMM_8X4_LUT X_PINS_OFF X_PINS_ON X_RFBYTE_1P_1DAC1 X_RFBYTE_2P_1DAC2 X_RFBYTE_2P_2DAC1 X_RFBYTE_4P_1DAC4 X_RFBYTE_4P_2DAC2 X_RFBYTE_4P_4DAC1 X_RFBYTE_8P_1DAC8 X_RFBYTE_8P_2DAC4 X_RFBYTE_8P_4DAC2 | X_RFBYTE_LUMA8 X_RFBYTE_RGB8 X_RFBYTE_RGBI8 X_RFLONG_16X2_LUT X_RFLONG_32P_4DAC8 X_RFLONG_32X1_LUT X_RFLONG_4X8_LUT X_RFLONG_8X4_LUT X_RFLONG_RGB24 X_RFWORD_16P_2DAC8 X_RFWORD_16P_4DAC4 X_RFWORD_RGB16 X_WRITE_OFF X_WRITE_ON XCONT XINIT | XOR XORC XORO32 XORZ XSTOP XYPOL XZERO YELLOW ZEROX ZSTR ZSTR_ |
| --- | --- | --- | --- | --- | --- |

# CHANGE LOG

| Date | Notes |
| --- | --- |
| 11/11/2021 | Live draft workspace published. |
| 11/02/2022 | Official release. |

# PARALLAX INCORPORATED

| Parallax Inc. 599 Menlo Drive, Suite 100 Rocklin, CA 95765 USA | Office: +1 916-624-8333 Toll Free US: 888-512-1024 | sales@parallax.com support@parallax.com | www.parallax.com/p2 forums.parallax.com |
| --- | --- | --- | --- |
| Purchase of the P2X8C4M64P does not include any license to emulate any other device nor to communicate via any specific proprietary protocol; P2X8C4M64P connectivity objects and code examples provided or referenced by Parallax, Inc. are NOT licensed and are provided for research and development purposes only; end users must seek permission to use licensed protocols for their applications and products from the protocol license holders. Parallax, Inc. makes no warranty, representation or guarantee regarding the suitability of its products for any particular purpose, nor does Parallax, Inc. assume any liability arising out of the application or use of any product, and specifically disclaims any and all liability, including without limitation consequential or incidental damages even if Parallax, Inc. has been advised of the possibility of such damages. Copyright © 2022 Parallax, Inc. All rights are reserved. Parallax, the Parallax logo, the P2 logo, and Propeller are trademarks of Parallax, Inc. |  |  |  |